What is the change

Change the second DHCP relay (helper) address on the 200V core switch, trinity200V-core (2 x ICX8200-24FX stack, RUCKUS ICX FastIron 10.0.10g), from 192.168.12.116 to 10.20.16.11 on all 22 VLAN interfaces (VEs) that relay DHCP for the new 200V subnets. The first helper, 10.20.16.10, is not changed.

| Item | Value |
| --- | --- |
| Device | trinity200V-core (management 10.20.8.2, ve 1960) |
| Setting | ip helper-address 2 on interface ve <VLAN> |
| Current value | 192.168.12.116 |
| New value | 10.20.16.11 |
| Unchanged | ip helper-address 1 10.20.16.10; VE addresses; ACLs; DHCP snooping; access switches |
| Interfaces | 22 VEs: 2000, 2010, 2020, 2100, 2200, 2300, 2310, 2320, 2400, 2500, 2510, 2520, 2530, 2600, 2700, 2800, 2810, 2900, 2950, 2951, 2952, 2953 (Appendix A) |
| Configuration file | 200V-core-dhcp-helper-ICX.txt (pre-check, change in two blocks, verify, rollback) |

Impact

- No outage expected. The core relays each DHCP request to both helpers; while the change is made, clients still get addresses from 10.20.16.10

- Devices that already have a lease keep their address. New and renewing clients use 10.20.16.11 as the second DHCP server from the moment each VE is changed

- No access switch, VLAN, IP address or ACL is changed

Duration of impact

None expected. The change takes a few minutes to paste and verify; each VE is updated in under a second.

How Change implemented

The change will be implemented by Alam Singh (Dycom) over SSH/telnet to trinity200V-core using 200V-core-dhcp-helper-ICX.txt. For each of the 22 VEs the old helper is removed and the new one added (no ip helper-address 2 192.168.12.116, then ip helper-address 2 10.20.16.11), pasted in two blocks of 11 VEs so a paste cannot be cut short. The configuration was checked against the running configuration of trinity200V-core taken on 12 October 2026: all 22 VEs have helper 2 set to 192.168.12.116 and are covered.

Implementation steps

- Pre-check: show running-config | include helper-address. Expect 44 lines: helper 1 10.20.16.10 and helper 2 192.168.12.116 on each of the 22 VEs.

- Paste change block 1 (VEs 2000 to 2510).

- Paste change block 2 (VEs 2520 to 2953).

- Verify: show running-config | include helper-address 2. Expect 22 lines, all 10.20.16.11.

- Save: write memory.

- Test: renew DHCP on a test client in at least two VLANs (for example staff wired 2100 and Wi-Fi 2300) and confirm it gets an address on the right subnet.

Prerequisites

| Prerequisite | Status |
| --- | --- |
| DHCP server 10.20.16.11 has scopes for every subnet in Appendix A, with the same options and reservations as 192.168.12.116 | Pending |
| 10.20.16.11 is reachable from trinity200V-core (ping 10.20.16.11) | Pending |
| Current running configuration of trinity200V-core backed up (12 October 2026) | Complete |
| Change approved | Pending |

How rollback implemented

Paste the two ROLLBACK blocks from 200V-core-dhcp-helper-ICX.txt, which put helper 2 back to 192.168.12.116 on all 22 VEs, then write memory. Rollback takes a few minutes and has no impact.

Roll back if

- test clients do not get an address after the change

- the DHCP server team reports 10.20.16.11 is not answering relayed requests

- Trinity asks for a rollback

How change tested

- show running-config | include helper-address 2 shows 22 lines, all 10.20.16.11

- A test client on staff wired (2100) and on Wi-Fi (2300) gets an address on its subnet after a DHCP renew

- The DHCP server logs on 10.20.16.11 show relayed requests from the 200V gateways (10.21.x.1)

Risks of implementing

- If 10.20.16.11 is missing a scope or reservation, then clients on that subnet that are served by it get no address or the wrong one. This has low likelihood if the scopes are checked first (prerequisite). If this risk occurs, roll back.

- If a paste is cut short, then some VEs keep the old helper or have no second helper. This has low likelihood because the change is pasted in two short blocks and verified. If this risk occurs, the missing lines are pasted again.

- Unicast DHCP renewals go straight from the client to the server and pass the VLAN ACLs on the core. AV-IN, WIFIMGMT-IN, PRINTERS-IN, RESTRICTED-GROUP-IN, GUEST-IN and SERVERS-IN permit 192.168.12.0/24 or deny other 10.0.0.0/8 addresses, but do not permit 10.20.16.11. Clients on those VLANs keep their address and renew by broadcast through the relay instead. This is low impact; if direct renewals are wanted, a follow-up change can permit UDP 67 to 10.20.16.10 and 10.20.16.11 in those ACLs.

Suggested Scheduling

Any time; no outage is expected. Prefer business hours so the DHCP server team can confirm relayed requests reach 10.20.16.11.

Appendix A – VLAN interfaces

All 22 VEs on trinity200V-core that relay DHCP. Helper 1 stays 10.20.16.10; helper 2 changes as below.

| VE | VLAN name | Subnet | Helper 2 now | Helper 2 new |
| --- | --- | --- | --- | --- |
| 2000 | VL-netmgmt-01 | 10.21.0.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2010 | VL-wifimgmt-01 | 10.21.1.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2020 | VL-servers-01 | 10.21.2.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2100 | VL-staff-wired-01 | 10.21.3.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2200 | VL-student-wired-01 | 10.21.4.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2300 | VL-wifi-01 | 10.21.8.0/21 | 192.168.12.116 | 10.20.16.11 |
| 2310 | VL-wifi-staff-01 | 10.21.16.0/23 | 192.168.12.116 | 10.20.16.11 |
| 2320 | VL-wifi-student-01 | 10.21.18.0/23 | 192.168.12.116 | 10.20.16.11 |
| 2400 | VL-byod-01 | 10.21.20.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2500 | VL-iot-01 | 10.21.21.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2510 | VL-bac-01 | 10.21.22.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2520 | VL-bms-01 | 10.21.23.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2530 | VL-library-01 | 10.21.24.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2600 | VL-cctv-01 | 10.21.25.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2700 | VL-av-01 | 10.21.36.0/23 | 192.168.12.116 | 10.20.16.11 |
| 2800 | VL-print-01 | 10.21.27.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2810 | VL-comms-01 | 10.21.28.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2900 | VL-guest-01 | 10.21.29.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2950 | VL-nac-restricted-01 | 10.21.32.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2951 | VL-nac-onboard-01 | 10.21.33.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2952 | VL-nac-remediate-01 | 10.21.34.0/24 | 192.168.12.116 | 10.20.16.11 |
| 2953 | VL-nac-fallback-01 | 10.21.35.0/24 | 192.168.12.116 | 10.20.16.11 |