What is the change

This change has two parts on the same eight 200V access switches. Part A cleans up access ports as recommended by Trinity. Part B moves each switch's management address from VLAN 100 (192.168.122.x) to VL-netmgmt-01, VLAN 2000 (10.21.0.x), so that all management traffic uses the new network. The two parts are independent and can be done in either order or in separate windows. The core switch, trinity200V-core, is not changed by either part.

Part A, port clean-up. Clean up 267 access ports on eight 200V switches as recommended by Trinity in column Q of Trinity_switch_ports_16-10-2026.xlsx (SmartZone export, 16 October 2026). Ports still on the old staff VLAN 1240 move to VL-staff-wired-01 (2100); unused ports still on old VLANs (mostly the old AV VLAN 1230) are returned to the default VLAN (VLAN 1); three UPS management cards move to VL-netmgmt-01 (2000); and old trunk (tagged) VLANs are removed from access ports where Trinity asked, with the port decisions agreed with Trinity on 22 October. 267 ports are in this change.

| Switch | To 2100 | To 2100, tags off | To 2000 | To VLAN 1 | To VLAN 1, tags off |
| --- | --- | --- | --- | --- | --- |
| switch-200V-stack-lv3 | 16 | 0 | 0 | 15 | 1 |
| switch-200V-stack-lv4 | 23 | 0 | 0 | 30 | 1 |
| switch-200V-stack-lv6 | 116 | 19 | 0 | 7 | 2 |
| switch-200V-LT2-1 | 1 | 0 | 0 | 3 | 1 |
| switch-200V-LT2-2 | 10 | 0 | 0 | 0 | 1 |
| switch-200V-LT2-3 | 0 | 10 | 0 | 0 | 0 |
| switch-200V-Tutorial-6.03 | 0 | 0 | 0 | 1 | 0 |
| switch-200V-8 | 7 | 0 | 3 | 0 | 0 |
| Total | 173 | 29 | 3 | 56 | 6 |

| Item | Value |
| --- | --- |
| Devices | switch-200V-stack-lv3, -lv4, -lv6, switch-200V-LT2-1, -LT2-2, -LT2-3, switch-200V-Tutorial-6.03, switch-200V-8 (RUCKUS ICX, FastIron 10.0.10g) |
| Setting | Untagged and tagged VLAN membership of access ports only |
| Unchanged | Uplinks, LAGs, stack ports, the core switch, VLANs, IP addressing, ACLs, DHCP |
| Default VLAN | VLAN 1 (DEFAULT-VLAN). Removing a port's untagged VLAN on ICX returns it to VLAN 1. VLAN 1 has no IP address on the core, so a device plugged into one of these ports gets no network |
| Configuration file | 200V-port-review-16oct-ICX.txt (per switch: pre-check, change, port bounce, verify, rollback) |

Part B, switch management to VLAN 2000. Each switch gets an address on interface ve 2000 in 10.21.0.0/24; its default route moves from 192.168.120.1 to 10.21.0.1 (trinity200V-core ve 2000); 'management source-interface ve 2000 protocol all' makes every management protocol the switch originates (SmartZone, SNMP traps, NTP, DNS, SSH/telnet, TFTP/SCP) use the new address; and finally the 192.168.122.x address is removed from ve 100. VLAN 100 itself stays, as it is still tagged on ports and uplinks.

| Switch | Now (ve 100) | New (ve 2000) |
| --- | --- | --- |
| switch-200V-stack-lv6 | 192.168.122.11/22 | 10.21.0.20/24 |
| switch-200V-LT2-1 | 192.168.122.12/22 | 10.21.0.21/24 |
| switch-200V-LT2-2 | 192.168.122.13/22 | 10.21.0.22/24 |
| switch-200V-LT2-3 | 192.168.122.14/22 | 10.21.0.23/24 |
| switch-200V-Tutorial-6.03 | 192.168.122.15/22 | 10.21.0.24/24 |
| switch-200V-8 | 192.168.122.16/22 | 10.21.0.25/24 |
| switch-200V-stack-lv4 | 192.168.122.21/22 | 10.21.0.26/24 |
| switch-200V-stack-lv3 | 192.168.122.22/22 | 10.21.0.27/24 |

| Item | Value |
| --- | --- |
| Gateway | 10.21.0.1, interface ve 2000 on trinity200V-core (no ACL on VLAN 2000) |
| Unchanged | trinity200V-core (management stays on ve 1960, 10.20.8.2), VLANs, ports, ACLs, SmartZone active-list, SNMP and NTP settings |
| Configuration file | 200V-mgmt-ve2000-ICX.txt (core pre-check; per switch: pre-check, add ve 2000, connect from the core, route and source-interface, verify, remove old address, rollback) |

Impact

- 242 of the 267 ports were down on 16 October: no impact; a device plugged in later comes up on the new VLAN

- 25 ports were up: staff devices on 1240 (10.9.9.x) and the three UPS cards on switch-200V-8. Each loses its connection for about 30 seconds while the port is bounced; staff devices then get a new address on 10.21.3.0/24 (VL-staff-wired-01)

- The three UPS cards (switch-200V-8 1/1/7 - 1/1/9, now 10.9.9.95 - .97) use DHCP: after the bounce they get new addresses on 10.21.0.0/24 (VL-netmgmt-01). Anything that monitors them by IP address needs updating

- Devices with a fixed 10.9.9.x address on the moved ports will not work on 2100 until they are set to DHCP

- Ports moved to VLAN 1 are unused; anything later plugged into them gets no network until the port is assigned a VLAN

- No uplink, wireless, AV, CCTV or building service is changed

- Part B: no impact on users or devices; only switch management traffic moves. Each switch's SmartZone connection drops and re-forms from its new address (up to 5 minutes), and a CLI session to the old 192.168.122.x address drops when the default route changes

- Part B: after the change the switches are managed on 10.21.0.20 - .27

Duration of impact

About 30 seconds for each of the ports that are up, while it is bounced and spanning tree forwards on 2100. Part A takes about 1 hour to paste and verify across the eight switches. Part B has no user impact and takes about 10 minutes per switch, mostly waiting for SmartZone to reconnect (about 1.5 hours in total).

How Change implemented

The change will be implemented by Alam Singh (Dycom) over SSH/telnet to each switch, using 200V-port-review-16oct-ICX.txt. For each switch the pre-check confirms the ports are still as exported on 16 October; the change removes the old untagged VLAN, adds 2100 untagged where required and removes the old tagged VLANs where required; ports that were up are bounced so their devices renew DHCP; the result is verified and saved. The same method was used for the 9 October cutover.

Part B is implemented from a session on trinity200V-core. The core is directly connected to 10.21.0.0/24, so after the new address is added a telnet session from the core to 10.21.0.x keeps working while the switch's default route and management source are changed. switch-200V-Tutorial-6.03 is done first as a pilot.

Implementation steps

- Confirm the prerequisites below.

- For each switch in turn: paste the PRE-CHECK; check every listed port is still untagged in its 'now' VLAN. Leave out any port that has changed.

- Paste the CHANGE block.

- Paste PORT BOUNCE part 1, wait 10 seconds, paste part 2 (switches with ports that were up).

- Paste the VERIFY block; check the ports have left the old VLANs, the 2100 ports are untagged in 2100 and the bounced ports' MACs are learned again.

- write memory.

- Ask Trinity to test a staff device on one of the moved ports (for example lv6 3/1/6 or lv3 3/1/16).

- Part B: on trinity200V-core run the core PRE-CHECK; ping 10.21.0.20 - .27 must get no reply.

- Part B, per switch (Tutorial-6.03 first): run the switch PRE-CHECK, then STEP 1 (add ve 2000) and ping 10.21.0.1.

- Part B: from trinity200V-core, telnet to the new address (STEP 2) and paste STEP 3 (default route via 10.21.0.1 and management source-interface ve 2000).

- Part B: VERIFY: one default route via 10.21.0.1, replies from 10.21.0.1, DNS and SmartZone, and show manager status connected; then write memory.

- Part B: when SmartZone shows the switch on its new address, paste STEP 4 to remove the 192.168.122.x address, check show ip interface lists only ve 2000, and write memory.

Prerequisites

| Prerequisite | Status |
| --- | --- |
| Change approved | Pending |
| Running configuration of the eight switches backed up (SmartZone backup or show running-config) | Scheduled: Monday 26 October 2026, morning |
| Port decisions agreed with Trinity (22 October): lv3 2/1/27, lv4 4/1/32 and lv4 5/1/19 (20:7B:D2 devices) to 2100; switch-200V-8 1/1/7 - 1/1/9 (UPS management cards, DHCP) to 2000; LT2-2 1/1/1 old VLANs removed, new VLANs 2000 - 2953 stay tagged; ports moved to VLAN 1 stay enabled | Complete |
| VLAN 2100 present on every switch and tagged on its uplink (checked in the 16 October export) | Complete |
| Part B: 10.21.0.20 - 10.21.0.27 excluded from the 10.21.0.0/24 DHCP scope on 10.20.16.10 / 10.20.16.11 | Complete (confirmed by Trinity 22 October) |
| Part B: ntp1.trinity.unimelb.edu.au resolves and is reachable from 10.21.0.0/24 | Complete (checked 22 October) |
| Part B: VLAN 2000 tagged on every switch uplink and ve 2000 10.21.0.1/24 on the core (checked in the 16 October export and the 12 October core configuration) | Complete |

How rollback implemented

Each switch has a ROLLBACK block in 200V-port-review-16oct-ICX.txt that puts every port back exactly as it was on 16 October (old untagged VLAN and old tagged VLANs). Paste it, bounce the ports that were up, and write memory. A single port can be rolled back by pasting only its lines.

Part B: each switch has a ROLLBACK block in 200V-mgmt-ve2000-ICX.txt, pasted from the core-hop session. It puts back the 192.168.122.x address on ve 100, the default route via 192.168.120.1 and the original management source; then, from a session to the old address, the ve 2000 address is removed.

Roll back if

- a staff device on a moved port does not get an address on 10.21.3.0/24 within a few minutes of the bounce

- a device that is not expected stops working, such as one on a port that had a different device to the export

- Trinity asks for a rollback

- Part B: a switch does not reconnect to SmartZone from its new address within 10 minutes, or cannot reach 10.21.0.1

How change tested

- show vlan for each old VLAN no longer lists the changed ports

- show vlan 2100 lists the ports moved to 2100 as untagged

- show mac-address ethernet <port> shows each bounced port's device learned on 2100

- A staff device on a moved port gets a 10.21.3.x address and reaches its services

- Part B: show ip interface lists only ve 2000 (10.21.0.x); show ip route has one default route via 10.21.0.1; show running-config shows management source-interface ve 2000 protocol all

- Part B: SmartZone shows all eight switches online with their 10.21.0.x address

- Part B: show ntp associations shows the NTP server reachable

Risks of implementing

- If a port has changed since the 16 October export, then the wrong device could be moved. This has low likelihood because the pre-check is run first. If this risk occurs, roll back that port.

- If a device on a port moved to 2100 has a fixed 10.9.9.x address, then it loses its connection. This has low likelihood (the devices seen are staff devices on DHCP). If this risk occurs, set the device to DHCP or roll back the port.

- If the UPS cards do not renew DHCP after the port bounce, then they keep their old 10.9.9.x address and cannot be reached. This has low likelihood because the bounce drops the link. If this risk occurs, bounce the port again or power cycle the card's network module.

- If a port moved to VLAN 1 is needed by a device that was switched off on 16 October (for example an AV device), then that device gets no network when it is turned on. This has low likelihood because no MAC was seen on these ports in the export. If this risk occurs, assign the port its VLAN (2700 for AV).

- If a paste is cut short, then some ports are left half changed. This has low likelihood because each switch is verified. If this risk occurs, paste the missing lines again.

- Part B: if a switch's new address cannot reach 10.21.0.1 (VLAN 2000 not carried end to end), then the switch loses management when the default route changes. This has low likelihood because STEP 1 tests the ping before STEP 3. If this risk occurs, the old session is still open: paste the ROLLBACK.

- Part B: while both ve 100 and ve 2000 have an address the switch can route between 192.168.120.0/22 and 10.21.0.0/24. This has low likelihood of use and lasts only until STEP 4.

Suggested Scheduling

Change window: Monday 26 October 2026, 6:00 pm to 7:00 pm, after the switch backups taken that morning.

Part A (port clean-up) is done first and takes about 1 hour across the eight switches, so it fills the window. Order: lv6 (144 ports), lv4, lv3, then the smaller switches. Part B (management to VLAN 2000) takes about 1.5 hours and has no user impact: start it in the window only if Part A finishes early (pilot Tutorial-6.03 first), otherwise schedule it separately, in business hours if preferred.

Appendix A – Affected ports

All ports from column Q of the 16 October export with a change. 'Tagged off' lists the tagged VLANs that are removed. Notes show the decisions of 22 October that change column Q.

| Switch | Port | Now (untagged [tagged off]) | New untagged | 16 Oct / note |
| --- | --- | --- | --- | --- |
| stack-lv3 | 1/1/12 | 100 [1212, 1213, 1230, 1240, 1250, 1271, 1272, 1273] | 1 (default) | Down |
| stack-lv3 | 1/1/34 | 1230 | 1 (default) | Down |
| stack-lv3 | 2/1/14 | 1230 | 1 (default) | Down |
| stack-lv3 | 2/1/24 | 1230 | 1 (default) | Down |
| stack-lv3 | 2/1/27 | 1230 | 2100 | item 1, Trinity 22 Oct: 20:7B:D2:30:98:C1 (200V3J14) to 2100 |
| stack-lv3 | 2/1/34 | 1230 | 1 (default) | Down |
| stack-lv3 | 3/1/3 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/7 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/10 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/11 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/13 | 1240 | 2100 | Up, 1C:1A:DF:C2:51:C2 |
| stack-lv3 | 3/1/15 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/16 | 1240 | 2100 | Up, A0:4A:5E:E7:D5:7C |
| stack-lv3 | 3/1/19 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/27 | 1240 | 2100 | Down |
| stack-lv3 | 3/1/28 | 230 | 1 (default) | Down |
| stack-lv3 | 4/1/2 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/23 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/24 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/25 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/26 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/27 | 1240 | 2100 | Down |
| stack-lv3 | 4/1/45 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/4 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/5 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/6 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/7 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/13 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/15 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/20 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/32 | 1230 | 1 (default) | Down |
| stack-lv3 | 5/1/43 | 1230 | 1 (default) | Down |
| stack-lv4 | 1/1/8 | 100 [1272, 1273, 2300, 2310, 2311, 2320, 2321, 2900] | 1 (default) | Down |
| stack-lv4 | 1/1/17 | 1230 | 1 (default) | Down |
| stack-lv4 | 1/1/32 | 1230 | 1 (default) | Down |
| stack-lv4 | 2/1/8 | 1212 | 1 (default) | Down |
| stack-lv4 | 2/1/18 | 1230 | 1 (default) | Down |
| stack-lv4 | 2/1/20 | 1230 | 1 (default) | Down |
| stack-lv4 | 3/1/4 | 1240 | 2100 | Up, A0:4A:5E:E7:35:43 |
| stack-lv4 | 3/1/5 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/9 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/11 | 1240 | 2100 | Up |
| stack-lv4 | 3/1/13 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/14 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/19 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/20 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/21 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/22 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/27 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/29 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/30 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/35 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/37 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/39 | 1240 | 2100 | Up, 1C:1A:DF:C2:51:D3 |
| stack-lv4 | 3/1/42 | 1240 | 2100 | Up, 4C:C5:D9:CD:B4:CC |
| stack-lv4 | 3/1/43 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/45 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/47 | 1240 | 2100 | Down |
| stack-lv4 | 3/1/48 | 1240 | 2100 | Down |
| stack-lv4 | 4/1/1 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/2 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/7 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/10 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/16 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/17 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/27 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/28 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/29 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/31 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/32 | 1272 | 2100 | item 1, Trinity 22 Oct: 20:7B:D2:30:99:15 (200V4H9) to 2100 |
| stack-lv4 | 4/1/38 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/39 | 1230 | 1 (default) | Down |
| stack-lv4 | 4/1/44 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/1 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/2 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/5 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/14 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/16 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/19 | 1272 | 2100 | item 1, Trinity 22 Oct: 20:7B:D2:30:99:25 (200V4I17) to 2100 |
| stack-lv4 | 5/1/24 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/26 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/35 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/38 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/40 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/43 | 1230 | 1 (default) | Down |
| stack-lv4 | 5/1/47 | 1230 | 1 (default) | Down |
| stack-lv6 | 1/1/1 | 1240 [1230, 1250, 1271, 1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/2 | 1240 [1230, 1250, 1271, 1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/4 | 1240 [1230, 1250, 1271, 1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/5 | 1240 [1230, 1250, 1271, 1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/7 | 1240 [1230, 1250, 1271, 1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/11 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/12 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/30 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/32 | 1240 [1272, 1273] | 2100 | Up, 1C:1A:DF:C2:4F:5E |
| stack-lv6 | 1/1/33 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/34 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/35 | 100 [1212, 1213, 1230, 1240, 1250, 1271, 1272, 1273] | 1 (default) | Down |
| stack-lv6 | 1/1/36 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/41 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/42 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/43 | 1240 [1272, 1273] | 2100 | Up, A0:4A:5E:E7:E8:70 |
| stack-lv6 | 1/1/44 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/45 | 1240 [1272] | 2100 | Down |
| stack-lv6 | 1/1/46 | 1240 [1272, 1273] | 2100 | Down |
| stack-lv6 | 1/1/47 | 1240 [1273] | 2100 | Down |
| stack-lv6 | 1/1/48 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/4 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/5 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/6 | 1240 | 2100 | Up, 1C:1A:DF:C2:51:28 |
| stack-lv6 | 2/1/7 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/8 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/9 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/10 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/11 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/12 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/13 | 1240 | 2100 | Down |
| stack-lv6 | 2/1/48 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/2 | 1230 | 1 (default) | Down |
| stack-lv6 | 3/1/3 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/4 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/5 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/6 | 1240 | 2100 | Up, 1C:1A:DF:C2:51:01 |
| stack-lv6 | 3/1/7 | 1240 | 2100 | Up, 1C:1A:DF:C0:98:69 |
| stack-lv6 | 3/1/8 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/9 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/10 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/21 | 1240 | 2100 | Down |
| stack-lv6 | 3/1/22 | 1240 | 2100 | Up, 1C:1A:DF:C0:9A:AB |
| stack-lv6 | 3/1/23 | 1240 | 2100 | Down |
| stack-lv6 | 4/1/2 | 1240 | 2100 | Down |
| stack-lv6 | 4/1/3 | 1240 | 2100 | Down |
| stack-lv6 | 4/1/7 | 1240 | 2100 | Down |
| stack-lv6 | 4/1/9 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/2 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/3 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/4 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/5 | 1240 | 2100 | Up, 1C:1A:DF:C2:51:DF |
| stack-lv6 | 5/1/6 | 1240 | 2100 | Up |
| stack-lv6 | 5/1/7 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/8 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/9 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/10 | 1240 | 2100 | Up, 1C:1A:DF:C0:9A:52 |
| stack-lv6 | 5/1/11 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/12 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/13 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/14 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/15 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/16 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/17 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/18 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/19 | 1240 | 2100 | Up, 1C:1A:DF:C2:50:05 |
| stack-lv6 | 5/1/20 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/21 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/22 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/23 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/24 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/25 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/26 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/27 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/28 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/29 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/31 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/32 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/33 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/35 | 100 [230, 1212, 1213, 1230, 1250, 1271, 1272, 1273] | 1 (default) | Down |
| stack-lv6 | 5/1/36 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/37 | 1230 | 1 (default) | Down |
| stack-lv6 | 5/1/39 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/40 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/41 | 1240 | 2100 | Up, 4C:C5:D9:D0:8D:23 |
| stack-lv6 | 5/1/43 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/44 | 1240 | 2100 | Up, 1C:1A:DF:C2:45:3F |
| stack-lv6 | 5/1/45 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/46 | 1240 | 2100 | Down |
| stack-lv6 | 5/1/47 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/1 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/2 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/3 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/4 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/5 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/6 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/7 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/8 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/9 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/10 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/11 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/12 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/13 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/14 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/15 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/17 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/18 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/19 | 1230 | 1 (default) | Down |
| stack-lv6 | 6/1/21 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/23 | 1230 | 1 (default) | Down |
| stack-lv6 | 6/1/25 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/29 | 1230 | 1 (default) | Down |
| stack-lv6 | 6/1/32 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/33 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/36 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/37 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/42 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/43 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/44 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/47 | 1240 | 2100 | Down |
| stack-lv6 | 6/1/48 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/2 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/6 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/7 | 1230 | 1 (default) | Down |
| stack-lv6 | 7/1/12 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/13 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/15 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/16 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/18 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/20 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/21 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/25 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/26 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/29 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/36 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/37 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/38 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/39 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/41 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/42 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/45 | 1230 | 1 (default) | Down |
| stack-lv6 | 7/1/46 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/47 | 1240 | 2100 | Down |
| stack-lv6 | 7/1/48 | 1240 | 2100 | Down |
| LT2-1 | 1/1/2 | 100 [1212, 1213, 1230, 1240, 1250, 1271, 1272, 1273] | 1 (default) | Down |
| LT2-1 | 1/1/3 | 1230 | 1 (default) | Down |
| LT2-1 | 1/1/19 | 1230 | 1 (default) | Down |
| LT2-1 | 1/1/21 | 1240 | 2100 | Up, 48:21:0B:56:AC:A4 |
| LT2-1 | 1/1/22 | 1230 | 1 (default) | Down |
| LT2-2 | 1/1/1 | 1240 [1212, 1213, 1230, 1250, 1271, 1272, 1273] | 1 (default) | item 3, decision 22 Oct: remove old VLANs only. Port becomes tagged-only with 2000 - 2953 (no untagged VLAN) |
| LT2-2 | 1/1/2 | 1240 | 2100 | Down |
| LT2-2 | 1/1/3 | 1240 | 2100 | Down |
| LT2-2 | 1/1/4 | 1240 | 2100 | Down |
| LT2-2 | 1/1/5 | 1240 | 2100 | Down |
| LT2-2 | 1/1/6 | 1240 | 2100 | Down |
| LT2-2 | 1/1/7 | 1240 | 2100 | Up |
| LT2-2 | 1/1/8 | 1240 | 2100 | Down |
| LT2-2 | 1/1/9 | 1240 | 2100 | Down |
| LT2-2 | 1/1/10 | 1240 | 2100 | Down |
| LT2-2 | 1/1/11 | 1240 | 2100 | Down |
| LT2-3 | 1/1/1 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/2 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/3 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/5 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/6 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/7 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/8 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/9 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/10 | 1240 [100] | 2100 | Down |
| LT2-3 | 1/1/11 | 1240 [100] | 2100 | Down |
| Tutorial-6.03 | 1/1/11 | 100 | 1 (default) | Down |
| 8 | 1/1/1 | 1240 | 2100 | Down |
| 8 | 1/1/2 | 1240 | 2100 | Down |
| 8 | 1/1/3 | 1240 | 2100 | Up, 1C:1A:DF:C0:99:7F |
| 8 | 1/1/4 | 1240 | 2100 | Down |
| 8 | 1/1/5 | 1240 | 2100 | Up, AC:B4:80:F1:61:27 |
| 8 | 1/1/7 | 1240 | 2000 | item 2, Trinity 22 Oct: UPS card 00:09:F5:32:DB:15 to 2000 |
| 8 | 1/1/8 | 1240 | 2000 | item 2, Trinity 22 Oct: UPS card 00:09:F5:32:DB:D1 to 2000 |
| 8 | 1/1/9 | 1240 | 2000 | item 2, Trinity 22 Oct: UPS card 00:09:F5:32:DB:14 to 2000 |
| 8 | 1/1/10 | 1240 | 2100 | Down |
| 8 | 1/1/11 | 1240 | 2100 | Down |