What is the change

Move the remaining 353 devices at 200V from their legacy VLANs to the new VLANs (2010, 2510, 2600, 2700 and 2800) by changing the untagged VLAN on 340 access ports on five RUCKUS ICX FastIron 10.0.10g switches.

| Switch | Management IP | Ports | Order | Notes |
| --- | --- | --- | --- | --- |
| switch-200V-LT2-1 | 192.168.122.12 | 16 | 1 | Smallest, done first |
| switch-200V-stack-lv3 | 192.168.122.22 | 84 | 2 |  |
| switch-200V-stack-lv4 | 192.168.122.21 | 89 | 3 |  |
| switch-200V-stack-lv6 | 192.168.122.11 | 145 | 4 | Largest; do after the system-max change |
| switch-200V-Tutorial-6.03 | 192.168.122.15 | 6 | 5 | Done over a console cable on site |

| Device type | Old VLAN | New VLAN | New subnet | Devices / ports |
| --- | --- | --- | --- | --- |
| AV | 1230 (200V_av) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 255 / 242 |
| AP management | 100 (management) | 2010 (VL-wifimgmt-01) | 10.21.1.x | 37 / 37 |
| AV (Airtame) | 1272 (200V_av) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 37 / 37 |
| AV (Room NUC) | 1240 (200V_staff) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 13 / 13 |
| Printer | 1240 (200V_staff) | 2800 (VL-print-01) | 10.21.27.x | 6 / 6 |
| CCTV | 1212 (200V_IPcam) | 2600 (VL-cctv-01) | 10.21.25.x | 4 / 4 |
| Building access | 1213 (200V_building) | 2510 (VL-bac-01) | 10.21.22.x | 1 / 1 |

Only untagged VLAN membership on the listed ports changes. Tagged VLANs on AP and other trunk ports are not changed, apart from adding VLAN 2900 to the AP ports (see Wi-Fi below). The full device list is in Appendix A. Device addressing, DHCP, DNS, firewall rules and any device-side changes are outside the scope of this change. switch-200V-8, LT2-2 and LT2-3 have no listed devices; their only change is the system-max change. Ports with no listed device, and devices not found on any switch, are tracked on the Review tab of 200V Device Port List.xlsx and are not part of this change.

Wi-Fi

The 200V APs are moved to the new SSIDs by changing the 200v WLAN group in SmartZone, in the same window after the switch changes. Trinity NetSegPilot is kept, the four existing WLANs are removed, and the three new WLANs are added:

| WLAN in the 200v WLAN group (SSID, VLAN) | Change |
| --- | --- |
| Trinity NetSegPilot (Trinity NetSegPilot, 2300) | Keep |
| Trinity Staff (Trinity Staff) | Remove |
| Trinity Visitor (Trinity Visitor) | Remove |
| Trinity Wireless - 802.11R (Trinity Wireless) | Remove |
| eduroam 2026 (eduroam) | Remove |
| Trinity Wireless v2300 (Trinity Wireless, 2300) | Add |
| eduroam v2900 (eduroam, 2900) | Add |
| Trinity Visitor v2900 (Trinity Visitor, 2900) | Add |

VLAN 2300 is already tagged on the AP ports, and Trinity NetSegPilot already carries traffic on it. VLAN 2900 is not, so it is added as a tagged VLAN on the 40 AP ports (lv3 1/1/1 to 1/1/11, lv4 1/1/1 to 1/1/11, lv6 1/1/13 to 1/1/29 and LT2-1 1/1/1) as part of the switch changes, before the WLAN group is changed. This only adds tagging and has no impact on the APs or their current SSIDs.

Impact

- Each switch is reloaded once for the system-max change, before the cutover. Everything on a switch is offline for a few minutes while it restarts. Reloading LT2-1 also drops LT2-2 and LT2-3, and reloading trinity200V-core drops all of 200V

- AV across 200V (305 devices on 292 ports) is unavailable while its ports are moved

- 37 APs and their Wi-Fi clients drop while the AP ports are moved

- Wi-Fi users at 200V drop and reconnect when the 200v WLAN group is changed; the Trinity Staff SSID is removed from the 200V APs (no clients on 6 October 2026)

- CCTV recording from 4 cameras has a gap while their ports are moved

- 6 printers are offline while their ports are moved

- 1 building access device (cloe-000-200v, lv6 4/1/24) is offline while its port is moved

- No other ports, VLANs or services are changed

Duration of impact

Each device is offline only while its port is moved and bounced (disabled for 10 seconds, then enabled), and until it is back on the network with an address on its new VLAN. All work takes place within the change window under Suggested Scheduling. Each switch reload for the system-max change takes a few minutes and happens once, before the cutover on that switch.

How Change implemented

The change will be implemented by Alam Singh (Dycom) over SSH to each switch, and over a console cable on site for switch-200V-Tutorial-6.03, which has no remote access, using the prepared configuration file 200V-cutover-vlan-migration-ICX.txt. That file has pre-check, change, port bounce, verification and rollback blocks for each switch, and its change and rollback blocks have been checked against each switch's current configuration. The port bounce disables and re-enables each port so DHCP devices request a new address; it does not remove PoE. On lv3, lv4, lv6 and LT2-1 the file also adds VLAN 2900 to the AP ports (see Wi-Fi). Before the cutover, the system-max change (system-max-spanning-tree-40-ICX.txt) is made and each switch reloaded. The switches are done one at a time: LT2-1 first, then lv3, lv4 and lv6, then Tutorial-6.03. 14 devices had their port taken from the 200V MAC table rather than the master list (marked * in Appendix A); these are checked in the pre-check.

Prerequisites (status at 9 October 2026)

| Prerequisite | Status |
| --- | --- |
| VLANs 2010, 2300, 2510, 2600, 2700, 2800 and 2900 exist on each switch and are tagged on its uplink to the core | Complete |
| system-max spanning-tree 40 set and reloaded on all switches, and spanning-tree enabled on 2600 and 2700 on lv6 (system-max-spanning-tree-40-ICX.txt) | Complete (9 October 2026) |
| Running-config backup of each switch taken (via SmartZone, 6 October 2026) | Complete |
| WLANs Trinity Wireless v2300, eduroam v2900 and Trinity Visitor v2900 exist in SmartZone | Complete |
| Current 200v WLAN group recorded (WLANs and any VLAN overrides) for rollback | Pending |
| Review tab items decided (ports filled in from the MAC table, devices not found, unlisted devices) | Pending |
| Console cable and on-site access arranged for switch-200V-Tutorial-6.03 | Complete |
| Trinity has confirmed the DHCP reservations are in place on the new subnets | Pending |
| Change approved | Pending |

If a switch fails verification, that switch is rolled back before the next one is started.

How rollback implemented

Each switch is rolled back with its ROLLBACK block from the configuration file, which puts each port back on its old VLAN. A single failed device doesn't need a whole-switch rollback: only that port's lines are used.

The system-max change has its own rollback block for each switch in system-max-spanning-tree-40-ICX.txt; it also needs a reload to take effect. The Wi-Fi change is rolled back in SmartZone by removing the three new WLANs from the 200v WLAN group and adding the four removed WLANs back with their recorded settings. The VLAN 2900 tagging on the AP ports can stay in place; it has no effect until an SSID uses it.

Roll back a switch if

- a device's MAC doesn't show up on its new VLAN within 10 minutes of the port bounce

- an AP doesn't show up in SmartZone within 15 minutes of its port moving

- anything unexpected drops, such as an uplink or devices not in Appendix A

- Trinity asks for a rollback

Rollback steps

- Paste that switch's ROLLBACK block (or just the lines for the affected port).

- Paste the ROLLBACK PORT BOUNCE block.

- Check the ports are back on their old VLAN and the MACs have been learned, then run write memory.

- Record the affected ports and the cause in the change record.

How change tested

A port passes when all three of these are true:

- show vlan <new VLAN> lists the port as untagged

- AP and other trunk ports still carry the same tagged VLANs as before, plus VLAN 2900 on the AP ports

- show mac-address vlan <new VLAN> lists the device's MAC on that port

For Wi-Fi, after the WLAN group change a test client on each SSID connects and gets an address on its subnet: Trinity Wireless and Trinity NetSegPilot on 10.21.8.0/21 (VLAN 2300), Trinity Visitor and eduroam on 10.21.29.0/24 (VLAN 2900).

The change is complete when every device in Appendix A passes, the Wi-Fi test passes, and write memory has been run on all five switches.

Risks of implementing

- If a port in the device list is wrong, then the wrong device is moved. This has low likelihood because each switch's ports were checked against its current configuration on 6 October 2026, and the 14 ports taken from the MAC table are checked again in the pre-check. If this risk occurs, that port is left out or moved back.

- If lv6 is changed before its spanning-tree limit is raised, then VLANs 2600 and 2700 run without spanning-tree and a patching loop on them would not be blocked. This has low likelihood if the system-max change is done first, as listed in the prerequisites. If it cannot be done first, lv6 is done after the other switches and its ports are checked for loops after the change.

- If an AP cannot reach SmartZone from VLAN 2010, then it and its Wi-Fi clients stay offline. This has low likelihood because both pilot APs joined SmartZone on 2010. If this risk occurs, the AP port is rolled back.

- If a device does not ask for a new address after its port is moved, then it stays unreachable on its old address. This has low likelihood because every port is bounced after the change. If this risk occurs, the device is restarted or its port is rolled back.

- If VLAN 2900 is missing on an AP port, then Trinity Visitor and eduroam clients on that AP get no address. This has low likelihood because VLAN 2900 is added to all 40 AP ports and checked with show vlan 2900 before the WLAN changeover. If this risk occurs, VLAN 2900 is tagged on that port.

- If a paste into the switch is cut short, then some ports are not moved. This has low likelihood because consecutive ports are grouped into ranges, which keeps each block short, and every port is checked in the verify step. If this risk occurs, the missing lines are pasted again.

Suggested Scheduling

Outage window: Friday 9 October 2026, 6:00 pm to Sunday 11 October 2026, 5:00 pm.

All switch changes are made on Friday 9 October 2026 from 6:00 pm, so the other teams can start their work on Saturday morning. The system-max change and reload of each switch are done first, then the VLAN cutover on each switch in the order above. The 200v WLAN group change in SmartZone follows the switch changes in the same window.

Note: the pilot (60 ports on lv3, lv4 and lv6) was completed and verified on Monday 5 October 2026.

Cutover record – 9 October 2026

The system-max change and the VLAN cutover were made on all switches on Friday 9 October 2026. Every switch came back after its reload with spanning-tree current at 40, and the core saw all six access switches on both links of each LAG. All ports in the configuration file were moved.

| Switch | Result |
| --- | --- |
| trinity200V-core | Reloaded for system-max 40. WIFIMGMT-IN ACL: three entries added (sequences 23, 24 and 25; unplanned, see below) |
| switch-200V-LT2-1 | Cutover done, 16 ports; VLAN 2900 added on 1/1/1 |
| switch-200V-stack-lv6 | Cutover done, 145 ports; spanning-tree enabled on 2600 and 2700. Units 3 and 4 left the stack twice (20:03 and 22:44 on 9 October); see below. All 7 units Ready again from 13:35 on 10 October. Config saved |
| switch-200V-stack-lv3 | Cutover done, 84 ports |
| switch-200V-stack-lv4 | Cutover done, 89 ports |
| switch-200V-Tutorial-6.03 | Cutover done over console, 6 ports plus 1/1/3 and 1/1/9 (see below) |
| switch-200V-LT2-2, switch-200V-LT2-3, switch-200V-8 | system-max change only (LT2-3 1/1/4 later moved; see Review tab changes) |
| SmartZone 200v WLAN group | Changed to eduroam v2900, Trinity Visitor v2900 and Trinity Wireless v2300, all with VLAN override Default. Trinity Wireless working after the WIFIMGMT-IN change |

Deviations from the plan

- Unplanned change on trinity200V-core: APs moved to VLAN 2010 kept dropping off SmartZone because the WIFIMGMT-IN ACL denied the SmartZone private node addresses. Two entries were added: sequence 23 permit ip 10.21.1.0 0.0.0.255 host 192.168.81.100 and sequence 24 permit ip 10.21.1.0 0.0.0.255 host 192.168.245.100. The APs were stable afterwards. The two pilot APs (r650-3d-98-40 on lv3 1/1/6 and r650-39-4b-f0 on lv6 1/1/19) had been on VLAN 2010 since 5 October and were flapping in SmartZone for the same reason; the same entries fixed them

- Unplanned change on trinity200V-core: Trinity Wireless clients could not authenticate. The SmartZone AAA server for the WLAN (pause-radius03 Trinity Wireless, FreeRADIUS at 10.128.131.7, UDP 18200) is not proxied, so the APs send RADIUS requests directly from 10.21.1.0/24, and WIFIMGMT-IN denied them (sequence 30 denies 10.0.0.0/8; only 10.128.128.0/24 was permitted). Sequence 25 permit udp 10.21.1.0 0.0.0.255 host 10.128.131.7 was added. Two example entries for 10.20.16.10 (UDP 1812 and 1813) were pasted in error at first and removed straight away. The path was checked with ping and traceroute from 10.21.1.1 (via 10.20.8.1, 10.136.0.2 and 10.128.16.6), and Trinity Wireless then authenticated

- Before the authentication issue, Trinity Wireless v2300 clients were reported as getting no address. The 200v WLAN group was checked and all VLAN overrides are Default (Trinity Wireless v2300 on VLAN 2300)

- VLAN 2900 was missing on the LT2-1 AP port at the start and was added before that AP was moved

- lv6: units 3 and 4 left the stack at 20:03 on 9 October during the cutover and rejoined at 20:44 after a restart. They left again at 22:44 the same night and stayed out until unit 3 was power cycled at 13:35 on 10 October (about 15 hours). See the lv6 stack section below

- lv6: printer 200v-lvl-6-staff-corridor (7/1/19) briefly ended up on VLAN 2700 after a cut-short paste and was moved to 2800 the same night

- On lv6, lv4 and Tutorial-6.03 the port enable step was missed at first and re-run

- Tutorial-6.03: ports 1/1/3 and 1/1/9 had devices on VLAN 1230 that were not in the device list (1/1/9 is the AV Switcher 00:1D:56:0A:DF:C0). Both were moved to VLAN 2700 on site

- Trinity NetSegPilot is not in the 200v WLAN group, although the plan was to keep it

Review tab changes

After the cutover, the ports that the Review tab comments gave a new VLAN were also moved, using 200V-review-changes-ICX.txt. 18 of the 20 ports were moved and saved; each switch was pre-checked and verified.

| Switch | Ports moved |
| --- | --- |
| switch-200V-LT2-3 | 1/1/4: 1240 to 2100 |
| switch-200V-stack-lv3 | 3/1/5, 3/1/31, 3/1/33, 3/1/43, 4/1/28: 1240 to 2100; 3/1/29, 4/1/29: 230 to 2530 |
| switch-200V-stack-lv4 | 3/1/7, 3/1/28: 1240 to 2100 |
| switch-200V-stack-lv6 | 1/1/31, 3/1/1, 4/1/1, 5/1/30, 5/1/48, 6/1/16, 6/1/22, 6/1/31: 1240 to 2100 (1/1/31 keeps 1272 and 1273 tagged) |

- Devices seen on their new VLAN: lv3 3/1/29 and 4/1/29 (2530); lv6 5/1/30, 6/1/16 and 6/1/31 (2100)

- Link up but no MAC learned yet, most likely idle devices: lv3 3/1/33 and 3/1/43; lv6 1/1/31, 4/1/1 and 5/1/48

- No link (nothing connected): LT2-3 1/1/4; lv3 3/1/5, 3/1/31 and 4/1/28; lv4 3/1/7 and 3/1/28; lv6 3/1/1 and 6/1/22. These devices get an address on the new VLAN when they next connect

- Not moved: lv3 1/1/48 and lv4 1/1/48 (00:09:F5 devices on VLAN 100) were left on VLAN 100 because it is not known whether they have a fixed address. They move to VLAN 2000 (10.21.0.0/24) once that is confirmed

16 further devices with near-consecutive MACs D8:43:AE:B1:xx:xx, one per room on the old AV VLAN 1230 and not in the device list, were moved to VLAN 2700 with the rest of the room AV using 200V-av-unlisted-2700-ICX.txt: lv3 2/1/16, 2/1/29, 4/1/41, 4/1/46, 5/1/18, 5/1/24, 5/1/26 and 5/1/45; lv4 1/1/14, 1/1/28, 2/1/21, 4/1/6, 5/1/7, 5/1/12, 5/1/21 and 5/1/29 (Review items 26, 27, 35-42, 44, 47 and 50-53).

On 10 October 2026, 13 outlets with devices 20:7B:D2:30:xx:xx were moved from VLAN 1230 to VLAN 2100 (staff wired) using 200V-207bd2-2100-ICX.txt. The ports were found from the outlet labels in the Dycom patching sheet (Switch Physical Patching, 30 July 2025): lv3 2/1/17, 4/1/43, 5/1/11, 5/1/23, 5/1/29, 5/1/40 and 5/1/47; lv4 1/1/15, 1/1/21, 1/1/30, 1/1/34, 4/1/4 and 4/1/26. All were untagged on 1230 with no device in the device list. The change was applied and saved on both switches; the devices were not tested.

Also on 10 October 2026, after reviewing show interfaces brief on lv3, lv4 and lv6 (Old VLAN ports tab of the spreadsheet): lv4 4/1/30 (F8:25:51:CC:2C:76, Review item 49) and lv4 4/1/19 (Airtame 384b.76e0.b0b1, Review item 48) were moved to VLAN 2700, and two live staff PCs, lv3 3/1/20 (1c1a.dfc2.5143) and lv4 3/1/1 (1c1a.dfc2.4cc3), were moved from 1240 to 2100. That review found no device from the device list still on an old VLAN; 237 ports with no listed device remain on old VLANs, almost all with no link.

Issues raised after the cutover (10 October 2026)

- Symphony CPX could not reach 200V AV devices on TCP 80 and 443 (tested to 10.21.37.14), while ping worked and the Mystro VC4 server connected. A packet capture on the CPX server showed it sends from 10.128.1.4, not 10.128.128.4, with SYNs going out and no reply. AV-IN on VLAN 2700 permits 10.128.128.0/24 (sequence 20) but denies other 10.0.0.0/8 destinations (sequence 30), so the AV devices' replies to 10.128.1.4 are dropped at the 200V core (ping works because sequence 23 permits echo-replies to any). Fix: AV-IN sequence 24 permit ip 10.21.36.0 0.0.1.255 host 10.128.1.4. ACL accounting on this build counts per ACL, not per entry, so it could not isolate the traffic

- Projector Screen 04:91:62:D7:90:01 (outlet 200V2K08, lv6 4/1/5, new address 10.21.37.43) was moved to VLAN 2700 in the cutover but was not working on 10 October. Port 4/1/5 is on lv6 unit 4, which was out of the stack from 22:44 on 9 October to 13:35 on 10 October (see below), so the port had no link; the device itself was not the cause. If it is still not reachable after the stack recovery and it has a fixed address (old 10.16.12.135), it needs setting to DHCP or to 10.21.37.43/23, gateway 10.21.36.1

lv6 stack: unit 3 failures

lv6 is a seven-unit ICX8200 stack cabled as a chain (4 - 3 - 2 - 1 - 5 - 6 - 7); unit 1 is active (priority 128) and unit 2 standby (110). Unit 4 reaches the stack only through unit 3, so when unit 3 drops, unit 4 drops with it.

| Time | Event (from show logging and show stack) |
| --- | --- |
| Fri 9 Oct 20:03 | Units 3 and 4 deleted from the stack during the cutover |
| Fri 9 Oct 20:44 | Units 3 and 4 rejoined after a restart |
| Fri 9 Oct 22:44:19 | Units 3 and 4 deleted again in the same second. All their ports went down, including unit 3's stack ports (3/2/1, 3/2/3) and uplink LAG member 3/2/2. No stack-port flap, power or temperature message before it |
| Sat 10 Oct 13:35 | Unit 3 power cycled on site; units 3 and 4 rejoined. Unit 3 uptime then 5 minutes, unit 4 20 hours (unit 4 never restarted) |

- Impact while out: all ports on units 3 and 4 had no link (AV on 3/1/11-48 and 4/1/4-23, building access on 4/1/24, staff ports) and lv6's uplink LAG ran on one 10G link (1/2/4) instead of two

- Assessment: unit 3 (ICX8200-48PF2, serial FNM4319U09A, FastIron 10.0.10g_cd3) hangs without warning and only recovers with a restart. This points to the unit itself rather than the stack cable

- SmartZone shows the lv6 switch online but with 5 stack members, not 7, after units 3 and 4 rejoined. A manager session reset (manager disable / no manager disable) did not refresh it. The switch itself shows all 7 units Ready; this is a SmartZone display issue only

- Unit IDs and priorities stay as they are: active and standby (units 1 and 2) are correct, and renumbering would change every port name used in the configuration and records

Port clean-up and switch management – 26 October 2026

A second change (CIP: Trinity 200V Port Clean-up Change.docx, approved 26 October) was implemented on Monday 26 October 2026, 6:00 pm - 11:00 pm, on all eight access switches. trinity200V-core was not changed.

- Part A, port clean-up: 267 access ports changed from Trinity's switch port export of 16 October (column Q). 173 ports 1240 to 2100; 29 ports 1240 to 2100 with old tagged VLANs removed; 3 UPS cards on switch-200V-8 1/1/7 - 1/1/9 to 2000; 61 unused ports to VLAN 1 (left enabled); LT2-2 1/1/1 old VLANs removed (tagged-only with 2000 - 2953). Config: 200V-port-review-16oct-ICX.txt. The workbook tab 'Port clean-up 26 Oct' lists every port

- The three remaining 20:7B:D2:30 devices were found on lv3 2/1/27, lv4 4/1/32 and lv4 5/1/19 (not on the outlets in the patching sheet) and moved to 2100 with Trinity's agreement

- Part B, switch management: each access switch moved from ve 100 (192.168.122.x) to ve 2000 on VL-netmgmt-01: lv6 10.21.0.20, LT2-1 .21, LT2-2 .22, LT2-3 .23, Tutorial-6.03 .24, switch-200V-8 .25, lv4 .26, lv3 .27; default route via 10.21.0.1, management source-interface ve 2000, old address removed. All eight reconnected to SmartZone. Config: 200V-mgmt-ve2000-ICX.txt

- During Part B: the core cannot telnet or SSH to the switches, so the SmartZone CLI and the admin jump host were used; SmartZone accepted a switch only after its old address was removed and its agent restarted; the 10-minute reload timer rolled lv3 back once (7:06 pm) and the second attempt succeeded

Open items

- Apply AV-IN sequence 24 for the CPX server (10.128.1.4) if not yet done, write memory, and have Symphony retest TCP 80/443. Decide whether all of 10.128.1.0/24 should be permitted on the new VLANs' ACLs, as only 10.128.128.0/24 is permitted today

- Projector Screen on lv6 4/1/5: after the AV team's power cycle, confirm 0491.62d7.9001 on VLAN 2700 and that 10.21.37.43 answers

- Trinity Wireless: confirm clients get a 10.21.8.x address, and confirm whether any change was also made on the FreeRADIUS server (clients.conf) or the firewall at 10.136.0.2 for 10.21.1.0/24, so it can be recorded

- eduroam v2900 and Trinity Visitor v2900: test both (10.21.29.x). If eduroam uses a RADIUS server outside 10.128.128.0/24, 192.168.12.0/24 and 10.128.131.7, WIFIMGMT-IN needs an entry for it as well

- Confirm whether Trinity NetSegPilot should be added back to the 200v WLAN group

- Test every SSID with a client and confirm all 200V APs, including lv4, are online in SmartZone

- Confirm write memory on lv4 and Tutorial-6.03 (lv3, lv6, LT2-3 and the core are saved)

- Check show span vlan 2600 on lv6 (lg10 forwarding)

- AV team to check on Saturday six AV devices not yet seen on VLAN 2700: Projector Screen 1 and 2 (LT2-1 1/1/16, 1/1/15), Projector Screen Left (lv4 4/1/41), Projector Right and Left (lv4 5/1/37, 5/1/39) and the PDU (Tutorial-6.03 1/1/8)

- Add the two Tutorial-6.03 devices on 1/1/3 and 1/1/9 to the master list

- Check the Review tab ports with link but no MAC (show mac-address vlan 2100 on lv3 and lv6)

- Confirm the addressing of the devices on lv3 1/1/48 and lv4 1/1/48, then move them to VLAN 2000

- Room NUC-type devices not on 2700: LT2-1 1/1/21 (CLA-TAN3130059J, still on 1240) and lv6 6/1/31 (48:21:0B:56:AE:44, moved to 2100 per the Review comment). Confirm with the AV team and move to 2700 if they are room NUCs

- Done 26 October: the 20:7B:D2:30 devices for outlets 200V3J14, 200V4I17 and 200V4H9 were found on lv3 2/1/27, lv4 5/1/19 and lv4 4/1/32 and moved to 2100 (the patching-sheet ports on 2700 are unchanged)

- Other unlisted AV-side device still on an old VLAN: LT2-1 1/1/9 (VMware MAC, 1230)

- Done 26 October: all remaining 1240 access ports moved to 2100 (port clean-up). Check the device on lv6 1/1/32 (1C:1A:DF:C2:4F:5E), which had no MAC on 2100 when checked

- Trinity to check the new DHCP leases of the UPS cards on switch-200V-8 (10.21.0.0/24) and update UPS monitoring

- Update the device list, monitoring and documentation for the switches' new management addresses (10.21.0.20 - .27)

- Later change: remove the legacy VLANs (100, 230, 1212, 1213, 1230, 1240, 1250, 1271 - 1273) from the uplinks, AP ports and the core

- lv6 (urgent): close the stack into a ring between unit 4 and unit 7. Configure 4/2/3 and 7/2/3 as stack ports (stack unit 4: stack-port 4/2/1 4/2/3; stack unit 7: stack-port 7/2/1 7/2/3), then cable 4/2/3 to 7/2/3, outside business hours. Unit 4 then stays up if unit 3 fails again

- lv6: open a Ruckus TAC case for unit 3 (FNM4319U09A) with show logging, show tech-support and show version, and ask for an RMA. Include the SmartZone 5-member display issue

- lv6: until unit 3 is replaced, check show stack on the switch (not SmartZone) at each check; if units 3 and 4 are missing, power cycle unit 3. Reseat or replace the unit 2 to unit 3 stack cable (2/2/3 to 3/2/1) while on site

- Later window: close the lv4 stack into a ring, and lower the spanning-tree priority on trinity200V-core (switch-200V-8 is currently root)

Appendix A – Cutover devices

353 devices on 340 ports across five switches: 16 on LT2-1, 84 on lv3, 89 on lv4, 145 on lv6, 6 on Tutorial-6.03. Port numbers are unit/module/port. * = port taken from the 200V MAC table, not the master list. Devices that share a port are listed on separate rows.

| Switch | Port | Old VLAN | New VLAN | Type | Device | Location | MAC | New IP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LT2-1 | 1/1/1 | 100 | 2010 | AP | r650-39-49-a0 |  | C8:A6:08:39:49:A0 | 10.21.1.33 |
| LT2-1 | 1/1/4 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 6.13 Lecture Theatre | D0:C8:57:81:64:24 | 10.21.37.85 |
| LT2-1 | 1/1/5 | 1272 | 2700 | AV | Airtame | 200V - 6.13 Lecture Theatre | 38:4b:76:f1:8f:ec | 10.21.37.78 |
| LT2-1 | 1/1/6 | 1230 | 2700 | AV | PDU * | 200V - 6.13 Lecture Theatre | B8-80-4F-28-58-24 | 10.21.37.84 |
| LT2-1 | 1/1/7 | 1230 | 2700 | AV | Recorder * | 200V - 6.13 Lecture Theatre | 00-1C-08-00-6C-7C | 10.21.37.91 |
| LT2-1 | 1/1/8 | 1230 | 2700 | AV | Audio DSP * | 200V - 6.13 Lecture Theatre | 78-45-01-0C-7D-3A | 10.21.37.79 |
| LT2-1 | 1/1/10 | 1230 | 2700 | AV | LCD 2 * | 200V - 6.13 Lecture Theatre | A8-4A-63-30-73-66 | 10.21.37.82 |
| LT2-1 | 1/1/11 | 1230 | 2700 | AV | LCD 1 * | 200V - 6.13 Lecture Theatre | A8-4A-63-30-73-67 | 10.21.37.81 |
| LT2-1 | 1/1/12 | 1230 | 2700 | AV | PTZ Camera * | 200V - 6.13 Lecture Theatre | DC-E2-AC-02-E4-F5 | 10.21.37.90 |
| LT2-1 | 1/1/13 | 1230 | 2700 | AV | Room Camera * | 200V - 6.13 Lecture Theatre | B8-A4-4F-69-B2-72 | 10.21.37.92 |
| LT2-1 | 1/1/14 | 1230 | 2700 | AV | Occupancy Sensor * | 200V - 6.13 Lecture Theatre | B8-98-B0-0F-1C-66 | 10.21.37.83 |
| LT2-1 | 1/1/15 | 1230 | 2700 | AV | Projector Screen 2 * | 200V - 6.13 Lecture Theatre | 04-91-62-D7-70-0D | 10.21.37.89 |
| LT2-1 | 1/1/16 | 1230 | 2700 | AV | Projector Screen 1 * | 200V - 6.13 Lecture Theatre | 54-10-EC-A2-E8-27 | 10.21.37.88 |
| LT2-1 | 1/1/17 | 1230 | 2700 | AV | Projector 1 * | 200V - 6.13 Lecture Theatre | 38-1A-52-B1-75-B6 | 10.21.37.86 |
| LT2-1 | 1/1/18 | 1230 | 2700 | AV | Projector 2 * | 200V - 6.13 Lecture Theatre | 38-1A-52-B1-74-BA | 10.21.37.87 |
| LT2-1 | 1/1/20 | 1230 | 2700 | AV | Touch Panel * | 200V - 6.13 Lecture Theatre | C4-42-68-26-D6-48 | 10.21.37.93 |
| lv3 | 1/1/1 | 100 | 2010 | AP | r650-19-9d-00 |  | C8:A6:08:19:9D:00 | 10.21.1.21 |
| lv3 | 1/1/2 | 100 | 2010 | AP | r650-19-9d-70 |  | C8:A6:08:19:9D:70 | 10.21.1.25 |
| lv3 | 1/1/3 | 100 | 2010 | AP | r650-19-9d-50 |  | C8:A6:08:19:9D:50 | 10.21.1.23 |
| lv3 | 1/1/4 | 100 | 2010 | AP | r650-19-9d-40 |  | C8:A6:08:19:9D:40 | 10.21.1.22 |
| lv3 | 1/1/5 | 100 | 2010 | AP | r650-19-9d-80 |  | C8:A6:08:19:9D:80 | 10.21.1.26 |
| lv3 | 1/1/7 | 100 | 2010 | AP | r650-32-68-f0 |  | CC:1B:5A:32:68:F0 | 10.21.1.49 |
| lv3 | 1/1/8 | 100 | 2010 | AP | r650-32-61-e0 |  | CC:1B:5A:32:61:E0 | 10.21.1.47 |
| lv3 | 1/1/9 | 100 | 2010 | AP | r650-3d-98-50 |  | 5C:83:6C:3D:98:50 | 10.21.1.12 |
| lv3 | 1/1/10 | 100 | 2010 | AP | r650-19-9d-a0 |  | C8:A6:08:19:9D:A0 | 10.21.1.27 |
| lv3 | 1/1/11 | 100 | 2010 | AP | r650-19-9d-60 |  | C8:A6:08:19:9D:60 | 10.21.1.24 |
| lv3 | 1/1/13 | 1230 | 2700 | AV | Touch Panel | 200V - 3.20 Meeting Room | C4:42:68:60:C1:B6 | 10.21.36.79 |
| lv3 | 1/1/14 | 1230 | 2700 | AV | Room Camera | 200V - 3.10 | B8:A4:4F:CB:CB:F7 | 10.21.36.71 |
| lv3 | 1/1/15 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.10 | C4:42:68:65:7E:7F | 10.21.36.68 |
| lv3 | 1/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 3.10 | C4:42:68:6C:42:DC | 10.21.36.72 |
| lv3 | 1/1/17 | 1230 | 2700 | AV | Room Camera | 200V - 3.09 | B8:A4:4F:CB:C6:FB | 10.21.36.64 |
| lv3 | 1/1/18 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.09 | C4:42:68:65:8E:04 | 10.21.36.61 |
| lv3 | 1/1/19 | 1230 | 2700 | AV | Touch Panel | 200V - 3.09 | C4:42:68:6C:44:8E | 10.21.36.65 |
| lv3 | 1/1/20 | 1230 | 2700 | AV | Room Camera | 200V - 3.08 | B8:A4:4F:CE:F4:C6 | 10.21.36.57 |
| lv3 | 1/1/21 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.08 | C4:42:68:65:7D:A0 | 10.21.36.54 |
| lv3 | 1/1/22 | 1230 | 2700 | AV | Touch Panel | 200V - 3.08 | C4:42:68:6D:2B:F1 | 10.21.36.58 |
| lv3 | 1/1/23 | 1230 | 2700 | AV | Touch Panel | 200V - 3.06 | C4:42:68:6D:C3:79 | 10.21.36.51 |
| lv3 | 1/1/24 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.06 | C4:42:68:65:91:9B | 10.21.36.47 |
| lv3 | 1/1/25 | 1230 | 2700 | AV | Room Camera | 200V - 3.06 | B8:A4:4F:CE:E5:EB | 10.21.36.50 |
| lv3 | 1/1/26 | 1230 | 2700 | AV | Room Camera | 200V - 3.05 | B8:A4:4F:CB:CA:CC | 10.21.36.43 |
| lv3 | 1/1/27 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.05 | C4:42:68:65:95:E7 | 10.21.36.40 |
| lv3 | 1/1/28 | 1230 | 2700 | AV | Touch Panel | 200V - 3.05 | C4:42:68:6D:C3:9E | 10.21.36.44 |
| lv3 | 1/1/29 | 1230 | 2700 | AV | Touch Panel | 200V - 3.04 | C4:42:68:6D:C3:AB | 10.21.36.37 |
| lv3 | 1/1/30 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.04 | C4:42:68:65:8F:C4 | 10.21.36.33 |
| lv3 | 1/1/31 | 1230 | 2700 | AV | Room Camera | 200V - 3.04 | B8:A4:4F:CE:E4:99 | 10.21.36.36 |
| lv3 | 1/1/32 | 1230 | 2700 | AV | Room Camera | 200V - 3.03 | B8:A4:4F:CB:C8:54 | 10.21.36.29 |
| lv3 | 1/1/33 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.03 | C4:42:68:65:91:74 | 10.21.36.23 |
| lv3 | 1/1/35 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.15 Consult Room | C4:42:68:1C:22:3B | 10.21.36.75 |
| lv3 | 1/1/36 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.14 Consult Room | C4:42:68:46:A3:3F | 10.21.36.73 |
| lv3 | 2/1/13 | 1230 | 2700 | AV | Projector | 200V - 3.03 | F8:25:51:D2:70:8E | 10.21.36.26 |
| lv3 | 2/1/15 | 1230 | 2700 | AV | Touch Panel | 200V - 3.03 | C4:42:68:6D:BF:ED | 10.21.36.30 |
| lv3 | 2/1/18 | 1272 | 2700 | AV | Airtame | 200V - 3.03 | 38:4B:76:F1:83:52 | 10.21.36.20 |
| lv3 | 2/1/19 | 1230 | 2700 | AV | PDU | 200V - 3.03 | 28:B5:E8:6F:27:18 | 10.21.36.24 |
| lv3 | 2/1/20 | 1230 | 2700 | AV | AV Switcher | 200V - 3.03 | C4:42:68:65:35:E8 | 10.21.36.21 |
| lv3 | 2/1/21 | 1230 | 2700 | AV | Room Camera | 200V - 3.02 | B8:A4:4F:CB:C5:56 | 10.21.36.18 |
| lv3 | 2/1/22 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.02 | C4:42:68:65:87:C4 | 10.21.36.15 |
| lv3 | 2/1/23 | 1230 | 2700 | AV | Projector | 200V - 3.02 | F8:25:51:CC:0F:7D | 10.21.36.17 |
| lv3 | 2/1/25 | 1230 | 2700 | AV | PDU | 200V - 3.02 | 28:B5:E8:6F:CB:2E | 10.21.36.16 |
| lv3 | 2/1/26 | 1230 | 2700 | AV | Touch Panel | 200V - 3.02 | C4:42:68:6D:C3:9C | 10.21.36.19 |
| lv3 | 2/1/28 | 1272 | 2700 | AV | AV Switcher | 200V - 3.02 | C4:42:68:65:39:D8 | 10.21.36.14 |
| lv3 | 2/1/30 | 1272 | 2700 | AV | Airtame | 200V - 3.02 | 38:4B:76:F1:83:51 | 10.21.36.13 |
| lv3 | 2/1/31 | 1230 | 2700 | AV | LCD | 200V - 3.01 Collaboration Space | F8:01:B4:53:86:B8 | 10.21.36.12 |
| lv3 | 2/1/32 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.01 Collaboration Space | C4:42:68:59:A4:45 | 10.21.36.11 |
| lv3 | 2/1/33 | 1272 | 2700 | AV | Airtame | 200V - 3.01 Collaboration Space | 38:4B:76:F1:95:78 | 10.21.36.10 |
| lv3 | 2/1/35 | 1230 | 2700 | AV | PTZ Camera | 200V - 3.03 | DC:E2:AC:02:E8:CA | 10.21.36.27 |
| lv3 | 2/1/36 | 1230 | 2700 | AV | Mic Charger | 200V - 3.03 | 00:1B:66:3F:15:17 | 10.21.36.22 |
| lv3 | 2/1/37 | 1230 | 2700 | AV | Recorder | 200V - 3.03 | 00:1C:08:00:70:A3 | 10.21.36.28 |
| lv3 | 2/1/38 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 3.03 | D0:C8:57:80:9F:8E | 10.21.36.25 |
| lv3 | 2/1/39 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.16 Consult Room | C4:42:68:46:82:ED | 10.21.36.76 |
| lv3 | 2/1/40 | 1230 | 2700 | AV | Cresnet Bridge For Outside Sign | 200V - 3.15 Consult Room | C4:42:68:5D:43:1D | 10.21.36.74 |
| lv3 | 4/1/4 | 1240 | 2800 | printer | 200v-lvl3-staffroom |  | 34:9F:7B:5E:9A:04 | 10.21.27.5 |
| lv3 | 4/1/40 | 1230 | 2700 | AV | AV Switcher | 200V - 3.10 | C4:42:68:65:29:8D | 10.21.36.67 |
| lv3 | 4/1/42 | 1230 | 2700 | AV | PDU | 200V - 3.10 | 28:B5:E8:68:78:AF | 10.21.36.69 |
| lv3 | 4/1/44 | 1230 | 2700 | AV | Projector | 200V - 3.10 | F8:25:51:D2:70:8A | 10.21.36.70 |
| lv3 | 4/1/47 | 1272 | 2700 | AV | Airtame | 200V - 3.09 | 8C:AE:4C:D2:7C:B8 | 10.21.36.59 |
| lv3 | 4/1/48 | 1230 | 2700 | AV | PDU | 200V - 3.09 | 28:B5:E8:6F:33:1E | 10.21.36.62 |
| lv3 | 5/1/1 | 1230 | 2700 | AV | LCD | 200V - 3.20 Meeting Room | 74:C1:7E:EC:3D:90 | 10.21.36.78 |
| lv3 | 5/1/2 | 1230 | 2700 | AV | UC Engine | 200V - 3.20 Meeting Room | 90:8D:6E:95:8E:ED | 10.21.36.80 |
| lv3 | 5/1/3 | 1272 | 2700 | AV | Airtame | 200V - 3.20 Meeting Room | 38:4B:76:F1:AB:19 | 10.21.36.77 |
| lv3 | 5/1/8 | 1230 | 2700 | AV | Display | 200V - L3 Digital Signage 2 | 28:E6:A9:8D:5A:69 | 10.21.37.95 |
| lv3 | 5/1/9 | 1272 | 2700 | AV | Airtame | 200V - 3.10 | 8C:AE:4C:D2:7C:BC | 10.21.36.66 |
| lv3 | 5/1/10 | 1230 | 2700 | AV | AV Switcher | 200V - 3.09 | C4:42:68:65:3F:BD | 10.21.36.60 |
| lv3 | 5/1/12 | 1230 | 2700 | AV | Projector | 200V - 3.09 | F8:25:51:CD:3F:DA | 10.21.36.63 |
| lv3 | 5/1/14 | 1230 | 2700 | AV | Projector | 200V - 3.08 | F8:25:51:CF:D3:D8 | 10.21.36.56 |
| lv3 | 5/1/16 | 1230 | 2700 | AV | PDU | 200V - 3.08 | 28:B5:E8:6F:CB:5E | 10.21.36.55 |
| lv3 | 5/1/17 | 1272 | 2700 | AV | Airtame | 200V - 3.08 | 8C:AE:4C:D2:7C:D8 | 10.21.36.52 |
| lv3 | 5/1/19 | 1230 | 2700 | AV | Projector | 200V - 3.05 | F8:25:51:D2:70:34 | 10.21.36.42 |
| lv3 | 5/1/21 | 1272 | 2700 | AV | Airtame | 200V - 3.05 | 38:4B:76:F1:95:77 | 10.21.36.38 |
| lv3 | 5/1/22 | 1230 | 2700 | AV | PDU | 200V - 3.05 | 28:B5:E8:68:7E:2B | 10.21.36.41 |
| lv3 | 5/1/25 | 1230 | 2700 | AV | AV Switcher | 200V - 3.05 | C4:42:68:65:3F:74 | 10.21.36.39 |
| lv3 | 5/1/27 | 1272 | 2700 | AV | Airtame | 200V - 3.04 | 38:4B:76:F1:95:75 | 10.21.36.31 |
| lv3 | 5/1/28 | 1230 | 2700 | AV | PDU | 200V - 3.04 | 28:B5:E8:6F:4A:58 | 10.21.36.34 |
| lv3 | 5/1/30 | 1230 | 2700 | AV | AV Switcher | 200V - 3.04 | C4:42:68:65:3F:93 | 10.21.36.32 |
| lv3 | 5/1/31 | 1230 | 2700 | AV | Projector | 200V - 3.04 | F8:25:51:CD:3F:E4 | 10.21.36.35 |
| lv3 | 5/1/33 | 1230 | 2700 | AV | Display | 200V - L3 Digital Signage 1 | 28:E6:A9:8D:5A:60 | 10.21.37.94 |
| lv3 | 5/1/41 | 1230 | 2700 | AV | AV Switcher | 200V - 3.08 | C4:42:68:65:38:8A | 10.21.36.53 |
| lv3 | 5/1/42 | 1230 | 2700 | AV | Projector | 200V - 3.06 | F8:25:51:D2:70:90 | 10.21.36.49 |
| lv3 | 5/1/44 | 1230 | 2700 | AV | PDU | 200V - 3.06 | 28:B5:E8:6F:D1:0D | 10.21.36.48 |
| lv3 | 5/1/46 | 1272 | 2700 | AV | Airtame | 200V - 3.06 | 8C:AE:4C:D2:00:95 | 10.21.36.45 |
| lv3 | 5/1/48 | 1230 | 2700 | AV | AV Switcher | 200V - 3.06 | C4:42:68:65:2F:04 | 10.21.36.46 |
| lv4 | 1/1/1 | 100 | 2010 | AP | r650-3d-bf-30 |  | 5C:83:6C:3D:BF:30 | 10.21.1.14 |
| lv4 | 1/1/2 | 100 | 2010 | AP | r650-19-9c-c0 |  | C8:A6:08:19:9C:C0 | 10.21.1.20 |
| lv4 | 1/1/3 | 100 | 2010 | AP | r650-19-9c-a0 |  | C8:A6:08:19:9C:A0 | 10.21.1.19 |
| lv4 | 1/1/4 | 100 | 2010 | AP | r650-3d-4c-80 |  | 5C:83:6C:3D:4C:80 | 10.21.1.10 |
| lv4 | 1/1/5 | 100 | 2010 | AP | r650-19-9c-90 |  | C8:A6:08:19:9C:90 | 10.21.1.18 |
| lv4 | 1/1/6 | 100 | 2010 | AP | r650-19-9c-80 |  | C8:A6:08:19:9C:80 | 10.21.1.17 |
| lv4 | 1/1/7 | 100 | 2010 | AP | r650-19-9b-90 |  | C8:A6:08:19:9B:90 | 10.21.1.15 |
| lv4 | 1/1/9 | 100 | 2010 | AP | r650-19-9c-70 |  | C8:A6:08:19:9C:70 | 10.21.1.16 |
| lv4 | 1/1/10 | 100 | 2010 | AP | r650-3d-be-c0 |  | 5C:83:6C:3D:BE:C0 | 10.21.1.13 |
| lv4 | 1/1/11 | 100 | 2010 | AP | r650-32-66-a0 |  | CC:1B:5A:32:66:A0 | 10.21.1.48 |
| lv4 | 1/1/12 | 1240 | 2800 | printer | 200v-lvl4-staffroom |  | 34:9F:7B:5F:D4:1E | 10.21.27.7 |
| lv4 | 1/1/13 | 1230 | 2700 | AV | PDU | 200V - 4.08 | 28:B5:E8:6F:CB:73 | 10.21.36.126 |
| lv4 | 1/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 4.07 | C4:42:68:6D:2B:79 | 10.21.36.122 |
| lv4 | 1/1/18 | 1230 | 2700 | AV | Projector | 200V - 4.05 | F8:25:51:CD:3F:E3 | 10.21.36.113 |
| lv4 | 1/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.05 | C4:42:68:65:91:53 | 10.21.36.111 |
| lv4 | 1/1/20 | 1272 | 2700 | AV | Airtame | 200V - 4.05 | 38:4B:76:F1:A3:28 | 10.21.36.109 |
| lv4 | 1/1/22 | 1230 | 2700 | AV | Projector | 200V - 4.04 | F8:25:51:CF:D4:66 | 10.21.36.106 |
| lv4 | 1/1/23 | 1230 | 2700 | AV | Touch Panel | 200V - 4.04 | C4:42:68:6C:2D:D7 | 10.21.36.108 |
| lv4 | 1/1/24 | 1272 | 2700 | AV | Airtame | 200V - 4.04 | 38:4B:76:F1:A1:72 | 10.21.36.102 |
| lv4 | 1/1/25 | 1230 | 2700 | AV | PDU | 200V - 4.04 | 28:B5:E8:68:78:FD | 10.21.36.105 |
| lv4 | 1/1/26 | 1230 | 2700 | AV | Room Camera | 200V - 4.03 | B8:A4:4F:CB:C5:7C | 10.21.36.100 |
| lv4 | 1/1/27 | 1230 | 2700 | AV | AV Switcher | 200V - 4.03 | C4:42:68:65:68:A6 | 10.21.36.92 |
| lv4 | 1/1/29 | 1272 | 2700 | AV | Airtame | 200V - 4.03 | 38:4B:76:F1:83:57 | 10.21.36.91 |
| lv4 | 1/1/31 | 1230 | 2700 | AV | Projector | 200V - 4.02 | F8:25:51:CC:0F:80 | 10.21.36.88 |
| lv4 | 1/1/33 | 1230 | 2700 | AV | Touch Panel | 200V - 4.02 | C4:42:68:6D:2C:10 | 10.21.36.90 |
| lv4 | 1/1/35 | 1230 | 2700 | AV | Room Camera | 200V - 4.07 | B8:A4:4F:CB:D2:17 | 10.21.36.121 |
| lv4 | 1/1/36 | 1230 | 2700 | AV | Projector Screen Right | 200V - 4.13 Lecture Theatre | 80:1F:12:4E:EB:71 | 10.21.36.167 |
| lv4 | 1/1/37 | 1230 | 2700 | AV | Pro Convert for NDI to HDMI | 200V - 4.13 Lecture Theatre | D0:C8:57:80:AA:C1 | 10.21.36.163 |
| lv4 | 1/1/39 | 1230 | 2700 | AV | Touch Panel | 200V - 4.10 | C4:42:68:6C:40:90 | 10.21.36.136 |
| lv4 | 1/1/40 | 1230 | 2700 | AV | Touch Panel | 200V - 4.05 | C4:42:68:6C:35:04 | 10.21.36.115 |
| lv4 | 1/1/41 | 1230 | 2700 | AV | Touch Panel | 200V - 4.03 | C4:42:68:6C:42:6B | 10.21.36.101 |
| lv4 | 1/1/43 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 4.03 | D0:C8:57:82:8B:22 | 10.21.36.96 |
| lv4 | 1/1/44 | 1230 | 2700 | AV | PTZ Camera | 200V - 4.03 | DC:E2:AC:02:EC:B8 | 10.21.36.98 |
| lv4 | 2/1/13 | 1230 | 2700 | AV | Wireless Mic Receiver | 200V - 4.13 Lecture Theatre | 00:1B:66:39:93:93 | 10.21.36.172 |
| lv4 | 2/1/14 | 1272 | 2700 | AV | Airtame | 200V - 4.13 Lecture Theatre | 38:4B:76:F1:4D:F1 | 10.21.36.153 |
| lv4 | 2/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 4.11 Drama Room | C4:42:68:6C:43:F3 | 10.21.36.141 |
| lv4 | 2/1/17 | 1230 | 2700 | AV | PDU | 200V - 4.11 Drama Room | 98:89:24:0D:AF:FA | 10.21.36.138 |
| lv4 | 2/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.10 | C4:42:68:65:8F:7B | 10.21.36.132 |
| lv4 | 2/1/22 | 1272 | 2700 | AV | Airtame | 200V - 4.10 | 38:4B:76:F1:9F:90 | 10.21.36.130 |
| lv4 | 2/1/23 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.08 | C4:42:68:65:95:96 | 10.21.36.125 |
| lv4 | 2/1/24 | 1272 | 2700 | AV | Airtame | 200V - 4.08 | 38:4B:76:F1:A2:70 | 10.21.36.123 |
| lv4 | 2/1/25 | 1230 | 2700 | AV | Touch Panel | 200V - 4.13 Lecture Theatre | C4:42:68:3E:0A:00 | 10.21.36.171 |
| lv4 | 2/1/26 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.13 Lecture Theatre | C4:42:68:65:96:20 | 10.21.36.160 |
| lv4 | 2/1/27 | 1230 | 2700 | AV | Cresnet Bridge For Outside Sign | 200V - 4.13 Lecture Theatre | C4:42:68:5D:50:D2 | 10.21.36.156 |
| lv4 | 2/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 4.13 Lecture Theatre | 00:1D:56:0A:8E:47 | 10.21.36.155 |
| lv4 | 2/1/30 | 1230 | 2700 | AV | Audio DSP | 200V - 4.13 Lecture Theatre | 78:45:01:1A:93:AF | 10.21.36.154 |
| lv4 | 2/1/33 | 1230 | 2700 | AV | Room Camera | 200V - 4.04 | B8:A4:4F:CB:C4:3D | 10.21.36.107 |
| lv4 | 2/1/34 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.04 | C4:42:68:65:91:9A | 10.21.36.104 |
| lv4 | 2/1/35 | 1230 | 2700 | AV | Room Camera | 200V - 4.02 | B8:A4:4F:CE:F3:7D | 10.21.36.89 |
| lv4 | 2/1/36 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.02 | C4:42:68:65:91:85 | 10.21.36.86 |
| lv4 | 2/1/37 | 1230 | 2700 | AV | Room Camera | 200V - 4.13 Lecture Theatre | B8:A4:4F:CB:CA:ED | 10.21.36.170 |
| lv4 | 2/1/40 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.11 Drama Room | C4:42:68:65:91:77 | 10.21.36.137 |
| lv4 | 2/1/41 | 1230 | 2700 | AV | Room Camera | 200V - 4.11 Drama Room | B8:A4:4F:CE:F1:43 | 10.21.36.140 |
| lv4 | 2/1/42 | 1230 | 2700 | AV | Room Camera | 200V - 4.10 | B8:A4:4F:CE:EF:42 | 10.21.36.135 |
| lv4 | 2/1/43 | 1230 | 2700 | AV | Room Camera | 200V - 4.08 | B8:A4:4F:CE:EA:58 | 10.21.36.128 |
| lv4 | 2/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.07 | C4:42:68:65:91:8B | 10.21.36.118 |
| lv4 | 2/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 4.05 | B8:A4:4F:CB:CA:0C | 10.21.36.114 |
| lv4 | 2/1/46 | 1230 | 2700 | AV | Touch Panel | 200V - 4.08 | C4:42:68:6D:2F:E4 | 10.21.36.129 |
| lv4 | 2/1/47 | 1230 | 2700 | AV | AV Switcher | 200V - 4.08 | C4:42:68:65:3F:79 | 10.21.36.124 |
| lv4 | 2/1/48 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.03 | C4:42:68:65:95:E0 | 10.21.36.94 |
| lv4 | 4/1/3 | 1230 | 2700 | AV | Mic Charger | 200V - 4.13 Lecture Theatre | 00:1B:66:3F:0D:8B | 10.21.36.159 |
| lv4 | 4/1/5 | 1230 | 2700 | AV | PDU 1 | 200V - 4.13 Lecture Theatre | 28:B5:E8:6F:27:66 | 10.21.36.161 |
| lv4 | 4/1/8 | 1230 | 2700 | AV | LCD Left | 200V - 4.13 Lecture Theatre | F8:01:B4:AE:DF:9E | 10.21.36.157 |
| lv4 | 4/1/9 | 1230 | 2700 | AV | PTZ Camera | 200V - 4.13 Lecture Theatre | DC:E2:AC:02:EC:9D | 10.21.36.168 |
| lv4 | 4/1/18 | 1230 | 2700 | AV | Projector | 200V - 4.11 Drama Room | F8:25:51:D2:70:36 | 10.21.36.139 |
| lv4 | 4/1/22 | 1230 | 2700 | AV | Projector | 200V - 4.10 | F8:25:51:CD:40:32 | 10.21.36.134 |
| lv4 | 4/1/24 | 1230 | 2700 | AV | PDU | 200V - 4.10 | 28:B5:E8:6F:CB:40 | 10.21.36.133 |
| lv4 | 4/1/25 | 1230 | 2700 | AV | AV Switcher | 200V - 4.10 | C4:42:68:65:31:AC | 10.21.36.131 |
| lv4 | 4/1/37 | 1230 | 2700 | AV | LCD Right | 200V - 4.13 Lecture Theatre | F8:01:B4:60:4C:AB | 10.21.36.158 |
| lv4 | 4/1/41 | 1230 | 2700 | AV | Projector Screen Left | 200V - 4.13 Lecture Theatre | 80:34:28:5C:F1:F2 | 10.21.36.166 |
| lv4 | 4/1/42 | 1230 | 2700 | AV | Recorder | 200V - 4.13 Lecture Theatre | 00:1C:08:00:70:2D | 10.21.36.169 |
| lv4 | 4/1/43 | 1230 | 2700 | AV | PDU 2 | 200V - 4.13 Lecture Theatre | 28:B5:E8:6F:61:FB | 10.21.36.162 |
| lv4 | 5/1/4 | 1230 | 2700 | AV | Projector | 200V - 4.07 | F8:25:51:CD:3F:E0 | 10.21.36.120 |
| lv4 | 5/1/8 | 1230 | 2700 | AV | PDU | 200V - 4.07 | 28:B5:E8:68:81:A6 | 10.21.36.119 |
| lv4 | 5/1/9 | 1272 | 2700 | AV | Airtame | 200V - 4.07 | 38:4B:76:F1:A1:45 | 10.21.36.116 |
| lv4 | 5/1/10 | 1230 | 2700 | AV | AV Switcher | 200V - 4.07 | C4:42:68:65:39:C6 | 10.21.36.117 |
| lv4 | 5/1/13 | 1230 | 2700 | AV | PDU | 200V - 4.05 | 28:B5:E8:6F:CB:46 | 10.21.36.112 |
| lv4 | 5/1/15 | 1230 | 2700 | AV | AV Switcher | 200V - 4.05 | C4:42:68:65:3A:3F | 10.21.36.110 |
| lv4 | 5/1/20 | 1230 | 2700 | AV | AV Switcher | 200V - 4.04 | C4:42:68:65:37:38 | 10.21.36.103 |
| lv4 | 5/1/23 | 1230 | 2700 | AV | PDU | 200V - 4.03 | 28:B5:E8:68:7E:6D | 10.21.36.95 |
| lv4 | 5/1/25 | 1230 | 2700 | AV | Projector | 200V - 4.03 | F8:25:51:CF:D3:D9 | 10.21.36.97 |
| lv4 | 5/1/31 | 1230 | 2700 | AV | PDU | 200V - 4.02 | 28:B5:E8:6F:27:3F | 10.21.36.87 |
| lv4 | 5/1/32 | 1272 | 2700 | AV | Airtame | 200V - 4.02 | 38:4B:76:F1:92:CF | 10.21.36.84 |
| lv4 | 5/1/34 | 1230 | 2700 | AV | AV Switcher | 200V - 4.02 | C4:42:68:65:3F:D0 | 10.21.36.85 |
| lv4 | 5/1/36 | 1230 | 2700 | AV | Display | 200V - L4 Digital Signage 3 | 28:E6:A9:8D:5A:73 | 10.21.37.96 |
| lv4 | 5/1/37 | 1230 | 2700 | AV | Projector Right | 200V - 4.13 Lecture Theatre | 38:1A:52:B2:00:9F | 10.21.36.165 |
| lv4 | 5/1/39 | 1230 | 2700 | AV | Projector Left | 200V - 4.13 Lecture Theatre | 38:1A:52:B1:FF:B9 | 10.21.36.164 |
| lv4 | 5/1/41 | 1230 | 2700 | AV | Recorder | 200V - 4.03 | 00:1C:08:00:71:05 | 10.21.36.99 |
| lv4 | 5/1/42 | 1230 | 2700 | AV | Mic Charger | 200V - 4.03 | 00:1B:66:3F:15:3A | 10.21.36.93 |
| lv6 | 1/1/8 | 1230 | 2700 | AV | PDU | 200V - 6.01 Lecture Theatre | B8:80:4F:28:50:5F | 10.21.37.39 |
| lv6 | 1/1/9 | 1230 | 2700 | AV | AV Switcher | 200V - 6.01 Lecture Theatre | 00:1D:56:0A:B4:A6 | 10.21.37.34 |
| lv6 | 1/1/10 | 1230 | 2700 | AV | Audio DSP | 200V - 6.01 Lecture Theatre | 78:45:01:0C:7D:CC | 10.21.37.33 |
| lv6 | 1/1/13 | 100 | 2010 | AP | r650-39-4b-d0 |  | C8:A6:08:39:4B:D0 | 10.21.1.36 |
| lv6 | 1/1/14 | 100 | 2010 | AP | r650-39-4b-c0 |  | C8:A6:08:39:4B:C0 | 10.21.1.35 |
| lv6 | 1/1/15 | 100 | 2010 | AP | r650-39-52-00 |  | C8:A6:08:39:52:00 | 10.21.1.42 |
| lv6 | 1/1/16 | 100 | 2010 | AP | r650-39-51-00 |  | C8:A6:08:39:51:00 | 10.21.1.40 |
| lv6 | 1/1/17 | 100 | 2010 | AP | r650-39-54-a0 |  | C8:A6:08:39:54:A0 | 10.21.1.45 |
| lv6 | 1/1/18 | 100 | 2010 | AP | r650-39-4a-60 |  | C8:A6:08:39:4A:60 | 10.21.1.34 |
| lv6 | 1/1/20 | 100 | 2010 | AP | r650-39-5c-50 |  | C8:A6:08:39:5C:50 | 10.21.1.46 |
| lv6 | 1/1/21 | 100 | 2010 | AP | r650-39-2d-b0 |  | C8:A6:08:39:2D:B0 | 10.21.1.28 |
| lv6 | 1/1/22 | 100 | 2010 | AP | r650-39-35-10 |  | C8:A6:08:39:35:10 | 10.21.1.31 |
| lv6 | 1/1/23 | 100 | 2010 | AP | r650-39-52-90 |  | C8:A6:08:39:52:90 | 10.21.1.43 |
| lv6 | 1/1/24 | 100 | 2010 | AP | r650-39-33-f0 |  | C8:A6:08:39:33:F0 | 10.21.1.29 |
| lv6 | 1/1/25 | 100 | 2010 | AP | r650-39-34-60 |  | C8:A6:08:39:34:60 | 10.21.1.30 |
| lv6 | 1/1/26 | 100 | 2010 | AP | r650-39-36-80 |  | C8:A6:08:39:36:80 | 10.21.1.32 |
| lv6 | 1/1/27 | 100 | 2010 | AP | r650-39-4c-f0 |  | C8:A6:08:39:4C:F0 | 10.21.1.38 |
| lv6 | 1/1/28 | 100 | 2010 | AP | r650-39-51-e0 |  | C8:A6:08:39:51:E0 | 10.21.1.41 |
| lv6 | 1/1/29 | 100 | 2010 | AP | r650-39-54-60 |  | C8:A6:08:39:54:60 | 10.21.1.44 |
| lv6 | 1/1/37 | 1212 | 2600 | cctv | cam-200V2B23.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:61 | 10.21.25.10 |
| lv6 | 1/1/38 | 1212 | 2600 | cctv | cam-200V2F10.trinity.unimelb.edu.au |  | c0:56:e3:85:a4:d8 | 10.21.25.11 |
| lv6 | 1/1/39 | 1212 | 2600 | cctv | cam-200V1D05.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:2a | 10.21.25.12 |
| lv6 | 1/1/40 | 1212 | 2600 | cctv | cam-200V2K14.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:6d | 10.21.25.13 |
| lv6 | 2/1/1 | 1230 | 2700 | AV | NDI-HDMI Decoder | 200V - 6.01 Lecture Theatre | D0:C8:57:81:64:1F | 10.21.37.37 |
| lv6 | 2/1/2 | 1230 | 2700 | AV | Room Camera | 200V - 5.05 | B8:A4:4F:68:F5:19 | 10.21.36.210 |
| lv6 | 2/1/3 | 1230 | 2700 | AV | Projector | 200V - 5.05 | F8:25:51:CB:8C:9B | 10.21.36.209 |
| lv6 | 2/1/14 | 1230 | 2700 | AV | Touch Panel | 200V - 5.07 | C4:42:68:2C:3C:0A | 10.21.36.228 |
| lv6 | 2/1/15 | 1272 | 2700 | AV | Airtame | 200V - 5.07 | 38:4B:76:F1:89:2C | 10.21.36.221 |
| lv6 | 2/1/16 | 1230 | 2700 | AV | AV Switcher | 200V - 5.07 | 00:1D:56:09:CE:57 | 10.21.36.222 |
| lv6 | 2/1/16 | 1230 | 2700 | AV | PDU | 200V - 5.07 | B8:80:4F:18:2D:88 | 10.21.36.224 |
| lv6 | 2/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 5.08 | C4:42:68:2C:3D:36 | 10.21.36.236 |
| lv6 | 2/1/18 | 1240 | 2700 | AV | Room NUC | 200V - 5.07 | 48:21:0B:56:AE:39 | 10.21.36.227 |
| lv6 | 2/1/19 | 1230 | 2700 | AV | AV Switcher | 200V - 5.08 | 00:1D:56:0A:DF:BB | 10.21.36.230 |
| lv6 | 2/1/19 | 1230 | 2700 | AV | PDU | 200V - 5.08 | B8:80:4F:18:1C:F0 | 10.21.36.232 |
| lv6 | 2/1/20 | 1272 | 2700 | AV | Airtame | 200V - 5.08 | 38:4b:76:f1:89:48 | 10.21.36.229 |
| lv6 | 2/1/21 | 1240 | 2700 | AV | Room NUC | 200V - 5.08 | 48:21:0B:56:AC:B1 | 10.21.36.235 |
| lv6 | 2/1/22 | 1230 | 2700 | AV | Touch Panel | 200V - 5.09 | C4:42:68:2C:3C:C0 | 10.21.36.244 |
| lv6 | 2/1/23 | 1230 | 2700 | AV | AV Switcher | 200V - 5.09 | 00:1D:56:0A:DF:C3 | 10.21.36.238 |
| lv6 | 2/1/23 | 1230 | 2700 | AV | PDU | 200V - 5.09 | B8:80:4F:28:4C:8A | 10.21.36.240 |
| lv6 | 2/1/24 | 1272 | 2700 | AV | Airtame | 200V - 5.09 | 38:4b:76:f1:8f:ef | 10.21.36.237 |
| lv6 | 2/1/25 | 1240 | 2700 | AV | Room NUC | 200V - 5.09 | 48:21:0B:56:AA:D3 | 10.21.36.243 |
| lv6 | 2/1/26 | 1230 | 2700 | AV | Touch Panel | 200V - 5.10 | C4:42:68:1A:48:25 | 10.21.36.251 |
| lv6 | 2/1/27 | 1272 | 2700 | AV | Airtame | 200V - 5.10 | 38:4b:76:f1:90:1e | 10.21.36.245 |
| lv6 | 2/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 5.10 | 00:1D:56:0A:DF:BC | 10.21.36.246 |
| lv6 | 2/1/28 | 1230 | 2700 | AV | PDU | 200V - 5.10 | B8:80:4F:18:31:9C | 10.21.36.248 |
| lv6 | 2/1/31 | 1240 | 2700 | AV | Room NUC | 200V - 5.12 | 48:21:0B:56:AA:BC | 10.21.37.22 |
| lv6 | 2/1/35 | 1230 | 2700 | AV | PTZ Camera | 200V - 6.01 Lecture Theatre | DC:E2:AC:02:E4:DA | 10.21.37.44 |
| lv6 | 2/1/36 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.12 | B8:98:B0:0F:1C:5F | 10.21.37.17 |
| lv6 | 2/1/37 | 1230 | 2700 | AV | Room Camera | 200V - 5.12 | B8:A4:4F:68:FE:85 | 10.21.37.20 |
| lv6 | 2/1/38 | 1230 | 2700 | AV | Touch Panel | 200V - 5.12 | C4:42:68:38:BB:41 | 10.21.37.23 |
| lv6 | 2/1/39 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.13 | B8:98:B0:0F:1C:78 | 10.21.37.26 |
| lv6 | 2/1/40 | 1230 | 2700 | AV | Room Camera | 200V - 5.13 | B8:A4:4F:69:AB:10 | 10.21.37.29 |
| lv6 | 2/1/41 | 1230 | 2700 | AV | Projector | 200V - 5.13 | F8:25:51:CB:8C:6E | 10.21.37.28 |
| lv6 | 2/1/42 | 1230 | 2700 | AV | Projector | 200V - 6.01 Lecture Theatre | 38:1A:52:B1:75:B7 | 10.21.37.41 |
| lv6 | 2/1/43 | 1230 | 2700 | AV | Touch Panel | 200V - 6.01 Lecture Theatre | C4:42:68:33:59:A9 | 10.21.37.47 |
| lv6 | 2/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.01 Lecture Theatre | B8:98:B0:0F:1C:63 | 10.21.37.38 |
| lv6 | 2/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 6.01 Lecture Theatre | B8:A4:4F:49:85:78 | 10.21.37.46 |
| lv6 | 3/1/11 | 1240 | 2700 | AV | Room NUC | 200V - 5.04 | 48:21:0B:56:AE:12 | 10.21.36.203 |
| lv6 | 3/1/12 | 1272 | 2700 | AV | Airtame | 200V - 5.04 | 38:4b:76:f1:9a:d0 | 10.21.36.197 |
| lv6 | 3/1/13 | 1230 | 2700 | AV | AV Switcher | 200V - 5.05 | 00:1D:56:0A:E0:4F | 10.21.36.206 |
| lv6 | 3/1/13 | 1230 | 2700 | AV | PDU | 200V - 5.05 | B8:80:4F:28:58:6F | 10.21.36.208 |
| lv6 | 3/1/14 | 1230 | 2700 | AV | Touch Panel | 200V - 5.05 | C4:42:68:2A:71:B4 | 10.21.36.212 |
| lv6 | 3/1/15 | 1240 | 2700 | AV | Room NUC | 200V - 5.05 | 48:21:0B:56:AA:62 | 10.21.36.211 |
| lv6 | 3/1/16 | 1272 | 2700 | AV | Airtame | 200V - 5.05 | 38:4b:76:f1:89:26 | 10.21.36.205 |
| lv6 | 3/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 5.06 | C4:42:68:2C:3D:B9 | 10.21.36.220 |
| lv6 | 3/1/18 | 1230 | 2700 | AV | AV Switcher | 200V - 5.06 | 00:1D:56:0A:E0:4E | 10.21.36.214 |
| lv6 | 3/1/18 | 1230 | 2700 | AV | PDU | 200V - 5.06 | B8:80:4F:18:26:32 | 10.21.36.216 |
| lv6 | 3/1/19 | 1272 | 2700 | AV | Airtame | 200V - 5.06 | 38:4b:76:f1:90:1c | 10.21.36.213 |
| lv6 | 3/1/20 | 1230 | 2700 | AV | Room NUC | 200V - 5.06 | 48:21:0B:56:98:6C | 10.21.36.219 |
| lv6 | 3/1/24 | 1230 | 2700 | AV | Touch Panel | 200V - 6.02 Tutorial Room | C4:42:68:38:BB:1E | 10.21.37.55 |
| lv6 | 3/1/25 | 1230 | 2700 | AV | Touch Panel | 200V - 5.01 | C4:42:68:1C:E2:1B | 10.21.36.180 |
| lv6 | 3/1/26 | 1230 | 2700 | AV | Projector | 200V - 6.03 Tutorial Room | F8:25:51:CB:8C:6B | 10.21.37.60 |
| lv6 | 3/1/27 | 1230 | 2700 | AV | Room Camera | 200V - 5.01 | B8:A4:4F:69:B2:70 | 10.21.36.178 |
| lv6 | 3/1/28 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.01 | B8:98:B0:0F:1C:73 | 10.21.36.175 |
| lv6 | 3/1/29 | 1230 | 2700 | AV | Touch Panel | 200V - 5.02 | C4:42:68:2C:3D:5F | 10.21.36.188 |
| lv6 | 3/1/30 | 1230 | 2700 | AV | Room Camera | 200V - 5.02 | B8:A4:4F:69:D0:DE | 10.21.36.186 |
| lv6 | 3/1/31 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.02 | B8:98:B0:0F:1C:5E | 10.21.36.183 |
| lv6 | 3/1/32 | 1230 | 2700 | AV | Touch Panel | 200V - 5.03 | C4:42:68:2A:6C:B0 | 10.21.36.196 |
| lv6 | 3/1/33 | 1230 | 2700 | AV | Room Camera | 200V - 5.03 | B8:A4:4F:68:FE:98 | 10.21.36.194 |
| lv6 | 3/1/34 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.03 | B8:98:B0:0F:1C:68 | 10.21.36.191 |
| lv6 | 3/1/35 | 1230 | 2700 | AV | Touch Panel | 200V - 5.04 | C4:42:68:2C:3C:FE | 10.21.36.204 |
| lv6 | 3/1/36 | 1230 | 2700 | AV | Room Camera | 200V - 5.04 | B8:A4:4F:69:AE:62 | 10.21.36.202 |
| lv6 | 3/1/37 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.04 | B8:98:B0:0F:1C:6B | 10.21.36.199 |
| lv6 | 3/1/38 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.05 | B8:98:B0:0F:1C:6F | 10.21.36.207 |
| lv6 | 3/1/39 | 1230 | 2700 | AV | Room Camera | 200V - 5.06 | B8:A4:4F:49:82:9A | 10.21.36.218 |
| lv6 | 3/1/40 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.06 | B8:98:B0:0F:1C:75 | 10.21.36.215 |
| lv6 | 3/1/41 | 1230 | 2700 | AV | Room Camera | 200V - 5.07 | B8:A4:4F:49:55:98 | 10.21.36.226 |
| lv6 | 3/1/42 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.07 | B8:98:B0:0F:1C:70 | 10.21.36.223 |
| lv6 | 3/1/43 | 1230 | 2700 | AV | Room Camera | 200V - 5.08 | B8:A4:4F:69:AE:77 | 10.21.36.234 |
| lv6 | 3/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.08 | B8:98:B0:0F:1C:65 | 10.21.36.231 |
| lv6 | 3/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 5.09 | B8:A4:4F:69:B2:86 | 10.21.36.242 |
| lv6 | 3/1/46 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.09 | B8:98:B0:0F:1C:6C | 10.21.36.239 |
| lv6 | 3/1/47 | 1230 | 2700 | AV | Room Camera | 200V - 5.10 | B8:A4:4F:69:D0:9F | 10.21.36.250 |
| lv6 | 3/1/48 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.10 | B8:98:B0:0F:1C:6D | 10.21.36.247 |
| lv6 | 4/1/4 | 1230 | 2700 | AV | Touch Panel | 200V - 5.13 | C4:42:68:38:BA:E0 | 10.21.37.31 |
| lv6 | 4/1/5 | 1230 | 2700 | AV | Projector Screen | 200V - 6.01 Lecture Theatre | 04:91:62:D7:90:01 | 10.21.37.43 |
| lv6 | 4/1/6 | 1230 | 2700 | AV | Room Camera | 200V - 6.11 Drama Studio | B8:A4:4F:69:D0:8F | 10.21.37.76 |
| lv6 | 4/1/8 | 1230 | 2700 | AV | Touch Panel | 200V - 6.11 Drama Studio | C4:42:68:2C:B4:62 | 10.21.37.77 |
| lv6 | 4/1/10 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.11 Drama Studio | B8:98:B0:0F:1C:6E | 10.21.37.73 |
| lv6 | 4/1/11 | 1230 | 2700 | AV | Room Camera | 200V - 6.02 Tutorial Room | B8:A4:4F:68:F4:DA | 10.21.37.53 |
| lv6 | 4/1/12 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.02 Tutorial Room | B8:98:B0:0F:1C:67 | 10.21.37.50 |
| lv6 | 4/1/13 | 1272 | 2700 | AV | Airtame | 200V - 6.02 Tutorial Room | 38:4b:76:f1:9a:cd | 10.21.37.48 |
| lv6 | 4/1/14 | 1230 | 2700 | AV | AV Switcher | 200V - 6.02 Tutorial Room | 00:1D:56:0A:E0:53 | 10.21.37.49 |
| lv6 | 4/1/15 | 1230 | 2700 | AV | PDU | 200V - 6.02 Tutorial Room | B8:80:4F:28:1D:83 | 10.21.37.51 |
| lv6 | 4/1/16 | 1230 | 2700 | AV | Projector | 200V - 6.02 Tutorial Room | F8:25:51:CF:6E:65 | 10.21.37.52 |
| lv6 | 4/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 6.03 Tutorial Room | C4:42:68:38:BB:AC | 10.21.37.63 |
| lv6 | 4/1/18 | 1240 | 2700 | AV | Room NUC | 200V - 6.02 Tutorial Room | 48:21:0B:56:AD:69 | 10.21.37.54 |
| lv6 | 4/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.03 Tutorial Room | B8:98:B0:0F:1C:6A | 10.21.37.58 |
| lv6 | 4/1/20 | 1230 | 2700 | AV | Room Camera | 200V - 6.03 Tutorial Room | B8:A4:4F:69:AE:6F | 10.21.37.61 |
| lv6 | 4/1/21 | 1272 | 2700 | AV | Airtame | 200V - 6.03 Tutorial Room | 38:4b:76:f1:8f:eb | 10.21.37.56 |
| lv6 | 4/1/22 | 1230 | 2700 | AV | AV Switcher | 200V - 6.03 Tutorial Room | 00:1D:56:0A:E0:88 | 10.21.37.57 |
| lv6 | 4/1/22 | 1230 | 2700 | AV | PDU | 200V - 6.03 Tutorial Room | B8:80:4F:28:54:6D | 10.21.37.59 |
| lv6 | 4/1/23 | 1240 | 2700 | AV | Room NUC | 200V - 6.03 Tutorial Room | 48:21:0B:56:AE:56 | 10.21.37.62 |
| lv6 | 4/1/24 | 1213 | 2510 | PhysicalAccess | cloe-000-200v * |  | 00:11:B9:08:2B:67 | 10.21.22.10 |
| lv6 | 5/1/1 | 1240 | 2800 | printer | 200v-lvl-5-quiet-study |  | 40:F8:DF:64:BE:D6 | 10.21.27.9 |
| lv6 | 5/1/34 | 1230 | 2700 | AV | Projector Screen | 200V - 6.01 Lecture Theatre | 04:91:62:D7:95:5D | 10.21.37.42 |
| lv6 | 5/1/38 | 1240 | 2800 | printer | 200v-lvl-5-eap-office |  | 6C:3C:7C:BE:91:93 | 10.21.27.8 |
| lv6 | 5/1/42 | 1240 | 2800 | printer | 200v-lvl-5-staffroom2 |  | 6C:3C:7C:BE:C0:A9 | 10.21.27.10 |
| lv6 | 6/1/20 | 1230 | 2700 | AV | Recorder | 200V - 6.01 Lecture Theatre | 00:1C:08:00:6B:B7 | 10.21.37.45 |
| lv6 | 6/1/24 | 1272 | 2700 | AV | Airtame | 200V - 6.01 Lecture Theatre | 38:4b:76:f1:9a:d1 | 10.21.37.32 |
| lv6 | 6/1/26 | 1272 | 2700 | AV | Airtame | 200V - 5.13 | 38:4b:76:f1:89:59 | 10.21.37.24 |
| lv6 | 6/1/27 | 1230 | 2700 | AV | AV Switcher | 200V - 5.13 | 00:1D:56:0A:DF:BD | 10.21.37.25 |
| lv6 | 6/1/27 | 1230 | 2700 | AV | PDU | 200V - 5.13 | B8:80:4F:18:74:D1 | 10.21.37.27 |
| lv6 | 6/1/28 | 1240 | 2700 | AV | Room NUC | 200V - 5.13 | 48:21:0B:56:AD:2C | 10.21.37.30 |
| lv6 | 6/1/34 | 1230 | 2700 | AV | Projector | 200V - 5.12 | F8:25:51:CF:6E:6D | 10.21.37.19 |
| lv6 | 6/1/38 | 1240 | 2700 | AV | Room NUC | 200V - 5.12 | 48:21:0B:56:AC:F2 | 10.21.37.21 |
| lv6 | 6/1/39 | 1272 | 2700 | AV | Airtame | 200V - 5.12 | 38:4b:76:f1:89:60 | 10.21.37.15 |
| lv6 | 6/1/40 | 1230 | 2700 | AV | AV Switcher | 200V - 5.12 | 00:1D:56:0A:E0:87 | 10.21.37.16 |
| lv6 | 6/1/40 | 1230 | 2700 | AV | PDU | 200V - 5.12 | B8:80:4F:18:22:12 | 10.21.37.18 |
| lv6 | 6/1/45 | 1230 | 2700 | AV | LCD | 200V - 6.01 Lecture Theatre | A8:4A:63:30:73:65 | 10.21.37.36 |
| lv6 | 6/1/46 | 1230 | 2700 | AV | LCD | 200V - 6.01 Lecture Theatre | A8:4A:63:30:73:E3 | 10.21.37.35 |
| lv6 | 7/1/3 | 1230 | 2700 | AV | Projector | 200V - 5.04 | F8:25:51:CB:8C:95 | 10.21.36.201 |
| lv6 | 7/1/4 | 1230 | 2700 | AV | Projector | 200V - 5.10 | F8:25:51:CB:8C:A9 | 10.21.36.249 |
| lv6 | 7/1/5 | 1230 | 2700 | AV | Projector | 200V - 5.01 | F8:25:51:CB:8A:CE | 10.21.36.177 |
| lv6 | 7/1/8 | 1230 | 2700 | AV | Projector | 200V - 6.11 Drama Studio | F8:25:51:CB:8B:46 | 10.21.37.75 |
| lv6 | 7/1/9 | 1230 | 2700 | AV | Projector | 200V - 5.02 | F8:25:51:CB:8C:97 | 10.21.36.185 |
| lv6 | 7/1/10 | 1272 | 2700 | AV | Airtame | 200V - 6.11 Drama Studio | 38:4b:76:f1:89:2b | 10.21.37.72 |
| lv6 | 7/1/11 | 1230 | 2700 | AV | Projector | 200V - 5.08 | F8:25:51:CB:8B:41 | 10.21.36.233 |
| lv6 | 7/1/14 | 1230 | 2700 | AV | PDU | 200V - 6.11 Drama Studio | B8:80:4F:28:50:35 | 10.21.37.74 |
| lv6 | 7/1/17 | 1230 | 2700 | AV | Projector | 200V - 5.07 | F8:25:51:CB:8C:69 | 10.21.36.225 |
| lv6 | 7/1/19 | 1240 | 2800 | printer | 200v-lvl-6-staff-corridor |  | 34:9F:7B:5E:35:55 | 10.21.27.11 |
| lv6 | 7/1/22 | 1272 | 2700 | AV | Airtame | 200V - 5.01 | 38:4b:76:f1:89:5e | 10.21.36.173 |
| lv6 | 7/1/23 | 1230 | 2700 | AV | AV Switcher | 200V - 5.01 | 00:1D:56:0A:E0:89 | 10.21.36.174 |
| lv6 | 7/1/23 | 1230 | 2700 | AV | PDU | 200V - 5.01 | B8:80:4F:28:19:F0 | 10.21.36.176 |
| lv6 | 7/1/24 | 1240 | 2700 | AV | Room NUC | 200V - 5.01 | 48:21:0B:56:AE:2F | 10.21.36.179 |
| lv6 | 7/1/27 | 1272 | 2700 | AV | Airtame | 200V - 5.02 | 38:4b:76:f1:9a:d4 | 10.21.36.181 |
| lv6 | 7/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 5.02 | 00:1D:56:0A:E0:4D | 10.21.36.182 |
| lv6 | 7/1/28 | 1230 | 2700 | AV | PDU | 200V - 5.02 | B8:80:4F:28:54:5B | 10.21.36.184 |
| lv6 | 7/1/30 | 1240 | 2700 | AV | Room NUC | 200V - 5.02 | 48:21:0B:56:AA:57 | 10.21.36.187 |
| lv6 | 7/1/31 | 1272 | 2700 | AV | Airtame | 200V - 5.03 | 38:4b:76:f1:90:19 | 10.21.36.189 |
| lv6 | 7/1/32 | 1230 | 2700 | AV | AV Switcher | 200V - 5.03 | 00:1D:56:0A:E0:35 | 10.21.36.190 |
| lv6 | 7/1/32 | 1230 | 2700 | AV | PDU | 200V - 5.03 | B8:80:4F:28:58:54 | 10.21.36.192 |
| lv6 | 7/1/33 | 1240 | 2700 | AV | Room NUC | 200V - 5.03 | 48:21:0B:56:AD:53 | 10.21.36.195 |
| lv6 | 7/1/34 | 1230 | 2700 | AV | Projector | 200V - 5.03 | F8:25:51:CB:8C:A8 | 10.21.36.193 |
| lv6 | 7/1/35 | 1230 | 2700 | AV | AV Switcher | 200V - 5.04 | 00:1D:56:0A:E0:36 | 10.21.36.198 |
| lv6 | 7/1/35 | 1230 | 2700 | AV | PDU | 200V - 5.04 | B8:80:4F:28:5C:11 | 10.21.36.200 |
| lv6 | 7/1/40 | 1230 | 2700 | AV | Projector | 200V - 5.09 | F8:25:51:CB:8B:3D | 10.21.36.241 |
| lv6 | 7/1/44 | 1230 | 2700 | AV | Projector | 200V - 6.01 Lecture Theatre | 38:1A:52:B1:75:B8 | 10.21.37.40 |
| Tutorial-6.03 | 1/1/2 | 1230 | 2700 | AV | Room Camera | 200V - 6.04 Tutorial Room | B8:A4:4F:68:F4:EE | 10.21.37.69 |
| Tutorial-6.03 | 1/1/4 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.04 Tutorial Room | B8:98:B0:0F:1C:5B | 10.21.37.66 |
| Tutorial-6.03 | 1/1/5 | 1230 | 2700 | AV | Room NUC | 200V - 6.04 Tutorial Room | 48:21:0B:56:AE:3A | 10.21.37.70 |
| Tutorial-6.03 | 1/1/6 | 1230 | 2700 | AV | Touch Panel | 200V - 6.04 Tutorial Room | C4:42:68:38:BB:80 | 10.21.37.71 |
| Tutorial-6.03 | 1/1/7 | 1272 | 2700 | AV | Airtame | 200V - 6.04 Tutorial Room | 38:4b:76:f1:89:5a | 10.21.37.64 |
| Tutorial-6.03 | 1/1/8 | 1230 | 2700 | AV | PDU | 200V - 6.04 Tutorial Room | B8:80:4F:28:58:2D | 10.21.37.67 |