Root cause analysis: lv6 stack unit 3 failures

| Item | Value |
| --- | --- |
| Switch | switch-200V-stack-lv6 (192.168.122.11), level 6 at 200V, 7-unit ICX8200 stack |
| Affected unit | Unit 3, ICX8200-48PF2, serial FNM4319U09A, MAC 3c46.a107.5aa2 |
| Firmware | RUCKUS FastIron 10.0.10g_cd3 (RDR10010g_cd3), boot 10.2.12; same on all 7 units |
| Incidents | Fri 9 Oct 2026 20:03 to 20:44 (41 minutes) and Fri 9 Oct 22:44 to Sat 10 Oct 13:35 (about 15 hours) |
| Impact | Units 3 and 4 out of the stack: all their ports had no link and the lv6 uplink ran on one 10G link |
| Root cause | Not yet confirmed. Most likely a fault in unit 3 itself (hardware or firmware); waiting on Ruckus TAC |
| Status | Open. Stack stable since the reload on 10 October (6 days 6 hours at 16 October) |
| Prepared by | Alam Singh (Dycom), 16 October 2026 |

Summary

During the 200V VLAN cutover on Friday 9 October, unit 3 of the lv6 stack left the stack twice. Unit 4 is cabled only to unit 3, so it went out with it both times. Both times unit 3 hung and did not recover by itself: the first time it was power cycled on site at about 20:42 and both units were back by 20:44; the second time it ran on its own (its console showed the standalone prompt) until it was power cycled on site at 13:35 the next day. No stack cable, power supply, fan or temperature message was logged before either failure, and the tech-support output taken on 16 October shows a clean stack cable, healthy power and normal temperatures. Unit 3 also has a reload-history entry of its own at 20:42:55 on 9 October, at the time of the first power cycle, with a stack trace that does not match any normal reload. A unit that hangs twice and recovers only on a power cycle points to unit 3 (hardware or firmware) rather than the cabling; Ruckus TAC needs to decode the trace.

Background

lv6 is a seven-unit stack cabled as a chain, 4 - 3 - 2 - 1 - 5 - 6 - 7, not a ring. Stack ports 4/2/3 and 7/2/3 are configured to close the ring but have no cable (see Actions). Each unit reaches the active unit only through its neighbours, so a failure of unit 3 also cuts off unit 4. The uplink to trinity200V-core is LAG lg10 ("core"), two 10G LR links on 1/2/4 and 3/2/2, to core lag 1.

| Unit | Model | Serial | Priority | Role and stack links |
| --- | --- | --- | --- | --- |
| 1 | ICX8200-48PF2 | FNM4319U08C | 128 | Active. 1/2/1 to unit 5, 1/2/3 to unit 2. Uplink 1/2/4 |
| 2 | ICX8200-48PF2 | FNM4319U07B | 110 | Standby. 2/2/1 to unit 1, 2/2/3 to unit 3 |
| 3 | ICX8200-48PF2 | FNM4319U09A | 100 | Member. 3/2/1 to unit 2, 3/2/3 to unit 4. Uplink 3/2/2 |
| 4 | ICX8200-24P | FND4318U11Y | 80 | Member. 4/2/1 to unit 3; 4/2/3 configured, no cable |
| 5 | ICX8200-48 | FNF4315U00C | 70 | Member. 5/2/1 to unit 1, 5/2/3 to unit 6 |
| 6 | ICX8200-48 | FNF4315U001 | 50 | Member. 6/2/1 to unit 5, 6/2/3 to unit 7 |
| 7 | ICX8200-48 | FNF4315U00H | 40 | Member. 7/2/1 to unit 6; 7/2/3 configured, no cable |

Stack links are RUCKUS 25G passive 3 m DACs (58-0000065-01); the uplinks are RUCKUS 10GE LR optics (57-0000076-01).

Timeline

Times are as the switch logs them. The switch clock is set to Australian Eastern Standard Time and does not change for daylight saving, so it runs one hour behind local time (for example 20:03 on the switch was 21:03 AEDT). The reload history in show stack failover is recorded in local time; it is converted below.

| Switch time | Event |
| --- | --- |
| Fri 9 Oct 17:19 | Planned reload of the whole lv6 stack to apply system-max spanning-tree 40 (all 7 units in the reload history). All 200V switches back up by about 18:11 |
| 9 Oct 18:38 - 18:58 | lv6 VLAN cutover pasted (Level 6 ports moved to the new VLANs) |
| 9 Oct 20:03:25 | Units 3 and 4 deleted from the stack, unit 4 one second later. No warning message beforehand |
| 9 Oct 20:38 | lv6 reported broken during Trinity Wireless testing |
| 9 Oct 20:42:55 | Unit 3 power cycled on site (confirmed by Dycom). The reload history has an entry for unit 3 only at this time, with a different stack trace from every normal reload |
| 9 Oct 20:44:52 | Units 3 and 4 rejoin the stack |
| 9 Oct 22:40:32 | VLAN 2952 spanning-tree root changes to LT2-3 (10f0.680b.d23e), learned through lg10 |
| 9 Oct 22:41:18 | SmartZone CLI session (sz-admin) on lv6 times out and logs out |
| 9 Oct 22:44:19 | Units 3 and 4 deleted again in the same second. All their ports go down, including stack ports 3/2/1 and 3/2/3 and uplink 3/2/2. No stack-port flap, power or temperature message before it. Unit 3 does not restart this time and is left running standalone (console prompt ICX8200-48PF2 Router) |
| Sat 10 Oct 13:29 | lv6 reported broken again |
| 10 Oct 13:35:14 | Unit 3 power cycled on site; units 3 and 4 rejoin. Unit 3 uptime then 5 minutes, unit 4 20 hours (unit 4 never restarted) |
| 10 Oct 14:20 | Stack ports 4/2/3 and 7/2/3 configured to close the ring |
| 10 Oct 14:35 | DAC fitted between 4/2/3 and 7/2/3 |
| 10 Oct 15:19 | Devices losing pings; the DAC is removed. Matches known defect FI-318592 (LLDP loop inside a ring-cabled stack, fixed in 10.0.10g_cd7; see Firmware release notes) |
| 10 Oct 15:33 - 15:34 | write memory, then reload of the whole stack from a SmartZone CLI session. Stack up at 15:35:55 |
| Fri 16 Oct 22:12 | show tech-support taken: all 7 units Ready, stack up 6 days 6 hours, no further failures |

- First outage: 41 minutes (20:03 to 20:44, 9 October)

- Second outage: about 14 hours 51 minutes (22:44 on 9 October to 13:35 on 10 October). It happened overnight and was not alerted, so it was found when staff reported it the next day

- While out: all ports on units 3 and 4 had no link (AV on 3/1/11-48 and 4/1/4-23, building access on 4/1/24, staff ports), and the lv6 uplink ran on 1/2/4 only (10G instead of 20G)

Evidence collected

From show tech-support and the other show commands run on lv6 on 16 October 2026 (22:12 switch time), and from the logs captured on 9 and 10 October.

| Check | Result | What it shows | OK? | Source |
| --- | --- | --- | --- | --- |
| Stack state | All 7 units Ready; standby unit 2 can fail over; topology probe OK between all 7 units | The stack has been healthy since the 10 October reload | Yes | show stack detail, show stack connection |
| Stack link 2/2/3 to 3/2/1 | Up 6 days 6 hours; 0 input errors, 0 CRC, 0 drops at both ends (1.1 billion and 0.7 billion packets); RUCKUS DAC 58-0000065-01 | The cable has been clean since 10 October | Yes | show interfaces, show media |
| Unit 3 power | PSU 1 and PSU 2 present, status ok. No PSU message in the logs before either failure | No sign of a power fault | Yes | show chassis, show logging |
| Unit 3 temperature and fans | 21.3 - 34.0 deg C (warning 69, shutdown 72); both fans ok | No overheating | Yes | show chassis |
| PoE | 1,480 W capacity, 928 W free across the stack | No PoE overload | Yes | show inline power |
| CPU and memory | CPU 3% (5 minutes); unit 3 memory 23% used | No load or memory problem now | Yes | show cpu-utilization, show memory |
| Reload history | Unit 3 has one entry no other unit has: 9 Oct 21:42:55 local (20:42:55 switch), with a different stack trace | Matches the 9 October power cycle. The 10 October power cycle left no entry, so why this one did is a question for TAC | No | show stack failover |
| Logs for 9 October | Not available: the 4,000-line buffer now starts on 15 October, and no syslog server is configured | The 9 October detail survives only in the outputs captured on the day | No | show logging |
| Command history | Starts 18 September but has no 9 October entries; 10 October shows the ring work and the 15:34 reload | No CLI reload of unit 3 recorded; both recoveries were power cycles on site | - | show cli-command-history |
| Power-cycle record | Unit 3 shows last power cycle 23 Aug 2025, although it was power cycled on 10 October | This record cannot be relied on | - | show chassis |

Possible causes

| # | Cause and evidence | Likelihood |
| --- | --- | --- |
| 1 | Fault in unit 3 (hardware or firmware). For: it dropped twice with no warning; both times it stayed hung and recovered only on a power cycle (after the second drop it ran standalone for 15 hours); unit 4 never restarted; the 9 October power cycle left an unusual stack trace on unit 3 only; the release notes list FI-307270 (a stack unit stopped forwarding on all ports, no trigger), fixed after our build. Against: it has run 6 days without a repeat | High |
| 2 | Stack link 2/2/3 to 3/2/1 (DAC or port). For: losing that one link gives the same symptom (units 3 and 4 drop together). Against: zero errors on both ends over 6 days, genuine RUCKUS part, and it would not explain unit 3 hanging until power cycled | Low - medium |
| 3 | Change activity on 9 October (system-max spanning-tree raised to 40, new VLANs with spanning tree, many port moves). For: both failures came within 5 hours of the reload and during the cutover; a spanning-tree root change came 3 minutes before the second drop, and FI-312886 (fixed in cd5) says a loop on the data ports can starve stack traffic and break the stack. Against: the same configuration has run 6 days without fault, and lv3 and lv4 had the same change without failures | Low - medium |
| 4 | Power supply or power feed to unit 3. For: none. Against: both PSUs ok and no PSU message logged | Low |
| 5 | Temperature. Against: 21 - 34 deg C, fans ok | Low |
| 6 | SmartZone action. For: its CLI session timed out 3 minutes before the second drop. Against: it was a logout with no commands | Low |

Firmware release notes

lv6 runs FastIron 10.0.10g_cd3. The release notes for 10.0.10g_cd3, cd5, cd8 and cd10 (cd10 published 12 August 2026) were checked on 16 October. 93 issues were fixed between cd3 and cd10. None matches unit 3 exactly (a member unit that leaves the stack without warning and stays hung until power cycled), but the issues below are relevant to lv6.

| Issue | Status | Symptom (from the release notes) | Relevance to lv6 |
| --- | --- | --- | --- |
| FI-318592 | Fixed in cd7 | In a stack cabled as a ring, LLDP packets are sent over the stack ports and cause a loop inside the stack. Workaround: cable the stack as a chain | High. Matches the packet loss when the ring was closed on 10 October. Do not close the ring until lv6 runs cd7 or later |
| FI-307270 | Fixed in cd7 | The second unit in the stack stopped forwarding traffic from all ports. No trigger | Medium. Closest symptom to unit 3. Ask TAC whether it matches |
| FI-312886 | Fixed in cd5 | A high rate of control packets on data ports (from an L2 loop) starves the stack ports; the stack may break | Medium. Possible on 9 October; does not explain unit 3 staying hung |
| FI-312861 | Fixed in cd6 | Critical, Alert and Warning events are not shown in SmartZone for a switch | Explains why SmartZone gave no alarm for the overnight outage |
| FI-319392 | Fixed in cd5 | show logging shows no output after a burst of log messages | Affects evidence collection |
| FI-311871 | Open in cd10 | Unexpected reload sometimes after supportsave or show media ethernet all (part of show tech-support) | Run tech-support captures at a quiet time |
| FI-290918 | Open in cd10 | A few CRC or input errors on stack and uplink ports with 25G passive 5 m DACs on ICX8200 | Use 3 m or shorter (as now) for the ring cable |

Checked and not applicable to lv6:

- FI-322504 (fixed in cd8): crash when an RSTP port is disabled. lv6 runs 802.1D spanning tree, not RSTP

- FI-307360 (cd7): stack unit reloads with sFlow enabled. sFlow is not configured

- FI-325415 (cd10): DHCP snooping database race. DHCP snooping and ARP inspection are not configured

- FI-319187 (cd7): stack member reboot from malformed web requests. No web sessions; and unit 3 hung rather than rebooting

- FI-319366 (cd8): reboot when temperature sensor 3 reads 0 deg C. Unit 3 temperatures are normal and it did not reboot

Conclusion

The most likely root cause is a fault in unit 3 (FNM4319U09A), either in the hardware or in FastIron 10.0.10g_cd3 on that unit. Both failures needed a power cycle to recover, which a cable fault would not. This is not yet confirmed: the 20:42:55 stack trace needs decoding by Ruckus TAC, and the 9 October logs are no longer on the switch.

Contributing factors, which made the impact bigger than it needed to be:

- The stack is a chain, not a ring, so unit 4 went down with unit 3

- The second uplink member (3/2/2) is on unit 3, so the uplink lost half its capacity

- No syslog server and no alert on stack member loss, so the second outage went unnoticed overnight and its logs have since been overwritten

- SmartZone still shows 5 stack members instead of 7, and on cd3 it does not show the switch's Critical, Alert or Warning events (FI-312861), so it cannot be relied on to show the stack's state

- lv6 runs 10.0.10g_cd3; later builds on the same release fix several stack-related defects

Actions

| # | Action | Owner | Priority | Status |
| --- | --- | --- | --- | --- |
| 1 | Open a Ruckus TAC case for unit 3 (FNM4319U09A, 10.0.10g_cd3). Send the 16 October tech-support and the 9 and 10 October log captures. Ask TAC to decode the unit 3 reload trace (9 Oct 21:42:55 local, at the power cycle), explain why the 10 October power cycle left no entry, say whether it matches FI-307270 or FI-312886 or another known defect, and confirm 10.0.10g_cd10 as the target release | Dycom | High | Open |
| 2 | Confirm whether anyone reloaded unit 3 from its console at about 20:42 on 9 October. Answer: no, it was power cycled on site | Dycom | High | Closed |
| 3 | Send logs off the switch: configure logging host <syslog server> on lv6 and the other 200V switches, so the buffer filling up cannot erase the evidence | Dycom / Trinity | High | Open |
| 4 | Move the second uplink member from unit 3 (3/2/2) to unit 2, using free core port 1/1/7 or 2/1/7, so the uplink keeps both links if unit 3 fails | Dycom | Medium | Open |
| 5 | After action 9 only: close the ring 4/2/3 to 7/2/3 with a RUCKUS 25G DAC of 3 m or shorter (the stack-port configuration is already in place). The ring tried on 10 October caused packet loss, which matches FI-318592 on cd3. Do it in a change window | Dycom | Medium | Open |
| 6 | Swap the 2/2/3 to 3/2/1 DAC at the same window, to rule out the cable | Dycom | Medium | Open |
| 7 | Check that unit 3's two power supplies are on separate circuits (or UPS) and the leads are secure | Trinity | Low | Open |
| 8 | Replace unit 3 (RMA) if TAC finds a hardware fault, or if it fails again | Dycom / Ruckus | - | Open |
| 9 | Upgrade lv6 to FastIron 10.0.10g_cd10 (ICX8200 image RDR10010g_cd10ufi.bin) in a change window; the stack reloads. Check the other 200V stacks' versions and plan the same upgrade before any of them is cabled as a ring | Dycom | High | Open |
| 10 | Add an alert for stack member loss (SNMP trap or SmartZone alarm) and raise the 5-of-7 member display with TAC. After the upgrade, check that SmartZone shows the switch's warning events (FI-312861) | Dycom / Trinity | Medium | Open |

If unit 3 fails again

- Note the time, and check show stack on the switch (not SmartZone).

- Before power cycling, connect to unit 3's console and capture skip-page-display, show stack, show logging and show tech-support to a file. This is the evidence the first two failures did not leave. (On cd3 and cd10, show tech-support can occasionally cause a reload, FI-311871; on a unit that is already hung this is acceptable.)

- From the active unit, capture show stack failover and show logging.

- Power cycle unit 3 and confirm units 3 and 4 rejoin (show stack: all 7 Ready).

- Send the captures to the TAC case and add the event to this document.

Appendix A – Unit 3 reload history

From show stack failover on 16 October. Times are local (AEDT). The 9 Oct 18:19:50 and 10 Oct 16:34:35 entries appear on all 7 units (the two whole-stack reloads). The 9 Oct 21:42:55 entry is on unit 3 only and matches its power cycle on site; the 10 October power cycle (about 14:35 local) left no entry.

| Time (local) | Stack trace |
| --- | --- |
| Fri 7 Nov 2025 21:57:40 | f7033ed4 f703407c 00685e10 020c6b7c 00a19640 00b14128 020fc89c 020fdb28 009890b0 007b4df4 0062fd1c f6f7ed80 f7a255cf |
| Fri 9 Oct 2026 18:19:50 | f7402f70 f7402e0c f740360c 006d3f1c 01a1e30c 00a7164c 00acd7e8 01ab2da8 01ab4114 00b5f530 00858fe4 006a65d8 f7345c68 f7ecc5cf |
| Fri 9 Oct 2026 21:42:55 | f72ccf70 f72cce0c f72cd60c 006d3f1c 01a1e30c 01b292b4 01b29884 0122f578 00b600f8 012380ac f720fc68 f7d965cf (unit 3 only) |
| Sat 10 Oct 2026 16:34:35 | f6cc7f70 f6cc7e0c f6cc860c 006d3f1c 01a1e30c 00a7164c 00acd7e8 01ab2da8 01ab4114 00b5f530 00858fe4 006a65d8 f6c0ac68 f77915cf |

Appendix B – Evidence files

- lv6 show tech-support and show commands, 16 October 2026 22:10 - 22:12 switch time (PuTTY log, 1.6 MB). Not stored in the repository because it contains the running configuration (password hashes and SNMP community strings); held by Dycom and to be sent to Ruckus TAC only

- lv6 show logging | include stack, and show logging around 22:44 on 9 October, captured on 10 October

- show stack outputs from 9 and 10 October (in the Trinity 200V VLAN Migration document)

- FastIron 10.0.10g_cd3, cd5, cd8 and cd10 release notes (RUCKUS support portal, login required); held by Dycom, not stored in the repository