What is the change

Move the 39 pilot devices and 22 Level 3 staff outlets at 200V from their legacy VLANs to the new VLANs (2010, 2100, 2600, 2700 and 2800) by changing the untagged VLAN on 60 access ports on three RUCKUS ICX FastIron 10.0.10g stacks.

switch-200V-stack-lv3

switch-200V-stack-lv4

switch-200V-stack-lv6

| Device type | Old VLAN | New VLAN | New subnet | Devices |
| --- | --- | --- | --- | --- |
| AV | 1230 (200V_av) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 18 |
| AV (Airtame) | 1272 (200V_w_wifi) | 2700 (VL-av-01) | 10.21.36.x | 3 |
| AV (Room NUC) | 1240 (200V_staff) | 2700 (VL-av-01) | 10.21.37.x | 1 |
| CCTV | 1212 (200V_IPcam) | 2600 (VL-cctv-01) | 10.21.25.x | 13 |
| AP management | 100 (management) | 2010 (VL-wifimgmt-01) | 10.21.1.x | 2 |
| Printer | 1240 (200V_staff) | 2800 (VL-print-01) | 10.21.27.x | 1 |
| Staff outlets (Level 3) | 1240 (200V_staff) | 2100 (VL-staff-wired-01) | 10.21.3.x | 17 outlets |
| Staff outlets (Level 3) | 1 (DEFAULT-VLAN) | 2100 (VL-staff-wired-01) | 10.21.3.x | 5 outlets |

Only untagged VLAN membership on the listed ports changes. The full port list is in Appendix A. Device addressing, DHCP, DNS, firewall rules and any device-side changes are outside the scope of this change.

Impact

- AV in rooms 4.01, 4.12 and 5.11 is unavailable while its ports are moved

- CCTV recording from 13 cameras has a gap while their ports are moved

- 2 APs and their Wi-Fi clients drop while the AP ports are moved

- The level 3 student corridor printer is offline while its port is moved

- Devices on the 22 Level 3 staff outlets (200V3B20 to B24, C1 and C5 to C20) lose their connection while their ports are moved

- No other ports, VLANs or services on the three stacks are changed

Duration of impact

Each device is offline only while its port is moved and then power-cycled (PoE ports) or bounced (other ports), and until it is back on the network on its new VLAN. All work takes place within the change window under Suggested Scheduling.

How Change implemented

The change will be implemented by Alam Singh (Dycom) over SSH to each stack, using the prepared configuration file 200V-pilot-vlan-migration-ICX.txt. That file has pre-check, change, port bounce, verification and rollback blocks for each stack. The port bounce power-cycles PoE ports with inline power and bounces the link on the other ports. The stacks are done one at a time: lv3 first, then lv6, then lv4. lv3 has an AP, cameras, the printer and the staff outlets, so it is done first.

Prerequisites (status at 4 October 2026)

| Prerequisite | Status |
| --- | --- |
| VLANs 2010, 2100, 2600, 2700 and 2800 exist on the stacks that use them and are tagged on each stack's uplink | Complete |
| Running-config backup of each stack taken and stored off the switch | Complete |
| AP port tagging confirmed on lv3 1/1/6 and lv6 1/1/19 (untagged 100, SSID VLANs tagged) | Complete |
| lv6 port 2/1/30 confirmed (unmanaged switch behind it, AV Switcher and PDU) | Complete |
| Change approved | Complete |
| Trinity has confirmed the DHCP reservations are in place for the devices | Complete |

If a stack fails verification, that stack is rolled back before the next one is started.

How rollback implemented

Each stack is rolled back with its ROLLBACK block from the configuration file, which puts each port back on its old VLAN. A single failed device doesn't need a whole-stack rollback: only that port's lines are used.

Roll back a stack if

- a device's MAC doesn't show up on its new VLAN within 10 minutes of the port bounce

- an AP doesn't show up in SmartZone within 15 minutes of its port moving

- anything unexpected drops, such as an uplink or devices not in Appendix A

- Trinity asks for a rollback

Rollback steps

- Paste that stack's ROLLBACK block (or just the lines for the affected port).

- Paste the ROLLBACK PORT BOUNCE block.

- Check the ports are back on their old VLAN and the MACs have been learned, then run write memory.

- Record the affected ports and the cause in the change record.

How change tested

A port passes when all three of these are true:

- show vlan ethernet <port> shows the new VLAN untagged

- the AP ports still carry the same tagged VLANs as before

- show mac-address vlan <new VLAN> lists the device's MAC on that port (for a staff outlet, the MAC of whatever is connected to it)

The change is complete when every port in Appendix A passes and write memory has been run on all three stacks.

Risks of implementing

- If a new VLAN is not carried on a stack's uplink, then every device moved to it on that stack goes offline. This has low likelihood because the new VLANs were confirmed tagged on each stack's uplink on 30 September and 2 October 2026. If this risk occurs, that stack is rolled back with its ROLLBACK block.

- If a port number in the source sheet is wrong, then the wrong device is moved. This has low likelihood because each port's VLAN and MAC address are checked against the sheet before any change, and the lv3 port VLANs have already been confirmed. If this risk occurs, that port is left out or moved back.

- If the AP ports lose their tagged VLANs, then Wi-Fi on those two APs stops. This has low likelihood because only the untagged VLAN is changed, and the tagged VLANs were recorded on 30 September 2026 and are checked afterwards. If this risk occurs, the AP port is rolled back.

- If a device does not ask for a new address after its port is moved, then it stays unreachable on its old address. This has low likelihood because PoE devices are power-cycled from the switch (no inline power, then inline power) so they reboot, and the other ports get a link bounce. If this risk occurs, the device is restarted or its port is rolled back.

Suggested Scheduling

Monday 5 October 2026, 8:00 am to 12:00 pm. The window allows for the change, verification of each stack and rollback if needed.

Main cutover – all 200V switches

The pilot on Monday 5 October 2026 moved 60 ports on lv3, lv4 and lv6 and all were verified. This section covers the main cutover of the remaining 200V devices.

What is the change

Move the remaining 353 devices at 200V from their legacy VLANs to the new VLANs (2010, 2510, 2600, 2700 and 2800) by changing the untagged VLAN on 340 access ports on five RUCKUS ICX FastIron 10.0.10g switches.

| Switch | Management IP | Ports | Order | Notes |
| --- | --- | --- | --- | --- |
| switch-200V-LT2-1 | 192.168.122.12 | 16 | 1 | Smallest, done first |
| switch-200V-stack-lv3 | 192.168.122.22 | 84 | 2 |  |
| switch-200V-stack-lv4 | 192.168.122.21 | 89 | 3 |  |
| switch-200V-stack-lv6 | 192.168.122.11 | 145 | 4 | Largest; do after the system-max change |
| switch-200V-Tutorial-6.03 | 192.168.122.15 | 6 | 5 | Only once remote access is restored |

| Device type | Old VLAN | New VLAN | New subnet | Devices / ports |
| --- | --- | --- | --- | --- |
| AV | 1230 (200V_av) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 255 / 242 |
| AP management | 100 (management) | 2010 (VL-wifimgmt-01) | 10.21.1.x | 37 / 37 |
| AV (Airtame) | 1272 (200V_w_wifi) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 37 / 37 |
| AV (Room NUC and others) | 1240 (200V_staff) | 2700 (VL-av-01) | 10.21.36.x / 10.21.37.x | 13 / 13 |
| Printer | 1240 (200V_staff) | 2800 (VL-print-01) | 10.21.27.x | 6 / 6 |
| CCTV | 1212 (200V_IPcam) | 2600 (VL-cctv-01) | 10.21.25.x | 4 / 4 |
| Building access | 1213 (200V_building) | 2510 (VL-bac-01) | 10.21.22.x | 1 / 1 |

Only untagged VLAN membership on the listed ports changes. Tagged VLANs on AP and other trunk ports are not changed. The full device list is in Appendix B. Device addressing, DHCP, DNS, firewall rules and any device-side changes are outside the scope of this change. switch-200V-8, LT2-2 and LT2-3 have no listed devices and are not changed. Ports with no listed device, and devices not found on any switch, are tracked on the Review tab of 200V Device Port List.xlsx and are not part of this change.

Impact

- AV across 200V (305 devices on 292 ports) is unavailable while its ports are moved

- 37 APs and their Wi-Fi clients drop while the AP ports are moved

- CCTV recording from 4 cameras has a gap while their ports are moved

- 6 printers are offline while their ports are moved

- 1 building access device (cloe-000-200v, lv6 4/1/24) is offline while its port is moved

- No other ports, VLANs or services on the five switches are changed

Duration of impact

Each device is offline only while its port is moved and bounced (disabled for 10 seconds, then enabled), and until it is back on the network with an address on its new VLAN. All work takes place within the change window under Suggested Scheduling.

How Change implemented

The change will be implemented by Alam Singh (Dycom) over SSH to each switch, using the prepared configuration file 200V-cutover-vlan-migration-ICX.txt. That file has pre-check, change, port bounce, verification and rollback blocks for each switch, and its change and rollback blocks have been checked against each switch's current configuration. The port bounce disables and re-enables each port so DHCP devices request a new address; it does not remove PoE. The switches are done one at a time: LT2-1 first, then lv3, lv4 and lv6, then Tutorial-6.03. 14 devices had their port taken from the 200V MAC table rather than the master list (marked * in Appendix B); these are checked in the pre-check.

Prerequisites (status at 6 October 2026)

| Prerequisite | Status |
| --- | --- |
| VLANs 2010, 2510, 2600, 2700 and 2800 exist on each switch and are tagged on its uplink to the core | Complete |
| New VLANs tagged on LT2-1 1/1/23 and 1/1/24 (trunks to LT2-2 and LT2-3) | Complete |
| system-max spanning-tree 40 set and reloaded on all switches, and spanning-tree enabled on 2600 and 2700 on lv6 (system-max-spanning-tree-40-ICX.txt) | Pending |
| Running-config backup of each switch taken and stored off the switch | Pending |
| Review tab items decided (ports filled in from the MAC table, devices not found, unlisted devices) | Pending |
| Remote access to switch-200V-Tutorial-6.03 restored | Pending |
| Trinity has confirmed the DHCP reservations are in place on the new subnets | Pending |
| Change approved | Pending |

If a switch fails verification, that switch is rolled back before the next one is started.

How rollback implemented

Each switch is rolled back with its ROLLBACK block from the configuration file, which puts each port back on its old VLAN. A single failed device doesn't need a whole-switch rollback: only that port's lines are used.

Roll back a switch if

- a device's MAC doesn't show up on its new VLAN within 10 minutes of the port bounce

- an AP doesn't show up in SmartZone within 15 minutes of its port moving

- anything unexpected drops, such as an uplink or devices not in Appendix B

- Trinity asks for a rollback

Rollback steps

- Paste that switch's ROLLBACK block (or just the lines for the affected port).

- Paste the ROLLBACK PORT BOUNCE block.

- Check the ports are back on their old VLAN and the MACs have been learned, then run write memory.

- Record the affected ports and the cause in the change record.

How change tested

A port passes when all three of these are true:

- show vlan <new VLAN> lists the port as untagged

- AP and other trunk ports still carry the same tagged VLANs as before

- show mac-address vlan <new VLAN> lists the device's MAC on that port

The change is complete when every device in Appendix B passes and write memory has been run on all five switches.

Risks of implementing

- If a port in the device list is wrong, then the wrong device is moved. This has low likelihood because each switch's ports were checked against its current configuration on 6 October 2026, and the 14 ports taken from the MAC table are checked again in the pre-check. If this risk occurs, that port is left out or moved back.

- If lv6 is changed before its spanning-tree limit is raised, then VLANs 2600 and 2700 run without spanning-tree and a patching loop on them would not be blocked. This has low likelihood if the system-max change is done first, as listed in the prerequisites. If it cannot be done first, lv6 is done last and its ports are checked for loops after the change.

- If an AP cannot reach SmartZone from VLAN 2010, then it and its Wi-Fi clients stay offline. This has low likelihood because both pilot APs joined SmartZone on 2010. If this risk occurs, the AP port is rolled back.

- If a device does not ask for a new address after its port is moved, then it stays unreachable on its old address. This has low likelihood because every port is bounced after the change. If this risk occurs, the device is restarted or its port is rolled back.

- If a paste into the switch is cut short, then some ports are not moved. This has low likelihood because consecutive ports are grouped into ranges, which keeps each block short, and every port is checked in the verify step. If this risk occurs, the missing lines are pasted again.

Suggested Scheduling

To be agreed with Trinity. Each switch can be done in its own window; lv6, with 145 ports, should have a window of its own. The system-max change, which needs a reload of each switch, should be scheduled before the cutover.

Appendix A – Affected ports

60 ports across the three stacks: 31 on lv3, 21 on lv4 and 8 on lv6. Port numbers are unit/module/port. The staff outlets are listed by their patch panel label.

| Stack | Port | Old VLAN | New VLAN | Type | Device | Room | MAC | New IP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| lv3 | 1/1/6 | 100 | 2010 | AP | r650-3d-98-40 |  | 5C:83:6C:3D:98:40 | 10.21.1.11 |
| lv3 | 2/1/1 | 1212 | 2600 | CCTV | cam-200v-AE-D7-1B |  | 00:02:D1:AE:D7:1B | 10.21.25.15 |
| lv3 | 2/1/2 | 1212 | 2600 | CCTV | cam-200v-AE-D7-3A |  | 00:02:D1:AE:D7:3A | 10.21.25.17 |
| lv3 | 2/1/3 | 1212 | 2600 | CCTV | cam-200v-ae-d6-f4 |  | 00:02:D1:AE:D6:F4 | 10.21.25.25 |
| lv3 | 2/1/4 | 1212 | 2600 | CCTV | cam-200v-AE-D6-F7 |  | 00:02:D1:AE:D6:F7 | 10.21.25.22 |
| lv3 | 2/1/5 | 1212 | 2600 | CCTV | cam-200v-AE-D7-0A |  | 00:02:D1:AE:D7:0A | 10.21.25.14 |
| lv3 | 2/1/6 | 1212 | 2600 | CCTV | cam-200v-AE-D6-ED |  | 00:02:D1:AE:D6:ED | 10.21.25.16 |
| lv3 | 2/1/7 | 1212 | 2600 | CCTV | cam-200v-AE-D7-01 |  | 00:02:D1:AE:D7:01 | 10.21.25.21 |
| lv3 | 3/1/37 | 1240 | 2800 | Printer | 200v-lvl3-student-corridor |  | DC:C2:C9:76:56:67 | 10.21.27.6 |
| lv3 | 3/1/44 | 1240 | 2100 | Staff | Outlet 200V3B20 |  |  |  |
| lv3 | 3/1/45 | 1240 | 2100 | Staff | Outlet 200V3B21 |  |  |  |
| lv3 | 3/1/46 | 1240 | 2100 | Staff | Outlet 200V3B22 |  |  |  |
| lv3 | 3/1/47 | 1 | 2100 | Staff | Outlet 200V3B23 |  |  |  |
| lv3 | 3/1/48 | 1240 | 2100 | Staff | Outlet 200V3B24 |  |  |  |
| lv3 | 4/1/1 | 1240 | 2100 | Staff | Outlet 200V3C1 |  |  |  |
| lv3 | 4/1/5 | 1 | 2100 | Staff | Outlet 200V3C5 |  |  |  |
| lv3 | 4/1/6 | 1 | 2100 | Staff | Outlet 200V3C6 |  |  |  |
| lv3 | 4/1/7 | 1240 | 2100 | Staff | Outlet 200V3C7 |  |  |  |
| lv3 | 4/1/8 | 1240 | 2100 | Staff | Outlet 200V3C8 |  |  |  |
| lv3 | 4/1/9 | 1240 | 2100 | Staff | Outlet 200V3C9 |  |  |  |
| lv3 | 4/1/10 | 1240 | 2100 | Staff | Outlet 200V3C10 |  |  |  |
| lv3 | 4/1/11 | 1240 | 2100 | Staff | Outlet 200V3C11 |  |  |  |
| lv3 | 4/1/12 | 1240 | 2100 | Staff | Outlet 200V3C12 |  |  |  |
| lv3 | 4/1/13 | 1 | 2100 | Staff | Outlet 200V3C13 |  |  |  |
| lv3 | 4/1/14 | 1 | 2100 | Staff | Outlet 200V3C14 |  |  |  |
| lv3 | 4/1/15 | 1240 | 2100 | Staff | Outlet 200V3C15 |  |  |  |
| lv3 | 4/1/16 | 1240 | 2100 | Staff | Outlet 200V3C16 |  |  |  |
| lv3 | 4/1/17 | 1240 | 2100 | Staff | Outlet 200V3C17 |  |  |  |
| lv3 | 4/1/18 | 1240 | 2100 | Staff | Outlet 200V3C18 |  |  |  |
| lv3 | 4/1/19 | 1240 | 2100 | Staff | Outlet 200V3C19 |  |  |  |
| lv3 | 4/1/20 | 1240 | 2100 | Staff | Outlet 200V3C20 |  |  |  |
| lv4 | 1/1/38 | 1230 | 2700 | AV | Touch Panel | 4.12 Multipurpose Room | C4:42:68:6D:30:08 | 10.21.36.151 |
| lv4 | 1/1/42 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 4.01 Open Collaboration | C4:42:68:59:EB:95 | 10.21.36.82 |
| lv4 | 2/1/1 | 1212 | 2600 | CCTV | cam-200v-ae-d7-00 |  | 00:02:D1:AE:D7:00 | 10.21.25.26 |
| lv4 | 2/1/2 | 1212 | 2600 | CCTV | cam-200v-ae-d7-09 |  | 00:02:D1:AE:D7:09 | 10.21.25.27 |
| lv4 | 2/1/3 | 1212 | 2600 | CCTV | cam-200v-AE-D6-FF |  | 00:02:D1:AE:D6:FF | 10.21.25.19 |
| lv4 | 2/1/4 | 1212 | 2600 | CCTV | cam-200v-AE-D6-F6 |  | 00:02:D1:AE:D6:F6 | 10.21.25.18 |
| lv4 | 2/1/5 | 1212 | 2600 | CCTV | cam-200v-AE-D6-F5 |  | 00:02:D1:AE:D6:F5 | 10.21.25.20 |
| lv4 | 2/1/6 | 1212 | 2600 | CCTV | cam-200v-AE-D6-F9 |  | 00:02:D1:AE:D6:F9 | 10.21.25.24 |
| lv4 | 2/1/7 | 1212 | 2600 | CCTV | cam-200v-AE-D6-F8 |  | 00:02:D1:AE:D6:F8 | 10.21.25.23 |
| lv4 | 2/1/15 | 1230 | 2700 | AV | Projector Screen | 4.12 Multipurpose Room | 80:34:28:14:46:0C | 10.21.36.149 |
| lv4 | 2/1/29 | 1230 | 2700 | AV | AV Switcher | 4.12 Multipurpose Room | C4:42:68:65:38:D6 | 10.21.36.144 |
| lv4 | 2/1/38 | 1230 | 2700 | AV | Occupancy Sensor | 4.12 Multipurpose Room | C4:42:68:65:5A:42 | 10.21.36.146 |
| lv4 | 2/1/39 | 1230 | 2700 | AV | Room Camera | 4.12 Multipurpose Room | B8:A4:4F:CE:EB:E4 | 10.21.36.150 |
| lv4 | 4/1/11 | 1230 | 2700 | AV | PDU | 4.12 Multipurpose Room | 28:B5:E8:6F:4F:DA | 10.21.36.147 |
| lv4 | 4/1/12 | 1230 | 2700 | AV | LCD | 4.12 Multipurpose Room | 74:C1:7E:33:03:7A | 10.21.36.145 |
| lv4 | 4/1/15 | 1230 | 2700 | AV | Projector Left | 4.12 Multipurpose Room | 38:1A:52:B2:00:B8 | 10.21.36.148 |
| lv4 | 5/1/30 | 1230 | 2700 | AV | LCD | 4.01 Open Collaboration | F8:01:B4:53:86:9A | 10.21.36.83 |
| lv4 | 5/1/33 | 1272 | 2700 | AV | Airtame | 4.01 Open Collaboration | 38:4B:76:F1:A1:73 | 10.21.36.81 |
| lv4 | 5/1/44 | 1272 | 2700 | AV | Airtame | 4.12 Multipurpose Room | 38:4B:76:F1:95:43 | 10.21.36.142 |
| lv4 | 5/1/45 | 1230 | 2700 | AV | Wireless Mic Receiver | 4.12 Multipurpose Room | 00:1B:66:39:93:95 | 10.21.36.152 |
| lv4 | 5/1/46 | 1272 | 2700 | AV | Airtame | 4.12 Multipurpose Room | 38:4B:76:F1:92:D2 | 10.21.36.143 |
| lv6 | 1/1/19 | 100 | 2010 | AP | r650-39-4b-f0 |  | C8:A6:08:39:4B:F0 | 10.21.1.37 |
| lv6 | 2/1/29 | 1240 | 2700 | AV | Room NUC | 5.11 | 48:21:0B:56:AB:F9 | 10.21.37.13 |
| lv6 | 2/1/30 | 1230 | 2700 | AV | AV Switcher | 5.11 | 00:1D:56:0A:DF:BA | 10.21.36.253 |
| lv6 | 2/1/30 | 1230 | 2700 | AV | PDU | 5.11 | B8:80:4F:28:19:F6 | 10.21.37.10 |
| lv6 | 2/1/32 | 1230 | 2700 | AV | Occupancy Sensor | 5.11 | B8:98:B0:0F:1C:5D | 10.21.36.254 |
| lv6 | 2/1/33 | 1230 | 2700 | AV | Room Camera | 5.11 | B8:A4:4F:49:4A:9B | 10.21.37.12 |
| lv6 | 2/1/34 | 1230 | 2700 | AV | Touch Panel | 5.11 | C4:42:68:15:2D:E5 | 10.21.37.14 |
| lv6 | 6/1/35 | 1272 | 2700 | AV | Airtame | 5.11 | 38:4B:76:F1:90:1D | 10.21.36.252 |
| lv6 | 6/1/41 | 1230 | 2700 | AV | Projector | 5.11 | F8:25:51:CB:8C:9A | 10.21.37.11 |

lv6 port 2/1/30 has an unmanaged switch behind it, which is why the AV Switcher and the PDU share that port.

Appendix B – Cutover devices

353 devices on 340 ports across five switches: 16 on LT2-1, 84 on lv3, 89 on lv4, 145 on lv6, 6 on Tutorial-6.03. Port numbers are unit/module/port. * = port taken from the 200V MAC table, not the master list. Devices that share a port are listed on separate rows.

| Switch | Port | Old VLAN | New VLAN | Type | Device | Location | MAC | New IP |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| LT2-1 | 1/1/1 | 100 | 2010 | AP | r650-39-49-a0 |  | C8:A6:08:39:49:A0 | 10.21.1.33 |
| LT2-1 | 1/1/4 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 6.13 Lecture Theatre | D0:C8:57:81:64:24 | 10.21.37.85 |
| LT2-1 | 1/1/5 | 1272 | 2700 | AV | Airtame | 200V - 6.13 Lecture Theatre | 38:4b:76:f1:8f:ec | 10.21.37.78 |
| LT2-1 | 1/1/6 | 1230 | 2700 | AV | PDU * | 200V - 6.13 Lecture Theatre | B8-80-4F-28-58-24 | 10.21.37.84 |
| LT2-1 | 1/1/7 | 1230 | 2700 | AV | Recorder * | 200V - 6.13 Lecture Theatre | 00-1C-08-00-6C-7C | 10.21.37.91 |
| LT2-1 | 1/1/8 | 1230 | 2700 | AV | Audio DSP * | 200V - 6.13 Lecture Theatre | 78-45-01-0C-7D-3A | 10.21.37.79 |
| LT2-1 | 1/1/10 | 1230 | 2700 | AV | LCD 2 * | 200V - 6.13 Lecture Theatre | A8-4A-63-30-73-66 | 10.21.37.82 |
| LT2-1 | 1/1/11 | 1230 | 2700 | AV | LCD 1 * | 200V - 6.13 Lecture Theatre | A8-4A-63-30-73-67 | 10.21.37.81 |
| LT2-1 | 1/1/12 | 1230 | 2700 | AV | PTZ Camera * | 200V - 6.13 Lecture Theatre | DC-E2-AC-02-E4-F5 | 10.21.37.90 |
| LT2-1 | 1/1/13 | 1230 | 2700 | AV | Room Camera * | 200V - 6.13 Lecture Theatre | B8-A4-4F-69-B2-72 | 10.21.37.92 |
| LT2-1 | 1/1/14 | 1230 | 2700 | AV | Occupancy Sensor * | 200V - 6.13 Lecture Theatre | B8-98-B0-0F-1C-66 | 10.21.37.83 |
| LT2-1 | 1/1/15 | 1230 | 2700 | AV | Projector Screen 2 * | 200V - 6.13 Lecture Theatre | 04-91-62-D7-70-0D | 10.21.37.89 |
| LT2-1 | 1/1/16 | 1230 | 2700 | AV | Projector Screen 1 * | 200V - 6.13 Lecture Theatre | 54-10-EC-A2-E8-27 | 10.21.37.88 |
| LT2-1 | 1/1/17 | 1230 | 2700 | AV | Projector 1 * | 200V - 6.13 Lecture Theatre | 38-1A-52-B1-75-B6 | 10.21.37.86 |
| LT2-1 | 1/1/18 | 1230 | 2700 | AV | Projector 2 * | 200V - 6.13 Lecture Theatre | 38-1A-52-B1-74-BA | 10.21.37.87 |
| LT2-1 | 1/1/20 | 1230 | 2700 | AV | Touch Panel * | 200V - 6.13 Lecture Theatre | C4-42-68-26-D6-48 | 10.21.37.93 |
| lv3 | 1/1/1 | 100 | 2010 | AP | r650-19-9d-00 |  | C8:A6:08:19:9D:00 | 10.21.1.21 |
| lv3 | 1/1/2 | 100 | 2010 | AP | r650-19-9d-70 |  | C8:A6:08:19:9D:70 | 10.21.1.25 |
| lv3 | 1/1/3 | 100 | 2010 | AP | r650-19-9d-50 |  | C8:A6:08:19:9D:50 | 10.21.1.23 |
| lv3 | 1/1/4 | 100 | 2010 | AP | r650-19-9d-40 |  | C8:A6:08:19:9D:40 | 10.21.1.22 |
| lv3 | 1/1/5 | 100 | 2010 | AP | r650-19-9d-80 |  | C8:A6:08:19:9D:80 | 10.21.1.26 |
| lv3 | 1/1/7 | 100 | 2010 | AP | r650-32-68-f0 |  | CC:1B:5A:32:68:F0 | 10.21.1.49 |
| lv3 | 1/1/8 | 100 | 2010 | AP | r650-32-61-e0 |  | CC:1B:5A:32:61:E0 | 10.21.1.47 |
| lv3 | 1/1/9 | 100 | 2010 | AP | r650-3d-98-50 |  | 5C:83:6C:3D:98:50 | 10.21.1.12 |
| lv3 | 1/1/10 | 100 | 2010 | AP | r650-19-9d-a0 |  | C8:A6:08:19:9D:A0 | 10.21.1.27 |
| lv3 | 1/1/11 | 100 | 2010 | AP | r650-19-9d-60 |  | C8:A6:08:19:9D:60 | 10.21.1.24 |
| lv3 | 1/1/13 | 1230 | 2700 | AV | Touch Panel | 200V - 3.20 Meeting Room | C4:42:68:60:C1:B6 | 10.21.36.79 |
| lv3 | 1/1/14 | 1230 | 2700 | AV | Room Camera | 200V - 3.10 | B8:A4:4F:CB:CB:F7 | 10.21.36.71 |
| lv3 | 1/1/15 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.10 | C4:42:68:65:7E:7F | 10.21.36.68 |
| lv3 | 1/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 3.10 | C4:42:68:6C:42:DC | 10.21.36.72 |
| lv3 | 1/1/17 | 1230 | 2700 | AV | Room Camera | 200V - 3.09 | B8:A4:4F:CB:C6:FB | 10.21.36.64 |
| lv3 | 1/1/18 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.09 | C4:42:68:65:8E:04 | 10.21.36.61 |
| lv3 | 1/1/19 | 1230 | 2700 | AV | Touch Panel | 200V - 3.09 | C4:42:68:6C:44:8E | 10.21.36.65 |
| lv3 | 1/1/20 | 1230 | 2700 | AV | Room Camera | 200V - 3.08 | B8:A4:4F:CE:F4:C6 | 10.21.36.57 |
| lv3 | 1/1/21 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.08 | C4:42:68:65:7D:A0 | 10.21.36.54 |
| lv3 | 1/1/22 | 1230 | 2700 | AV | Touch Panel | 200V - 3.08 | C4:42:68:6D:2B:F1 | 10.21.36.58 |
| lv3 | 1/1/23 | 1230 | 2700 | AV | Touch Panel | 200V - 3.06 | C4:42:68:6D:C3:79 | 10.21.36.51 |
| lv3 | 1/1/24 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.06 | C4:42:68:65:91:9B | 10.21.36.47 |
| lv3 | 1/1/25 | 1230 | 2700 | AV | Room Camera | 200V - 3.06 | B8:A4:4F:CE:E5:EB | 10.21.36.50 |
| lv3 | 1/1/26 | 1230 | 2700 | AV | Room Camera | 200V - 3.05 | B8:A4:4F:CB:CA:CC | 10.21.36.43 |
| lv3 | 1/1/27 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.05 | C4:42:68:65:95:E7 | 10.21.36.40 |
| lv3 | 1/1/28 | 1230 | 2700 | AV | Touch Panel | 200V - 3.05 | C4:42:68:6D:C3:9E | 10.21.36.44 |
| lv3 | 1/1/29 | 1230 | 2700 | AV | Touch Panel | 200V - 3.04 | C4:42:68:6D:C3:AB | 10.21.36.37 |
| lv3 | 1/1/30 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.04 | C4:42:68:65:8F:C4 | 10.21.36.33 |
| lv3 | 1/1/31 | 1230 | 2700 | AV | Room Camera | 200V - 3.04 | B8:A4:4F:CE:E4:99 | 10.21.36.36 |
| lv3 | 1/1/32 | 1230 | 2700 | AV | Room Camera | 200V - 3.03 | B8:A4:4F:CB:C8:54 | 10.21.36.29 |
| lv3 | 1/1/33 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.03 | C4:42:68:65:91:74 | 10.21.36.23 |
| lv3 | 1/1/35 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.15 Consult Room | C4:42:68:1C:22:3B | 10.21.36.75 |
| lv3 | 1/1/36 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.14 Consult Room | C4:42:68:46:A3:3F | 10.21.36.73 |
| lv3 | 2/1/13 | 1230 | 2700 | AV | Projector | 200V - 3.03 | F8:25:51:D2:70:8E | 10.21.36.26 |
| lv3 | 2/1/15 | 1230 | 2700 | AV | Touch Panel | 200V - 3.03 | C4:42:68:6D:BF:ED | 10.21.36.30 |
| lv3 | 2/1/18 | 1272 | 2700 | AV | Airtame | 200V - 3.03 | 38:4B:76:F1:83:52 | 10.21.36.20 |
| lv3 | 2/1/19 | 1230 | 2700 | AV | PDU | 200V - 3.03 | 28:B5:E8:6F:27:18 | 10.21.36.24 |
| lv3 | 2/1/20 | 1230 | 2700 | AV | AV Switcher | 200V - 3.03 | C4:42:68:65:35:E8 | 10.21.36.21 |
| lv3 | 2/1/21 | 1230 | 2700 | AV | Room Camera | 200V - 3.02 | B8:A4:4F:CB:C5:56 | 10.21.36.18 |
| lv3 | 2/1/22 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 3.02 | C4:42:68:65:87:C4 | 10.21.36.15 |
| lv3 | 2/1/23 | 1230 | 2700 | AV | Projector | 200V - 3.02 | F8:25:51:CC:0F:7D | 10.21.36.17 |
| lv3 | 2/1/25 | 1230 | 2700 | AV | PDU | 200V - 3.02 | 28:B5:E8:6F:CB:2E | 10.21.36.16 |
| lv3 | 2/1/26 | 1230 | 2700 | AV | Touch Panel | 200V - 3.02 | C4:42:68:6D:C3:9C | 10.21.36.19 |
| lv3 | 2/1/28 | 1272 | 2700 | AV | AV Switcher | 200V - 3.02 | C4:42:68:65:39:D8 | 10.21.36.14 |
| lv3 | 2/1/30 | 1272 | 2700 | AV | Airtame | 200V - 3.02 | 38:4B:76:F1:83:51 | 10.21.36.13 |
| lv3 | 2/1/31 | 1230 | 2700 | AV | LCD | 200V - 3.01 Collaboration Space | F8:01:B4:53:86:B8 | 10.21.36.12 |
| lv3 | 2/1/32 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.01 Collaboration Space | C4:42:68:59:A4:45 | 10.21.36.11 |
| lv3 | 2/1/33 | 1272 | 2700 | AV | Airtame | 200V - 3.01 Collaboration Space | 38:4B:76:F1:95:78 | 10.21.36.10 |
| lv3 | 2/1/35 | 1230 | 2700 | AV | PTZ Camera | 200V - 3.03 | DC:E2:AC:02:E8:CA | 10.21.36.27 |
| lv3 | 2/1/36 | 1230 | 2700 | AV | Mic Charger | 200V - 3.03 | 00:1B:66:3F:15:17 | 10.21.36.22 |
| lv3 | 2/1/37 | 1230 | 2700 | AV | Recorder | 200V - 3.03 | 00:1C:08:00:70:A3 | 10.21.36.28 |
| lv3 | 2/1/38 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 3.03 | D0:C8:57:80:9F:8E | 10.21.36.25 |
| lv3 | 2/1/39 | 1230 | 2700 | AV | Keypad for Room-In-Use light | 200V - 3.16 Consult Room | C4:42:68:46:82:ED | 10.21.36.76 |
| lv3 | 2/1/40 | 1230 | 2700 | AV | Cresnet Bridge For Outside Sign | 200V - 3.15 Consult Room | C4:42:68:5D:43:1D | 10.21.36.74 |
| lv3 | 4/1/4 | 1240 | 2800 | printer | 200v-lvl3-staffroom |  | 34:9F:7B:5E:9A:04 | 10.21.27.5 |
| lv3 | 4/1/40 | 1230 | 2700 | AV | AV Switcher | 200V - 3.10 | C4:42:68:65:29:8D | 10.21.36.67 |
| lv3 | 4/1/42 | 1230 | 2700 | AV | PDU | 200V - 3.10 | 28:B5:E8:68:78:AF | 10.21.36.69 |
| lv3 | 4/1/44 | 1230 | 2700 | AV | Projector | 200V - 3.10 | F8:25:51:D2:70:8A | 10.21.36.70 |
| lv3 | 4/1/47 | 1272 | 2700 | AV | Airtame | 200V - 3.09 | 8C:AE:4C:D2:7C:B8 | 10.21.36.59 |
| lv3 | 4/1/48 | 1230 | 2700 | AV | PDU | 200V - 3.09 | 28:B5:E8:6F:33:1E | 10.21.36.62 |
| lv3 | 5/1/1 | 1230 | 2700 | AV | LCD | 200V - 3.20 Meeting Room | 74:C1:7E:EC:3D:90 | 10.21.36.78 |
| lv3 | 5/1/2 | 1230 | 2700 | AV | UC Engine | 200V - 3.20 Meeting Room | 90:8D:6E:95:8E:ED | 10.21.36.80 |
| lv3 | 5/1/3 | 1272 | 2700 | AV | Airtame | 200V - 3.20 Meeting Room | 38:4B:76:F1:AB:19 | 10.21.36.77 |
| lv3 | 5/1/8 | 1230 | 2700 | AV | Display | 200V - L3 Digital Signage 2 | 28:E6:A9:8D:5A:69 | 10.21.37.95 |
| lv3 | 5/1/9 | 1272 | 2700 | AV | Airtame | 200V - 3.10 | 8C:AE:4C:D2:7C:BC | 10.21.36.66 |
| lv3 | 5/1/10 | 1230 | 2700 | AV | AV Switcher | 200V - 3.09 | C4:42:68:65:3F:BD | 10.21.36.60 |
| lv3 | 5/1/12 | 1230 | 2700 | AV | Projector | 200V - 3.09 | F8:25:51:CD:3F:DA | 10.21.36.63 |
| lv3 | 5/1/14 | 1230 | 2700 | AV | Projector | 200V - 3.08 | F8:25:51:CF:D3:D8 | 10.21.36.56 |
| lv3 | 5/1/16 | 1230 | 2700 | AV | PDU | 200V - 3.08 | 28:B5:E8:6F:CB:5E | 10.21.36.55 |
| lv3 | 5/1/17 | 1272 | 2700 | AV | Airtame | 200V - 3.08 | 8C:AE:4C:D2:7C:D8 | 10.21.36.52 |
| lv3 | 5/1/19 | 1230 | 2700 | AV | Projector | 200V - 3.05 | F8:25:51:D2:70:34 | 10.21.36.42 |
| lv3 | 5/1/21 | 1272 | 2700 | AV | Airtame | 200V - 3.05 | 38:4B:76:F1:95:77 | 10.21.36.38 |
| lv3 | 5/1/22 | 1230 | 2700 | AV | PDU | 200V - 3.05 | 28:B5:E8:68:7E:2B | 10.21.36.41 |
| lv3 | 5/1/25 | 1230 | 2700 | AV | AV Switcher | 200V - 3.05 | C4:42:68:65:3F:74 | 10.21.36.39 |
| lv3 | 5/1/27 | 1272 | 2700 | AV | Airtame | 200V - 3.04 | 38:4B:76:F1:95:75 | 10.21.36.31 |
| lv3 | 5/1/28 | 1230 | 2700 | AV | PDU | 200V - 3.04 | 28:B5:E8:6F:4A:58 | 10.21.36.34 |
| lv3 | 5/1/30 | 1230 | 2700 | AV | AV Switcher | 200V - 3.04 | C4:42:68:65:3F:93 | 10.21.36.32 |
| lv3 | 5/1/31 | 1230 | 2700 | AV | Projector | 200V - 3.04 | F8:25:51:CD:3F:E4 | 10.21.36.35 |
| lv3 | 5/1/33 | 1230 | 2700 | AV | Display | 200V - L3 Digital Signage 1 | 28:E6:A9:8D:5A:60 | 10.21.37.94 |
| lv3 | 5/1/41 | 1230 | 2700 | AV | AV Switcher | 200V - 3.08 | C4:42:68:65:38:8A | 10.21.36.53 |
| lv3 | 5/1/42 | 1230 | 2700 | AV | Projector | 200V - 3.06 | F8:25:51:D2:70:90 | 10.21.36.49 |
| lv3 | 5/1/44 | 1230 | 2700 | AV | PDU | 200V - 3.06 | 28:B5:E8:6F:D1:0D | 10.21.36.48 |
| lv3 | 5/1/46 | 1272 | 2700 | AV | Airtame | 200V - 3.06 | 8C:AE:4C:D2:00:95 | 10.21.36.45 |
| lv3 | 5/1/48 | 1230 | 2700 | AV | AV Switcher | 200V - 3.06 | C4:42:68:65:2F:04 | 10.21.36.46 |
| lv4 | 1/1/1 | 100 | 2010 | AP | r650-3d-bf-30 |  | 5C:83:6C:3D:BF:30 | 10.21.1.14 |
| lv4 | 1/1/2 | 100 | 2010 | AP | r650-19-9c-c0 |  | C8:A6:08:19:9C:C0 | 10.21.1.20 |
| lv4 | 1/1/3 | 100 | 2010 | AP | r650-19-9c-a0 |  | C8:A6:08:19:9C:A0 | 10.21.1.19 |
| lv4 | 1/1/4 | 100 | 2010 | AP | r650-3d-4c-80 |  | 5C:83:6C:3D:4C:80 | 10.21.1.10 |
| lv4 | 1/1/5 | 100 | 2010 | AP | r650-19-9c-90 |  | C8:A6:08:19:9C:90 | 10.21.1.18 |
| lv4 | 1/1/6 | 100 | 2010 | AP | r650-19-9c-80 |  | C8:A6:08:19:9C:80 | 10.21.1.17 |
| lv4 | 1/1/7 | 100 | 2010 | AP | r650-19-9b-90 |  | C8:A6:08:19:9B:90 | 10.21.1.15 |
| lv4 | 1/1/9 | 100 | 2010 | AP | r650-19-9c-70 |  | C8:A6:08:19:9C:70 | 10.21.1.16 |
| lv4 | 1/1/10 | 100 | 2010 | AP | r650-3d-be-c0 |  | 5C:83:6C:3D:BE:C0 | 10.21.1.13 |
| lv4 | 1/1/11 | 100 | 2010 | AP | r650-32-66-a0 |  | CC:1B:5A:32:66:A0 | 10.21.1.48 |
| lv4 | 1/1/12 | 1240 | 2800 | printer | 200v-lvl4-staffroom |  | 34:9F:7B:5F:D4:1E | 10.21.27.7 |
| lv4 | 1/1/13 | 1230 | 2700 | AV | PDU | 200V - 4.08 | 28:B5:E8:6F:CB:73 | 10.21.36.126 |
| lv4 | 1/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 4.07 | C4:42:68:6D:2B:79 | 10.21.36.122 |
| lv4 | 1/1/18 | 1230 | 2700 | AV | Projector | 200V - 4.05 | F8:25:51:CD:3F:E3 | 10.21.36.113 |
| lv4 | 1/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.05 | C4:42:68:65:91:53 | 10.21.36.111 |
| lv4 | 1/1/20 | 1272 | 2700 | AV | Airtame | 200V - 4.05 | 38:4B:76:F1:A3:28 | 10.21.36.109 |
| lv4 | 1/1/22 | 1230 | 2700 | AV | Projector | 200V - 4.04 | F8:25:51:CF:D4:66 | 10.21.36.106 |
| lv4 | 1/1/23 | 1230 | 2700 | AV | Touch Panel | 200V - 4.04 | C4:42:68:6C:2D:D7 | 10.21.36.108 |
| lv4 | 1/1/24 | 1272 | 2700 | AV | Airtame | 200V - 4.04 | 38:4B:76:F1:A1:72 | 10.21.36.102 |
| lv4 | 1/1/25 | 1230 | 2700 | AV | PDU | 200V - 4.04 | 28:B5:E8:68:78:FD | 10.21.36.105 |
| lv4 | 1/1/26 | 1230 | 2700 | AV | Room Camera | 200V - 4.03 | B8:A4:4F:CB:C5:7C | 10.21.36.100 |
| lv4 | 1/1/27 | 1230 | 2700 | AV | AV Switcher | 200V - 4.03 | C4:42:68:65:68:A6 | 10.21.36.92 |
| lv4 | 1/1/29 | 1272 | 2700 | AV | Airtame | 200V - 4.03 | 38:4B:76:F1:83:57 | 10.21.36.91 |
| lv4 | 1/1/31 | 1230 | 2700 | AV | Projector | 200V - 4.02 | F8:25:51:CC:0F:80 | 10.21.36.88 |
| lv4 | 1/1/33 | 1230 | 2700 | AV | Touch Panel | 200V - 4.02 | C4:42:68:6D:2C:10 | 10.21.36.90 |
| lv4 | 1/1/35 | 1230 | 2700 | AV | Room Camera | 200V - 4.07 | B8:A4:4F:CB:D2:17 | 10.21.36.121 |
| lv4 | 1/1/36 | 1230 | 2700 | AV | Projector Screen Right | 200V - 4.13 Lecture Theatre | 80:1F:12:4E:EB:71 | 10.21.36.167 |
| lv4 | 1/1/37 | 1230 | 2700 | AV | Pro Convert for NDI to HDMI | 200V - 4.13 Lecture Theatre | D0:C8:57:80:AA:C1 | 10.21.36.163 |
| lv4 | 1/1/39 | 1230 | 2700 | AV | Touch Panel | 200V - 4.10 | C4:42:68:6C:40:90 | 10.21.36.136 |
| lv4 | 1/1/40 | 1230 | 2700 | AV | Touch Panel | 200V - 4.05 | C4:42:68:6C:35:04 | 10.21.36.115 |
| lv4 | 1/1/41 | 1230 | 2700 | AV | Touch Panel | 200V - 4.03 | C4:42:68:6C:42:6B | 10.21.36.101 |
| lv4 | 1/1/43 | 1230 | 2700 | AV | Pro convert for NDI to HDMI | 200V - 4.03 | D0:C8:57:82:8B:22 | 10.21.36.96 |
| lv4 | 1/1/44 | 1230 | 2700 | AV | PTZ Camera | 200V - 4.03 | DC:E2:AC:02:EC:B8 | 10.21.36.98 |
| lv4 | 2/1/13 | 1230 | 2700 | AV | Wireless Mic Receiver | 200V - 4.13 Lecture Theatre | 00:1B:66:39:93:93 | 10.21.36.172 |
| lv4 | 2/1/14 | 1272 | 2700 | AV | Airtame | 200V - 4.13 Lecture Theatre | 38:4B:76:F1:4D:F1 | 10.21.36.153 |
| lv4 | 2/1/16 | 1230 | 2700 | AV | Touch Panel | 200V - 4.11 Drama Room | C4:42:68:6C:43:F3 | 10.21.36.141 |
| lv4 | 2/1/17 | 1230 | 2700 | AV | PDU | 200V - 4.11 Drama Room | 98:89:24:0D:AF:FA | 10.21.36.138 |
| lv4 | 2/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.10 | C4:42:68:65:8F:7B | 10.21.36.132 |
| lv4 | 2/1/22 | 1272 | 2700 | AV | Airtame | 200V - 4.10 | 38:4B:76:F1:9F:90 | 10.21.36.130 |
| lv4 | 2/1/23 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.08 | C4:42:68:65:95:96 | 10.21.36.125 |
| lv4 | 2/1/24 | 1272 | 2700 | AV | Airtame | 200V - 4.08 | 38:4B:76:F1:A2:70 | 10.21.36.123 |
| lv4 | 2/1/25 | 1230 | 2700 | AV | Touch Panel | 200V - 4.13 Lecture Theatre | C4:42:68:3E:0A:00 | 10.21.36.171 |
| lv4 | 2/1/26 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.13 Lecture Theatre | C4:42:68:65:96:20 | 10.21.36.160 |
| lv4 | 2/1/27 | 1230 | 2700 | AV | Cresnet Bridge For Outside Sign | 200V - 4.13 Lecture Theatre | C4:42:68:5D:50:D2 | 10.21.36.156 |
| lv4 | 2/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 4.13 Lecture Theatre | 00:1D:56:0A:8E:47 | 10.21.36.155 |
| lv4 | 2/1/30 | 1230 | 2700 | AV | Audio DSP | 200V - 4.13 Lecture Theatre | 78:45:01:1A:93:AF | 10.21.36.154 |
| lv4 | 2/1/33 | 1230 | 2700 | AV | Room Camera | 200V - 4.04 | B8:A4:4F:CB:C4:3D | 10.21.36.107 |
| lv4 | 2/1/34 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.04 | C4:42:68:65:91:9A | 10.21.36.104 |
| lv4 | 2/1/35 | 1230 | 2700 | AV | Room Camera | 200V - 4.02 | B8:A4:4F:CE:F3:7D | 10.21.36.89 |
| lv4 | 2/1/36 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.02 | C4:42:68:65:91:85 | 10.21.36.86 |
| lv4 | 2/1/37 | 1230 | 2700 | AV | Room Camera | 200V - 4.13 Lecture Theatre | B8:A4:4F:CB:CA:ED | 10.21.36.170 |
| lv4 | 2/1/40 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.11 Drama Room | C4:42:68:65:91:77 | 10.21.36.137 |
| lv4 | 2/1/41 | 1230 | 2700 | AV | Room Camera | 200V - 4.11 Drama Room | B8:A4:4F:CE:F1:43 | 10.21.36.140 |
| lv4 | 2/1/42 | 1230 | 2700 | AV | Room Camera | 200V - 4.10 | B8:A4:4F:CE:EF:42 | 10.21.36.135 |
| lv4 | 2/1/43 | 1230 | 2700 | AV | Room Camera | 200V - 4.08 | B8:A4:4F:CE:EA:58 | 10.21.36.128 |
| lv4 | 2/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.07 | C4:42:68:65:91:8B | 10.21.36.118 |
| lv4 | 2/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 4.05 | B8:A4:4F:CB:CA:0C | 10.21.36.114 |
| lv4 | 2/1/46 | 1230 | 2700 | AV | Touch Panel | 200V - 4.08 | C4:42:68:6D:2F:E4 | 10.21.36.129 |
| lv4 | 2/1/47 | 1230 | 2700 | AV | AV Switcher | 200V - 4.08 | C4:42:68:65:3F:79 | 10.21.36.124 |
| lv4 | 2/1/48 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 4.03 | C4:42:68:65:95:E0 | 10.21.36.94 |
| lv4 | 4/1/3 | 1230 | 2700 | AV | Mic Charger | 200V - 4.13 Lecture Theatre | 00:1B:66:3F:0D:8B | 10.21.36.159 |
| lv4 | 4/1/5 | 1230 | 2700 | AV | PDU 1 | 200V - 4.13 Lecture Theatre | 28:B5:E8:6F:27:66 | 10.21.36.161 |
| lv4 | 4/1/8 | 1230 | 2700 | AV | LCD Left | 200V - 4.13 Lecture Theatre | F8:01:B4:AE:DF:9E | 10.21.36.157 |
| lv4 | 4/1/9 | 1230 | 2700 | AV | PTZ Camera | 200V - 4.13 Lecture Theatre | DC:E2:AC:02:EC:9D | 10.21.36.168 |
| lv4 | 4/1/18 | 1230 | 2700 | AV | Projector | 200V - 4.11 Drama Room | F8:25:51:D2:70:36 | 10.21.36.139 |
| lv4 | 4/1/22 | 1230 | 2700 | AV | Projector | 200V - 4.10 | F8:25:51:CD:40:32 | 10.21.36.134 |
| lv4 | 4/1/24 | 1230 | 2700 | AV | PDU | 200V - 4.10 | 28:B5:E8:6F:CB:40 | 10.21.36.133 |
| lv4 | 4/1/25 | 1230 | 2700 | AV | AV Switcher | 200V - 4.10 | C4:42:68:65:31:AC | 10.21.36.131 |
| lv4 | 4/1/37 | 1230 | 2700 | AV | LCD Right | 200V - 4.13 Lecture Theatre | F8:01:B4:60:4C:AB | 10.21.36.158 |
| lv4 | 4/1/41 | 1230 | 2700 | AV | Projector Screen Left | 200V - 4.13 Lecture Theatre | 80:34:28:5C:F1:F2 | 10.21.36.166 |
| lv4 | 4/1/42 | 1230 | 2700 | AV | Recorder | 200V - 4.13 Lecture Theatre | 00:1C:08:00:70:2D | 10.21.36.169 |
| lv4 | 4/1/43 | 1230 | 2700 | AV | PDU 2 | 200V - 4.13 Lecture Theatre | 28:B5:E8:6F:61:FB | 10.21.36.162 |
| lv4 | 5/1/4 | 1230 | 2700 | AV | Projector | 200V - 4.07 | F8:25:51:CD:3F:E0 | 10.21.36.120 |
| lv4 | 5/1/8 | 1230 | 2700 | AV | PDU | 200V - 4.07 | 28:B5:E8:68:81:A6 | 10.21.36.119 |
| lv4 | 5/1/9 | 1272 | 2700 | AV | Airtame | 200V - 4.07 | 38:4B:76:F1:A1:45 | 10.21.36.116 |
| lv4 | 5/1/10 | 1230 | 2700 | AV | AV Switcher | 200V - 4.07 | C4:42:68:65:39:C6 | 10.21.36.117 |
| lv4 | 5/1/13 | 1230 | 2700 | AV | PDU | 200V - 4.05 | 28:B5:E8:6F:CB:46 | 10.21.36.112 |
| lv4 | 5/1/15 | 1230 | 2700 | AV | AV Switcher | 200V - 4.05 | C4:42:68:65:3A:3F | 10.21.36.110 |
| lv4 | 5/1/20 | 1230 | 2700 | AV | AV Switcher | 200V - 4.04 | C4:42:68:65:37:38 | 10.21.36.103 |
| lv4 | 5/1/23 | 1230 | 2700 | AV | PDU | 200V - 4.03 | 28:B5:E8:68:7E:6D | 10.21.36.95 |
| lv4 | 5/1/25 | 1230 | 2700 | AV | Projector | 200V - 4.03 | F8:25:51:CF:D3:D9 | 10.21.36.97 |
| lv4 | 5/1/31 | 1230 | 2700 | AV | PDU | 200V - 4.02 | 28:B5:E8:6F:27:3F | 10.21.36.87 |
| lv4 | 5/1/32 | 1272 | 2700 | AV | Airtame | 200V - 4.02 | 38:4B:76:F1:92:CF | 10.21.36.84 |
| lv4 | 5/1/34 | 1230 | 2700 | AV | AV Switcher | 200V - 4.02 | C4:42:68:65:3F:D0 | 10.21.36.85 |
| lv4 | 5/1/36 | 1230 | 2700 | AV | Display | 200V - L4 Digital Signage 3 | 28:E6:A9:8D:5A:73 | 10.21.37.96 |
| lv4 | 5/1/37 | 1230 | 2700 | AV | Projector Right | 200V - 4.13 Lecture Theatre | 38:1A:52:B2:00:9F | 10.21.36.165 |
| lv4 | 5/1/39 | 1230 | 2700 | AV | Projector Left | 200V - 4.13 Lecture Theatre | 38:1A:52:B1:FF:B9 | 10.21.36.164 |
| lv4 | 5/1/41 | 1230 | 2700 | AV | Recorder | 200V - 4.03 | 00:1C:08:00:71:05 | 10.21.36.99 |
| lv4 | 5/1/42 | 1230 | 2700 | AV | Mic Charger | 200V - 4.03 | 00:1B:66:3F:15:3A | 10.21.36.93 |
| lv6 | 1/1/8 | 1230 | 2700 | AV | PDU | 200V - 6.01 Lecture Theatre | B8:80:4F:28:50:5F | 10.21.37.39 |
| lv6 | 1/1/9 | 1230 | 2700 | AV | AV Switcher | 200V - 6.01 Lecture Theatre | 00:1D:56:0A:B4:A6 | 10.21.37.34 |
| lv6 | 1/1/10 | 1230 | 2700 | AV | Audio DSP | 200V - 6.01 Lecture Theatre | 78:45:01:0C:7D:CC | 10.21.37.33 |
| lv6 | 1/1/13 | 100 | 2010 | AP | r650-39-4b-d0 |  | C8:A6:08:39:4B:D0 | 10.21.1.36 |
| lv6 | 1/1/14 | 100 | 2010 | AP | r650-39-4b-c0 |  | C8:A6:08:39:4B:C0 | 10.21.1.35 |
| lv6 | 1/1/15 | 100 | 2010 | AP | r650-39-52-00 |  | C8:A6:08:39:52:00 | 10.21.1.42 |
| lv6 | 1/1/16 | 100 | 2010 | AP | r650-39-51-00 |  | C8:A6:08:39:51:00 | 10.21.1.40 |
| lv6 | 1/1/17 | 100 | 2010 | AP | r650-39-54-a0 |  | C8:A6:08:39:54:A0 | 10.21.1.45 |
| lv6 | 1/1/18 | 100 | 2010 | AP | r650-39-4a-60 |  | C8:A6:08:39:4A:60 | 10.21.1.34 |
| lv6 | 1/1/20 | 100 | 2010 | AP | r650-39-5c-50 |  | C8:A6:08:39:5C:50 | 10.21.1.46 |
| lv6 | 1/1/21 | 100 | 2010 | AP | r650-39-2d-b0 |  | C8:A6:08:39:2D:B0 | 10.21.1.28 |
| lv6 | 1/1/22 | 100 | 2010 | AP | r650-39-35-10 |  | C8:A6:08:39:35:10 | 10.21.1.31 |
| lv6 | 1/1/23 | 100 | 2010 | AP | r650-39-52-90 |  | C8:A6:08:39:52:90 | 10.21.1.43 |
| lv6 | 1/1/24 | 100 | 2010 | AP | r650-39-33-f0 |  | C8:A6:08:39:33:F0 | 10.21.1.29 |
| lv6 | 1/1/25 | 100 | 2010 | AP | r650-39-34-60 |  | C8:A6:08:39:34:60 | 10.21.1.30 |
| lv6 | 1/1/26 | 100 | 2010 | AP | r650-39-36-80 |  | C8:A6:08:39:36:80 | 10.21.1.32 |
| lv6 | 1/1/27 | 100 | 2010 | AP | r650-39-4c-f0 |  | C8:A6:08:39:4C:F0 | 10.21.1.38 |
| lv6 | 1/1/28 | 100 | 2010 | AP | r650-39-51-e0 |  | C8:A6:08:39:51:E0 | 10.21.1.41 |
| lv6 | 1/1/29 | 100 | 2010 | AP | r650-39-54-60 |  | C8:A6:08:39:54:60 | 10.21.1.44 |
| lv6 | 1/1/37 | 1212 | 2600 | cctv | cam-200V2B23.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:61 | 10.21.25.10 |
| lv6 | 1/1/38 | 1212 | 2600 | cctv | cam-200V2F10.trinity.unimelb.edu.au |  | c0:56:e3:85:a4:d8 | 10.21.25.11 |
| lv6 | 1/1/39 | 1212 | 2600 | cctv | cam-200V1D05.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:2a | 10.21.25.12 |
| lv6 | 1/1/40 | 1212 | 2600 | cctv | cam-200V2K14.trinity.unimelb.edu.au |  | c0:56:e3:85:a5:6d | 10.21.25.13 |
| lv6 | 2/1/1 | 1230 | 2700 | AV | NDI-HDMI Decoder | 200V - 6.01 Lecture Theatre | D0:C8:57:81:64:1F | 10.21.37.37 |
| lv6 | 2/1/2 | 1230 | 2700 | AV | Room Camera | 200V - 5.05 | B8:A4:4F:68:F5:19 | 10.21.36.210 |
| lv6 | 2/1/3 | 1230 | 2700 | AV | Projector | 200V - 5.05 | F8:25:51:CB:8C:9B | 10.21.36.209 |
| lv6 | 2/1/14 | 1230 | 2700 | AV | Touch Panel | 200V - 5.07 | C4:42:68:2C:3C:0A | 10.21.36.228 |
| lv6 | 2/1/15 | 1272 | 2700 | AV | Airtame | 200V - 5.07 | 38:4B:76:F1:89:2C | 10.21.36.221 |
| lv6 | 2/1/16 | 1230 | 2700 | AV | AV Switcher | 200V - 5.07 | 00:1D:56:09:CE:57 | 10.21.36.222 |
| lv6 | 2/1/16 | 1230 | 2700 | AV | PDU | 200V - 5.07 | B8:80:4F:18:2D:88 | 10.21.36.224 |
| lv6 | 2/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 5.08 | C4:42:68:2C:3D:36 | 10.21.36.236 |
| lv6 | 2/1/18 | 1240 | 2700 | AV | Room NUC | 200V - 5.07 | 48:21:0B:56:AE:39 | 10.21.36.227 |
| lv6 | 2/1/19 | 1230 | 2700 | AV | AV Switcher | 200V - 5.08 | 00:1D:56:0A:DF:BB | 10.21.36.230 |
| lv6 | 2/1/19 | 1230 | 2700 | AV | PDU | 200V - 5.08 | B8:80:4F:18:1C:F0 | 10.21.36.232 |
| lv6 | 2/1/20 | 1272 | 2700 | AV | Airtame | 200V - 5.08 | 38:4b:76:f1:89:48 | 10.21.36.229 |
| lv6 | 2/1/21 | 1240 | 2700 | AV | Room NUC | 200V - 5.08 | 48:21:0B:56:AC:B1 | 10.21.36.235 |
| lv6 | 2/1/22 | 1230 | 2700 | AV | Touch Panel | 200V - 5.09 | C4:42:68:2C:3C:C0 | 10.21.36.244 |
| lv6 | 2/1/23 | 1230 | 2700 | AV | AV Switcher | 200V - 5.09 | 00:1D:56:0A:DF:C3 | 10.21.36.238 |
| lv6 | 2/1/23 | 1230 | 2700 | AV | PDU | 200V - 5.09 | B8:80:4F:28:4C:8A | 10.21.36.240 |
| lv6 | 2/1/24 | 1272 | 2700 | AV | Airtame | 200V - 5.09 | 38:4b:76:f1:8f:ef | 10.21.36.237 |
| lv6 | 2/1/25 | 1240 | 2700 | AV | Room NUC | 200V - 5.09 | 48:21:0B:56:AA:D3 | 10.21.36.243 |
| lv6 | 2/1/26 | 1230 | 2700 | AV | Touch Panel | 200V - 5.10 | C4:42:68:1A:48:25 | 10.21.36.251 |
| lv6 | 2/1/27 | 1272 | 2700 | AV | Airtame | 200V - 5.10 | 38:4b:76:f1:90:1e | 10.21.36.245 |
| lv6 | 2/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 5.10 | 00:1D:56:0A:DF:BC | 10.21.36.246 |
| lv6 | 2/1/28 | 1230 | 2700 | AV | PDU | 200V - 5.10 | B8:80:4F:18:31:9C | 10.21.36.248 |
| lv6 | 2/1/31 | 1240 | 2700 | AV | Room NUC | 200V - 5.12 | 48:21:0B:56:AA:BC | 10.21.37.22 |
| lv6 | 2/1/35 | 1230 | 2700 | AV | PTZ Camera | 200V - 6.01 Lecture Theatre | DC:E2:AC:02:E4:DA | 10.21.37.44 |
| lv6 | 2/1/36 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.12 | B8:98:B0:0F:1C:5F | 10.21.37.17 |
| lv6 | 2/1/37 | 1230 | 2700 | AV | Room Camera | 200V - 5.12 | B8:A4:4F:68:FE:85 | 10.21.37.20 |
| lv6 | 2/1/38 | 1230 | 2700 | AV | Touch Panel | 200V - 5.12 | C4:42:68:38:BB:41 | 10.21.37.23 |
| lv6 | 2/1/39 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.13 | B8:98:B0:0F:1C:78 | 10.21.37.26 |
| lv6 | 2/1/40 | 1230 | 2700 | AV | Room Camera | 200V - 5.13 | B8:A4:4F:69:AB:10 | 10.21.37.29 |
| lv6 | 2/1/41 | 1230 | 2700 | AV | Projector | 200V - 5.13 | F8:25:51:CB:8C:6E | 10.21.37.28 |
| lv6 | 2/1/42 | 1230 | 2700 | AV | Projector | 200V - 6.01 Lecture Theatre | 38:1A:52:B1:75:B7 | 10.21.37.41 |
| lv6 | 2/1/43 | 1230 | 2700 | AV | Touch Panel | 200V - 6.01 Lecture Theatre | C4:42:68:33:59:A9 | 10.21.37.47 |
| lv6 | 2/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.01 Lecture Theatre | B8:98:B0:0F:1C:63 | 10.21.37.38 |
| lv6 | 2/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 6.01 Lecture Theatre | B8:A4:4F:49:85:78 | 10.21.37.46 |
| lv6 | 3/1/11 | 1240 | 2700 | AV | Room NUC | 200V - 5.04 | 48:21:0B:56:AE:12 | 10.21.36.203 |
| lv6 | 3/1/12 | 1272 | 2700 | AV | Airtame | 200V - 5.04 | 38:4b:76:f1:9a:d0 | 10.21.36.197 |
| lv6 | 3/1/13 | 1230 | 2700 | AV | AV Switcher | 200V - 5.05 | 00:1D:56:0A:E0:4F | 10.21.36.206 |
| lv6 | 3/1/13 | 1230 | 2700 | AV | PDU | 200V - 5.05 | B8:80:4F:28:58:6F | 10.21.36.208 |
| lv6 | 3/1/14 | 1230 | 2700 | AV | Touch Panel | 200V - 5.05 | C4:42:68:2A:71:B4 | 10.21.36.212 |
| lv6 | 3/1/15 | 1240 | 2700 | AV | Room NUC | 200V - 5.05 | 48:21:0B:56:AA:62 | 10.21.36.211 |
| lv6 | 3/1/16 | 1272 | 2700 | AV | Airtame | 200V - 5.05 | 38:4b:76:f1:89:26 | 10.21.36.205 |
| lv6 | 3/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 5.06 | C4:42:68:2C:3D:B9 | 10.21.36.220 |
| lv6 | 3/1/18 | 1230 | 2700 | AV | AV Switcher | 200V - 5.06 | 00:1D:56:0A:E0:4E | 10.21.36.214 |
| lv6 | 3/1/18 | 1230 | 2700 | AV | PDU | 200V - 5.06 | B8:80:4F:18:26:32 | 10.21.36.216 |
| lv6 | 3/1/19 | 1272 | 2700 | AV | Airtame | 200V - 5.06 | 38:4b:76:f1:90:1c | 10.21.36.213 |
| lv6 | 3/1/20 | 1230 | 2700 | AV | Room NUC | 200V - 5.06 | 48:21:0B:56:98:6C | 10.21.36.219 |
| lv6 | 3/1/24 | 1230 | 2700 | AV | Touch Panel | 200V - 6.02 Tutorial Room | C4:42:68:38:BB:1E | 10.21.37.55 |
| lv6 | 3/1/25 | 1230 | 2700 | AV | Touch Panel | 200V - 5.01 | C4:42:68:1C:E2:1B | 10.21.36.180 |
| lv6 | 3/1/26 | 1230 | 2700 | AV | Projector | 200V - 6.03 Tutorial Room | F8:25:51:CB:8C:6B | 10.21.37.60 |
| lv6 | 3/1/27 | 1230 | 2700 | AV | Room Camera | 200V - 5.01 | B8:A4:4F:69:B2:70 | 10.21.36.178 |
| lv6 | 3/1/28 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.01 | B8:98:B0:0F:1C:73 | 10.21.36.175 |
| lv6 | 3/1/29 | 1230 | 2700 | AV | Touch Panel | 200V - 5.02 | C4:42:68:2C:3D:5F | 10.21.36.188 |
| lv6 | 3/1/30 | 1230 | 2700 | AV | Room Camera | 200V - 5.02 | B8:A4:4F:69:D0:DE | 10.21.36.186 |
| lv6 | 3/1/31 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.02 | B8:98:B0:0F:1C:5E | 10.21.36.183 |
| lv6 | 3/1/32 | 1230 | 2700 | AV | Touch Panel | 200V - 5.03 | C4:42:68:2A:6C:B0 | 10.21.36.196 |
| lv6 | 3/1/33 | 1230 | 2700 | AV | Room Camera | 200V - 5.03 | B8:A4:4F:68:FE:98 | 10.21.36.194 |
| lv6 | 3/1/34 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.03 | B8:98:B0:0F:1C:68 | 10.21.36.191 |
| lv6 | 3/1/35 | 1230 | 2700 | AV | Touch Panel | 200V - 5.04 | C4:42:68:2C:3C:FE | 10.21.36.204 |
| lv6 | 3/1/36 | 1230 | 2700 | AV | Room Camera | 200V - 5.04 | B8:A4:4F:69:AE:62 | 10.21.36.202 |
| lv6 | 3/1/37 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.04 | B8:98:B0:0F:1C:6B | 10.21.36.199 |
| lv6 | 3/1/38 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.05 | B8:98:B0:0F:1C:6F | 10.21.36.207 |
| lv6 | 3/1/39 | 1230 | 2700 | AV | Room Camera | 200V - 5.06 | B8:A4:4F:49:82:9A | 10.21.36.218 |
| lv6 | 3/1/40 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.06 | B8:98:B0:0F:1C:75 | 10.21.36.215 |
| lv6 | 3/1/41 | 1230 | 2700 | AV | Room Camera | 200V - 5.07 | B8:A4:4F:49:55:98 | 10.21.36.226 |
| lv6 | 3/1/42 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.07 | B8:98:B0:0F:1C:70 | 10.21.36.223 |
| lv6 | 3/1/43 | 1230 | 2700 | AV | Room Camera | 200V - 5.08 | B8:A4:4F:69:AE:77 | 10.21.36.234 |
| lv6 | 3/1/44 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.08 | B8:98:B0:0F:1C:65 | 10.21.36.231 |
| lv6 | 3/1/45 | 1230 | 2700 | AV | Room Camera | 200V - 5.09 | B8:A4:4F:69:B2:86 | 10.21.36.242 |
| lv6 | 3/1/46 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.09 | B8:98:B0:0F:1C:6C | 10.21.36.239 |
| lv6 | 3/1/47 | 1230 | 2700 | AV | Room Camera | 200V - 5.10 | B8:A4:4F:69:D0:9F | 10.21.36.250 |
| lv6 | 3/1/48 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 5.10 | B8:98:B0:0F:1C:6D | 10.21.36.247 |
| lv6 | 4/1/4 | 1230 | 2700 | AV | Touch Panel | 200V - 5.13 | C4:42:68:38:BA:E0 | 10.21.37.31 |
| lv6 | 4/1/5 | 1230 | 2700 | AV | Projector Screen | 200V - 6.01 Lecture Theatre | 04:91:62:D7:90:01 | 10.21.37.43 |
| lv6 | 4/1/6 | 1230 | 2700 | AV | Room Camera | 200V - 6.11 Drama Studio | B8:A4:4F:69:D0:8F | 10.21.37.76 |
| lv6 | 4/1/8 | 1230 | 2700 | AV | Touch Panel | 200V - 6.11 Drama Studio | C4:42:68:2C:B4:62 | 10.21.37.77 |
| lv6 | 4/1/10 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.11 Drama Studio | B8:98:B0:0F:1C:6E | 10.21.37.73 |
| lv6 | 4/1/11 | 1230 | 2700 | AV | Room Camera | 200V - 6.02 Tutorial Room | B8:A4:4F:68:F4:DA | 10.21.37.53 |
| lv6 | 4/1/12 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.02 Tutorial Room | B8:98:B0:0F:1C:67 | 10.21.37.50 |
| lv6 | 4/1/13 | 1272 | 2700 | AV | Airtame | 200V - 6.02 Tutorial Room | 38:4b:76:f1:9a:cd | 10.21.37.48 |
| lv6 | 4/1/14 | 1230 | 2700 | AV | AV Switcher | 200V - 6.02 Tutorial Room | 00:1D:56:0A:E0:53 | 10.21.37.49 |
| lv6 | 4/1/15 | 1230 | 2700 | AV | PDU | 200V - 6.02 Tutorial Room | B8:80:4F:28:1D:83 | 10.21.37.51 |
| lv6 | 4/1/16 | 1230 | 2700 | AV | Projector | 200V - 6.02 Tutorial Room | F8:25:51:CF:6E:65 | 10.21.37.52 |
| lv6 | 4/1/17 | 1230 | 2700 | AV | Touch Panel | 200V - 6.03 Tutorial Room | C4:42:68:38:BB:AC | 10.21.37.63 |
| lv6 | 4/1/18 | 1240 | 2700 | AV | Room NUC | 200V - 6.02 Tutorial Room | 48:21:0B:56:AD:69 | 10.21.37.54 |
| lv6 | 4/1/19 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.03 Tutorial Room | B8:98:B0:0F:1C:6A | 10.21.37.58 |
| lv6 | 4/1/20 | 1230 | 2700 | AV | Room Camera | 200V - 6.03 Tutorial Room | B8:A4:4F:69:AE:6F | 10.21.37.61 |
| lv6 | 4/1/21 | 1272 | 2700 | AV | Airtame | 200V - 6.03 Tutorial Room | 38:4b:76:f1:8f:eb | 10.21.37.56 |
| lv6 | 4/1/22 | 1230 | 2700 | AV | AV Switcher | 200V - 6.03 Tutorial Room | 00:1D:56:0A:E0:88 | 10.21.37.57 |
| lv6 | 4/1/22 | 1230 | 2700 | AV | PDU | 200V - 6.03 Tutorial Room | B8:80:4F:28:54:6D | 10.21.37.59 |
| lv6 | 4/1/23 | 1240 | 2700 | AV | Room NUC | 200V - 6.03 Tutorial Room | 48:21:0B:56:AE:56 | 10.21.37.62 |
| lv6 | 4/1/24 | 1213 | 2510 | PhysicalAccess | cloe-000-200v * |  | 00:11:B9:08:2B:67 | 10.21.22.10 |
| lv6 | 5/1/1 | 1240 | 2800 | printer | 200v-lvl-5-quiet-study |  | 40:F8:DF:64:BE:D6 | 10.21.27.9 |
| lv6 | 5/1/34 | 1230 | 2700 | AV | Projector Screen | 200V - 6.01 Lecture Theatre | 04:91:62:D7:95:5D | 10.21.37.42 |
| lv6 | 5/1/38 | 1240 | 2800 | printer | 200v-lvl-5-eap-office |  | 6C:3C:7C:BE:91:93 | 10.21.27.8 |
| lv6 | 5/1/42 | 1240 | 2800 | printer | 200v-lvl-5-staffroom2 |  | 6C:3C:7C:BE:C0:A9 | 10.21.27.10 |
| lv6 | 6/1/20 | 1230 | 2700 | AV | Recorder | 200V - 6.01 Lecture Theatre | 00:1C:08:00:6B:B7 | 10.21.37.45 |
| lv6 | 6/1/24 | 1272 | 2700 | AV | Airtame | 200V - 6.01 Lecture Theatre | 38:4b:76:f1:9a:d1 | 10.21.37.32 |
| lv6 | 6/1/26 | 1272 | 2700 | AV | Airtame | 200V - 5.13 | 38:4b:76:f1:89:59 | 10.21.37.24 |
| lv6 | 6/1/27 | 1230 | 2700 | AV | AV Switcher | 200V - 5.13 | 00:1D:56:0A:DF:BD | 10.21.37.25 |
| lv6 | 6/1/27 | 1230 | 2700 | AV | PDU | 200V - 5.13 | B8:80:4F:18:74:D1 | 10.21.37.27 |
| lv6 | 6/1/28 | 1240 | 2700 | AV | Room NUC | 200V - 5.13 | 48:21:0B:56:AD:2C | 10.21.37.30 |
| lv6 | 6/1/34 | 1230 | 2700 | AV | Projector | 200V - 5.12 | F8:25:51:CF:6E:6D | 10.21.37.19 |
| lv6 | 6/1/38 | 1240 | 2700 | AV | Room NUC | 200V - 5.12 | 48:21:0B:56:AC:F2 | 10.21.37.21 |
| lv6 | 6/1/39 | 1272 | 2700 | AV | Airtame | 200V - 5.12 | 38:4b:76:f1:89:60 | 10.21.37.15 |
| lv6 | 6/1/40 | 1230 | 2700 | AV | AV Switcher | 200V - 5.12 | 00:1D:56:0A:E0:87 | 10.21.37.16 |
| lv6 | 6/1/40 | 1230 | 2700 | AV | PDU | 200V - 5.12 | B8:80:4F:18:22:12 | 10.21.37.18 |
| lv6 | 6/1/45 | 1230 | 2700 | AV | LCD | 200V - 6.01 Lecture Theatre | A8:4A:63:30:73:65 | 10.21.37.36 |
| lv6 | 6/1/46 | 1230 | 2700 | AV | LCD | 200V - 6.01 Lecture Theatre | A8:4A:63:30:73:E3 | 10.21.37.35 |
| lv6 | 7/1/3 | 1230 | 2700 | AV | Projector | 200V - 5.04 | F8:25:51:CB:8C:95 | 10.21.36.201 |
| lv6 | 7/1/4 | 1230 | 2700 | AV | Projector | 200V - 5.10 | F8:25:51:CB:8C:A9 | 10.21.36.249 |
| lv6 | 7/1/5 | 1230 | 2700 | AV | Projector | 200V - 5.01 | F8:25:51:CB:8A:CE | 10.21.36.177 |
| lv6 | 7/1/8 | 1230 | 2700 | AV | Projector | 200V - 6.11 Drama Studio | F8:25:51:CB:8B:46 | 10.21.37.75 |
| lv6 | 7/1/9 | 1230 | 2700 | AV | Projector | 200V - 5.02 | F8:25:51:CB:8C:97 | 10.21.36.185 |
| lv6 | 7/1/10 | 1272 | 2700 | AV | Airtame | 200V - 6.11 Drama Studio | 38:4b:76:f1:89:2b | 10.21.37.72 |
| lv6 | 7/1/11 | 1230 | 2700 | AV | Projector | 200V - 5.08 | F8:25:51:CB:8B:41 | 10.21.36.233 |
| lv6 | 7/1/14 | 1230 | 2700 | AV | PDU | 200V - 6.11 Drama Studio | B8:80:4F:28:50:35 | 10.21.37.74 |
| lv6 | 7/1/17 | 1230 | 2700 | AV | Projector | 200V - 5.07 | F8:25:51:CB:8C:69 | 10.21.36.225 |
| lv6 | 7/1/19 | 1240 | 2800 | printer | 200v-lvl-6-staff-corridor |  | 34:9F:7B:5E:35:55 | 10.21.27.11 |
| lv6 | 7/1/22 | 1272 | 2700 | AV | Airtame | 200V - 5.01 | 38:4b:76:f1:89:5e | 10.21.36.173 |
| lv6 | 7/1/23 | 1230 | 2700 | AV | AV Switcher | 200V - 5.01 | 00:1D:56:0A:E0:89 | 10.21.36.174 |
| lv6 | 7/1/23 | 1230 | 2700 | AV | PDU | 200V - 5.01 | B8:80:4F:28:19:F0 | 10.21.36.176 |
| lv6 | 7/1/24 | 1240 | 2700 | AV | Room NUC | 200V - 5.01 | 48:21:0B:56:AE:2F | 10.21.36.179 |
| lv6 | 7/1/27 | 1272 | 2700 | AV | Airtame | 200V - 5.02 | 38:4b:76:f1:9a:d4 | 10.21.36.181 |
| lv6 | 7/1/28 | 1230 | 2700 | AV | AV Switcher | 200V - 5.02 | 00:1D:56:0A:E0:4D | 10.21.36.182 |
| lv6 | 7/1/28 | 1230 | 2700 | AV | PDU | 200V - 5.02 | B8:80:4F:28:54:5B | 10.21.36.184 |
| lv6 | 7/1/30 | 1240 | 2700 | AV | Room NUC | 200V - 5.02 | 48:21:0B:56:AA:57 | 10.21.36.187 |
| lv6 | 7/1/31 | 1272 | 2700 | AV | Airtame | 200V - 5.03 | 38:4b:76:f1:90:19 | 10.21.36.189 |
| lv6 | 7/1/32 | 1230 | 2700 | AV | AV Switcher | 200V - 5.03 | 00:1D:56:0A:E0:35 | 10.21.36.190 |
| lv6 | 7/1/32 | 1230 | 2700 | AV | PDU | 200V - 5.03 | B8:80:4F:28:58:54 | 10.21.36.192 |
| lv6 | 7/1/33 | 1240 | 2700 | AV | Room NUC | 200V - 5.03 | 48:21:0B:56:AD:53 | 10.21.36.195 |
| lv6 | 7/1/34 | 1230 | 2700 | AV | Projector | 200V - 5.03 | F8:25:51:CB:8C:A8 | 10.21.36.193 |
| lv6 | 7/1/35 | 1230 | 2700 | AV | AV Switcher | 200V - 5.04 | 00:1D:56:0A:E0:36 | 10.21.36.198 |
| lv6 | 7/1/35 | 1230 | 2700 | AV | PDU | 200V - 5.04 | B8:80:4F:28:5C:11 | 10.21.36.200 |
| lv6 | 7/1/40 | 1230 | 2700 | AV | Projector | 200V - 5.09 | F8:25:51:CB:8B:3D | 10.21.36.241 |
| lv6 | 7/1/44 | 1230 | 2700 | AV | Projector | 200V - 6.01 Lecture Theatre | 38:1A:52:B1:75:B8 | 10.21.37.40 |
| Tutorial-6.03 | 1/1/2 | 1230 | 2700 | AV | Room Camera | 200V - 6.04 Tutorial Room | B8:A4:4F:68:F4:EE | 10.21.37.69 |
| Tutorial-6.03 | 1/1/4 | 1230 | 2700 | AV | Occupancy Sensor | 200V - 6.04 Tutorial Room | B8:98:B0:0F:1C:5B | 10.21.37.66 |
| Tutorial-6.03 | 1/1/5 | 1230 | 2700 | AV | Room NUC | 200V - 6.04 Tutorial Room | 48:21:0B:56:AE:3A | 10.21.37.70 |
| Tutorial-6.03 | 1/1/6 | 1230 | 2700 | AV | Touch Panel | 200V - 6.04 Tutorial Room | C4:42:68:38:BB:80 | 10.21.37.71 |
| Tutorial-6.03 | 1/1/7 | 1272 | 2700 | AV | Airtame | 200V - 6.04 Tutorial Room | 38:4b:76:f1:89:5a | 10.21.37.64 |
| Tutorial-6.03 | 1/1/8 | 1230 | 2700 | AV | PDU | 200V - 6.04 Tutorial Room | B8:80:4F:28:58:2D | 10.21.37.67 |